3GPP TSG-RAN WG4 Meeting #112	R4-2411352

# Maastricht, NL, Aug 19 – 23, 2024

Title: 	Discussion on less than 5MHz for NTN RRM requirement

Source: 	CATT

Agenda item:	8.8.3.4

Document for:	Discussion

- Introduction

A new RAN4 leading work item for enhanced requirements and conductive test methodology for NR NTN and IoT NTN was approved in RAN#103 [1], and the WID have been revised in RAN#104 [2].The latest objectives of core part WI for less than 5MHz for NTN are as following:

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

This document will discuss the general RRM issues about less than 5MHz for NTN, and give our understanding and proposals.

- Discussion

In Rel-18, the feature to support less than 5MHz for NR is defined for TN to cater for use-cases where there are spectrum limitation/constraints. Satellite deployments in L-Band and S-Band face the same constraints and would benefit from the work done in the Rel-18 WID ‘NR support for dedicated spectrum less than 5 MHz for FR1’ by extending support for NTN.

This feature will enable more efficient deployment of the FR1 NTN bands where parts of the spectrum for satellite use is limited. Therefore, as a baseline, RAN4 should define the related RRM requirement based on NTN requirements. This objective is to extend NR-NTN by considering support for dedicated spectrum less than 5 MHz for FR.

General：

When defining RRM requirements for less than 5MHz for NR-NTN in FR1-NTN bands, RAN4 firstly need to discuss what the impact of less than 5MHz on RRM requirements, the following requirements were effected by  less than 5MHz in Rel-18 [3]:

Table 1 RRM Requirements defined for less than 5MHz in Rel-18

| RRC_CONNECTED state mobility | - Handover - NR Handover |
| --- | --- |
| Signalling characteristics | - Radio Link Monitoring - Link Recovery Procedures |
| Measurement Procedure | - NR intra-frequency measurements - NR inter-frequency measurements |

The requirements for less than 5MHz for NR-NTN in Rel-19 should be based on the requirements defined for NR-NTN in FR1-NTN bands, and then refer to the impact of less than 5MHz in Rel-18. According to the above, the related requirements in Table 1 defined for NR-NTN in FR1-NTN bands should be revised.

Observation 1: The requirements for less than 5MHz for NR-NTN in Rel-19 should be based on the requirements defined for NR-NTN in FR1-NTN bands, and then refer to the impact of less than 5MHz in Rel-18.

Proposal 1: The requirements listed in Table 1 defined for NR-NTN in FR1-NTN bands should be revised.

For the specification structure of requirements for less than 5MHz for NR-NTN in TS 38.133[3], the less than 5MHz related requirements can be directly added in the sections defined for NR-NTN.

Proposal 2: The less than 5MHz related requirements can be directly added in the sections defined for NR-NTN in FR1-NTN bands.

For the specific supported bands, according to the WID, FR1 NR-NTN bands n255, n256, and n254 should be considered. For the supported less than 5MHz channel bandwidth, considering that 3MHz had been designed in RAN1 in Rel-18, RAN4 should also support 3MHz for NR-NTN in FR1-NTN bands naturally.

Proposal 3:RAN4 should define RRM requirements for bands n255, n256, and n254 for less than 5MHz in NR-NTN.

Proposal 4: RAN4 should define RRM requirements for 3MHz channel bandwidth for less than 5MHz in NR-NTN.

Handover

RAN4 only need to define the requirements for the scenario of NR FR1 - NR FR1 Handover. For the interruption time, Tsearch should consider the impact of the target cell SSB BW is 12PRBs.

Proposal 4: Tsearch in interruption time should consider the impact of the target cell SSB BW is 12PRBs.

Signalling characteristics

In Radio Link Monitoring requirements, the applicable scenarios should consider the case of PCell operating with less than 5MHz BW, and the UE is configured with only PCell, which is served by satellite access node (SAN).

For the requirements for SSB based radio link monitoring, RAN4 need to define PDCCH transmission parameters for out-of-sync/ in-sync evaluation for a UE operating on a cell with less than 5MHz BW in FR1-NTN. For the requirements for SSB based beam failure detection, RAN4 also need to define PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5MHz BW in FR1-NTN.

Proposal 5: The applicable scenarios for Radio Link Monitoring requirements should consider the case of PCell operating with less than 5MHz BW, and the UE is configured with only PCell, which is served by satellite access node (SAN).

Proposal 6: RAN4 need to define PDCCH transmission parameters for out-of-sync/ in-sync evaluation for a UE operating on a cell with less than 5MHz BW in FR1-NTN.

Proposal 7: RAN4 need to define PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5MHz BW in FR1-NTN.

Measurement Procedure

For NR intra/inter-frequency measurements without and with measurement gaps for SAN, the time period for time index detection (Frequency range FR1) for a target cell with 12 or 15 PRB SSB need to be defined.

Proposal 8: The time period for time index detection (Frequency range FR1) for a target cell with 12 or 15 PRB SSB need to be defined for the following cases:

- NR intra -frequency measurements without measurement gaps for SAN.

- NR intra -frequency measurements with measurement gaps for SAN.

- NR inter-frequency measurements with measurement gaps for SAN.

- NR inter-frequency measurements without measurement gaps for SAN.

- Conclusion

This document discussed preliminary the issues for less than 5MHz for NR NTN RRM requirements and presented following observation and proposals:

Observation 1: The requirements for less than 5MHz for NR-NTN in Rel-19 should be based on the requirements defined for NR-NTN in FR1-NTN bands, and then refer to the impact of less than 5MHz in Rel-18.

Proposal 1: The requirements listed in Table 1 defined for NR-NTN in FR1-NTN bands should be revised.

Proposal 2: The less than 5MHz related requirements can be directly added in the sections defined for NR-NTN in FR1-NTN bands.

Proposal 3:RAN4 should define RRM requirements for bands n255, n256, and n254 for less than 5MHz in NR-NTN.

Proposal 4: RAN4 should define RRM requirements for 3MHz channel bandwidth for less than 5MHz in NR-NTN.

Proposal 5: The applicable scenarios for Radio Link Monitoring requirements should consider the case of PCell operating with less than 5MHz BW, and the UE is configured with only PCell, which is served by satellite access node (SAN).

Proposal 6: RAN4 need to define PDCCH transmission parameters for out-of-sync/ in-sync evaluation for a UE operating on a cell with less than 5MHz BW in FR1-NTN.

Proposal 7: RAN4 need to define PDCCH transmission parameters for beam failure instance for a UE operating on a cell with less than 5MHz BW in FR1-NTN.

Proposal 8: The time period for time index detection (Frequency range FR1) for a target cell with 12 or 15 PRB SSB need to be defined for the following cases:

- NR intra -frequency measurements without measurement gaps for SAN.

- NR intra -frequency measurements with measurement gaps for SAN.

- NR inter-frequency measurements with measurement gaps for SAN.

- NR inter-frequency measurements without measurement gaps for SAN.

- Reference

[1]	  RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, Huawei

[2]	RP-241281, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung

[3]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.6.0