3GPP TSG-RAN WG4 Meeting #112	R4-2411353

Maastricht, NL, Aug 19 – 23, 2024

Title: 	Discussion on enhancements for Rel-19 ATG RRM requirements

Source: 	CATT

Agenda item:	8.10.4

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Air-to-ground network for NR was approved in RAN#103 [1] and the revised WID was approved in RAN#104 [2]. The core part objectives of the work item are as following:

The core part of the work item includes:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: CA_n79C

  - FR1+FR1 inter-band CA

    - Example band combination: CA_n3A-n39A

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

- Specify signaling and UE capabilities if necessary [RAN2].

In the Justification of WID, following clarifications are presented:

Considering the particularity of ATG network deployment, the following aspects should be addressed in Rel-19 ATG work item.

- Enhance throughput to meet ATG capacity demand

- Fully utilize operators’ spectrum especially for the fragment spectrum

This document will discuss the general RRM issues about ATG and give our understanding and proposals.

- Discussion

General

Issue 1-1: Scenarios to be considered for R19 ATG RRM

According to the WI, R19 ATG will mainly consider the scenarios of intra-band co-located and inter-band co-located DL CA, which include FR1 intra-band contiguous CA and FR1+FR1 inter-band CA. Therefore, CA related RRM requirements will be applicable to R19 ATG.

Proposal 1: CA related RRM requirements will be applicable to R19 ATG, which only consider FR1 intra-band contiguous CA and FR1+FR1 inter-band CA.

Issue 1-2: CA related RRM impact for R19 ATG

To our understanding, almost all NR RRM core requirements related to CA will be considered for ATG UE, like normal UE in TN network. The RRM core requirements for intra-band co-located and inter-band co-located DL CA defined for R19 ATG UE may be as shown in Table 1.

Table 1 CA related RRM requirements list considered for R19 ATG UE

| SA: RRC_IDLE state mobility | - Cell Re-selection - Idle Mode CA/DC Measurements - FFS: Measurement report for fast CA/DC setup |
| --- | --- |
| SA: RRC_INACTIVE state mobility | - Cell Re-selection - Inactive Mode CA/DC Measurements - Configured Grant based Small Data Transmissions (CG-SDT) - Random access based Small Data Transmissions (RA-SDT) - FFS: Measurement report for fast CA/DC setup |
| Timing | - Maximum Transmission Timing Difference - Minimum Requirements for NR Carrier Aggregation - Maximum Receive Timing Difference - Minimum Requirements for NR Carrier Aggregation |
| Signalling characteristics | - Radio Link Monitoring - Interruption - SA: Interruptions with Standalone NR Carrier Aggregation - SCell Activation and Deactivation Delay - Link Recovery Procedures - Active BWP switch delay - Pre-configured measurement gap activation/deactivation delay |
| Measurement Procedure | - General measurement requirement - NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - L1-SINR measurements for Reporting - CSI-RS based L3 measurements - NR measurements with autonomous gaps - L1-RSRP measurements for a cell with different PCI from serving cell |

For the specification structure of R19 ATG RRM requirement in TS 38.133[3], the CA related requirements can be directly added in the sections defined for R18 ATG UE.

Proposal 2: Considering the new CA scenario in R19 ATG, the proposed RRM requirements need to be updated or specified, as shown in Table 1.

Proposal 3: For the specification structure of R19 ATG RRM requirements in TS 38.133, the CA related requirements can be directly added in the sections defined for R18 ATG UE.

  - RRC_IDLE/ RRC_INACTIVE state mobility

The requirements of idle mode and inactive mode CA/DC measurements were specifically introduced for CA/DC scenarios, so these requirements were not introduced in R18 ATG. Considering that R19 ATG will introduce intra-band co-located and inter-band co-located DL CA, the related parts ((clauses 4.4 and 5.4 in TS 38.133[3]) should be defined for R19 ATG. An ATG UE supporting idleInactiveNR-MeasReport-r16 or idleInactiveNR-MeasReport-r17 shall perform the idle/inactive mode measurement on the inter-frequency CA candidate frequencies/cells indicated by higher layers and meet the requirement specified in related clauses.

In addition, measurement report for fast CA/DC setup in idle/inactive mode have been introduced in R18 mobility WI, RAN4 need to consider whether to define the CA related requirements for R19 ATG UE.

Proposal 4: The requirements of idle/inactive mode CA measurements (clauses 4.4 and 5.4 in TS 38.133) should be defined for ATG UE in Rel-19.

- RAN4 to consider whether to define the requirements of measurement report for fast CA setup in idle/inactive mode for R19 ATG UE.

  - Timing

Due to the requirements of Maximum Transmission Timing Difference and Maximum Receive Timing Difference are only defined for the scenarios of CA and DC, they were not defined in R18 ATG. Considering that R19 ATG will introduce intra-band co-located and inter-band co-located DL CA, the FR1 related requirements of MTTD and MRTD for Carrier Aggregation (clauses 7.5.4 and 7.6.4 in TS 38.133[3]) should be introduced. The UE shall be capable of handling at least a relative transmission/receive timing difference between slot timing of all pairs of TAGs in FR1 according to the requirements.

Proposal 5: The FR1 related requirements of MTTD and MRTD for Carrier Aggregation (clauses 7.5.4 and 7.6.4 in TS 38.133) should be introduced for ATG UE.

  - Signalling characteristics

Due to interruption requirements are only defined for the scenarios of CA and DC, they were not defined in R18 ATG. RAN4 need to define interruptions with Standalone NR Carrier Aggregation (clause 8.2.2 in TS 38.133[3]) in R19 ATG, which contains the requirements related to the interruptions on PCell and activated SCell if configured.

Proposal 6: The FR1 related requirements of interruptions on PCell and activated SCell with Standalone NR Carrier Aggregation (clause 8.2.2 in TS 38.133) should be introduced for ATG UE.

The requirements of SCell Activation and Deactivation Delay apply for EN-DC, standalone NR carrier aggregation, NE-DC, and NR-DC (clause 8.3 in TS 38.133[3]). For R19 ATG UE, RAN4 only need to introduce the requirements for the delay within which the UE shall be able to activate at least one deactivated SCell and deactivate at least one activated SCell in standalone NR carrier aggregation.

Proposal 7: The FR1 related requirements of SCell Activation and Deactivation Delay (clause 8.3 in TS 38.133) for standalone NR carrier aggregation should be introduced for ATG UE.

- Conclusion

This document discussed preliminary issues for R19 ATG RRM requirements and presented following proposals:

Proposal 1: CA related RRM requirements will be applicable to R19 ATG, which only consider FR1 intra-band contiguous CA and FR1+FR1 inter-band CA.

Proposal 2: Considering the new CA scenario in R19 ATG, the proposed RRM requirements need to be updated or specified, as shown in Table 1.

Proposal 3: For the specification structure of R19 ATG RRM requirements in TS 38.133, the CA related requirements can be directly added in the sections defined for R18 ATG UE.

Proposal 4: The requirements of idle/inactive mode CA measurements (clauses 4.4 and 5.4 in TS 38.133) should be defined for ATG UE in Rel-19.

- RAN4 to consider whether to define the requirements of measurement report for fast CA setup in idle/inactive mode for R19 ATG UE.

Proposal 5: The FR1 related requirements of MTTD and MRTD for Carrier Aggregation (clauses 7.5.4 and 7.6.4 in TS 38.133) should be introduced for ATG UE.

Proposal 6: The FR1 related requirements of interruptions on PCell and activated SCell with Standalone NR Carrier Aggregation (clause 8.2.2 in TS 38.133) should be introduced for ATG UE.

Proposal 7: The FR1 related requirements of SCell Activation and Deactivation Delay (clause 8.3 in TS 38.133) for standalone NR carrier aggregation should be introduced for ATG UE.

- Reference

[1]	RP-240839, New WID on Enhancements for Air-to-ground network for NR, Apple

[2]	RP-241213, Revised WID on Enhancements for Air-to-ground network for NR, CMCC

[3]      3GPP TS 38.133, NR Requirements for support of radio resource management, V18.6.0