3GPP TSG-RAN WG4 Meeting #112	R4-2411354

# Maastricht, NL, Aug 19 – 23, 2024

Title: 	Discussion on Rel-19 RRM requirement for mobility enhancements

Source: 	CATT

Agenda item:	8.23.2

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Rel-19 mobility enhancements was approved in RAN#102 [1], and the latest revised version was approved in RAN#104 [2]. The core part objectives of the work item are as following:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3]

  - Prioritize the case when CU is acting as MN when DC is not configured

  - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases:

    - As secondary priority, support the case where CU is acting as SN and MN is unchanged

    - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released

  - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM

    - Coordination with SA3 needed with respect to security key handling

  - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support

- Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1]

  - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM

  - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1]

    - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work

  - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1]

- Specify support of conditional LTM [RAN2, RAN3, RAN1]

  - Specify UE evaluated conditions for triggering LTM

  - Aim to support conditional LTM including subsequent LTM

  - Prioritise intra-CU LTM

  - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint

- Specify RRM requirements related to the above objectives as necessary [RAN4]

Considering some restrictions on Rel-18 mobility in the justification of WID, the following aspects should be addressed in Rel-19 work item.

- Rel-18 LTM operation only is on supported for mobility between cells of the same gNB (same CU).

- Rel-18 LTM mobility doesn’t support event triggering for L1 measurements.

- L1 measurements for Rel-18 LTM procedures are limited to SSB measurements.

- Rel-18 LTM offers short interruption time but not with the same level of robustness as the conditional L3 mobility procedures.

This document will discuss the general RRM issues about Rel-19 mobility enhancements, and give our understanding and proposals.

- Discussion

CSI-RS based L1 measurement

According to the WID, Rel-19 mobility enhancements will support CSI-RS measurements for LTM procedures and enable CSI-RS based beam management[2].

- General

In Rel-18 LTM, all of requirments were defined based on SSB. In general, we beileve that all requirments defined based on SSB will be defined for CSI-RS, which includes the following requirments:

- PDCCH ordered Random Access for LTM

- LTM PCell/ PSCell Cell Switch

- Link Recovery Procedures

- TRP specific Link Recovery Procedures

- TCI state activation for LTM candidate cell

- L1-SINR measurements for Reporting

- Intra-frequency L1-RSRP measurements for neighbor cell

- NR inter-frequency L1 measurement

Proposal 1: All requirments defined based on SSB will be defined for CSI-RS, which includes the following requirments:

- PDCCH ordered Random Access for LTM

- LTM PCell/ PSCell Cell Switch

- Link Recovery Procedures

- TRP specific Link Recovery Procedures

- TCI state activation for LTM candidate cell

- L1-SINR measurements for Reporting

- Intra-frequency L1-RSRP measurements for neighbor cell

- NR inter-frequency L1 measurement

- CSI-RS based L1 inter-frequency with/without gap measurement

For RRM measurement requirments, we beilieve that both CSI-RS based L1 intra-frequency and inter-frequency measurement should be supported.

For L1 inter-frequency measurement, at least CSI-RS based L1 inter-frequency without gap measurement should be supported. Regarding whether to support CSI-RS based L1 inter-frequency with gap measurement in Rel-19 LTM, considering that RAN4 has defined the requirements for SSB based L1 inter-frequency with gap measurement in Rel-18 LTM and the requirements for CSI-RS based L3 inter-frequency with gap measurement also has been defined in the spec, it is reasonable to support CSI-RS based L1 inter-frequency with gap measurement in Rel-19 and RAN4 need to define the related RRM requirements.

Observation 1: RAN4 has defined the requirements for SSB based L1 inter-frequency with gap measurement in Rel-18 LTM.

Observation 2: The requirements for CSI-RS based L3 inter-frequency with gap measurement also has been defined in the current spec.

Proposal 2: For CSI-RS based L1 inter-frequency measurement in Rel-19 LTM, RAN4 need to define RRM requirements for the following：

- CSI-RS based L1 inter-frequency without gap measurement

- CSI-RS based L1 inter-frequency with gap measurement

- The impact on gap for CSI-RS based L1 inter-f measurement

In Rel-18, RAN1 has dicussed the type of CSI-RS for L1 measurement in RAN1#110bis meeting, but finally only SSB based L1 measurement was supported in R18 LTM, and the agreements are as follows:

RAN1#110bis Agreement - For Rel-18 L1/L2 mobility, - SSB is supported for L1 intra-frequency measurement - SSB is supported for L1 inter-frequency measurement if inter-frequency L1 measurements are supported - Further study the following L1 measurement RS for candidate cell - CSI-RS for tracking, beam management, CSI and mobility, CSI-IM, which is for L1 intra-frequency and L1 inter-frequency (if supported)

For the measurement gap, we beilieve that it is related to the type of CSI-RS for L1 measurement. Since the configurable periodicity of CSI-RS and the method of transmiting and reporting for different purposes may vary, it may affect the gap configuration and the type of gap, even existing gap patterns may not cover the measurements well. For example, CSI-RS for mobility only support periodic transmission and reporting, while CSI-RS for beam management support periodic/semi-persistent/non-periodic transmission and reporting, if CSI-RS for beam management is supported in RAN1, RAN4 need to consider which type of gap will be supported to deal with the semi-persistent/non-periodic transmission of CSI-RS.

Observation 3: The configurable periodicity of CSI-RS and the method of transmitting and reporting for different purposes may vary, which may affect the gap configuration and the type of gap.

In Rel-18, RAN4 only define the requirement for inter-frequency L1-RSRP measurement with type 1 MG. In Rel-19, RAN4 need to discuss whether to reuse or introduce the new gap for CSI-RS based measurement. In our view, at least the type 1 MG should be supported, and whether to introduce the new gap should wait for more RAN1 conclusions about the type of CSI-RS for L1 measurement.

Proposal 3: In Rel-19 LTM, RAN4 need to discuss whether to reuse or introduce the new gap for CSI-RS based measurement.

Proposal 4: For CSI-RS based L1 inter-frequency with gap measurement, at least the type 1 MG needed to be supported in Rel-19 LTM.

- FFS: Whether to support other types of gap or gap patterns need to wait for more RAN1 conclusions on the type of CSI-RS for L1 measurement.

Event triggered L1 measurement reporting

For Event triggered L1 measurement reporting, RAN2 discussed the defination of trigger events in RAN2#126 meeting, and the agreements are the following:

RAN2#126 Agreements: Proposal 6 (modified): Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events. - Event LTM2: Beam of serving cell becomes worse than absolute threshold; - Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell; - Event LTM4: Beam of candidate cell becomes better than absolute threshold; - Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2. FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

According to the above, RAN2 agreed L1 LTM measurement events based on beam specific quality of serving cell and candidate cells, and the specific absolute threshold for beam of serving cell and candidate cell should be defined. RAN4 need to discuss the absolute threshold for beam of serving cell and candidate cell in L1 LTM measurement events, including the measurement matrix and the value.

Proposal 5: RAN4 need to discuss the absolute threshold for beam of serving cell and candidate cell in L1 LTM measurement events, including the measurement matrix and the value.

- RAN4 can wait for more RAN2 input.

- Conclusion

This document discussed preliminary the issues for LTM RRM requirements in Rel-19 and presented following proposals and observations and proposals:

Proposal 1: All requirments defined based on SSB will be defined for CSI-RS, which includes the following requirments:

- PDCCH ordered Random Access for LTM

- LTM PCell/ PSCell Cell Switch

- Link Recovery Procedures

- TRP specific Link Recovery Procedures

- TCI state activation for LTM candidate cell

- L1-SINR measurements for Reporting

- Intra-frequency L1-RSRP measurements for neighbor cell

- NR inter-frequency L1 measurement

Observation 1: RAN4 has defined the requirements for SSB based L1 inter-frequency with gap measurement in Rel-18 LTM.

Observation 2: The requirements for CSI-RS based L3 inter-frequency with gap measurement also has been defined in the current spec.

Proposal 2: For CSI-RS based L1 inter-frequency measurement in Rel-19 LTM, RAN4 need to define RRM requirements for the following：

- CSI-RS based L1 inter-frequency without gap measurement

- CSI-RS based L1 inter-frequency with gap measurement

Observation 3: The configurable periodicity of CSI-RS and the method of transmitting and reporting for different purposes may vary, which may affect the gap configuration and the type of gap.

Proposal 3: In Rel-19 LTM, RAN4 need to discuss whether to reuse or introduce the new gap for CSI-RS based measurement.

Proposal 4: For CSI-RS based L1 inter-frequency with gap measurement, at least the type 1 MG needed to be supported in Rel-19 LTM.

- FFS: Whether to support other types of gap or gap patterns need to wait for more RAN1 conclusions on the type of CSI-RS for L1 measurement.

Proposal 5: RAN4 need to discuss the absolute threshold for beam of serving cell and candidate cell in L1 LTM measurement events, including the measurement matrix and the value.

- RAN4 can wait for more RAN2 input.

- Reference

[1]	RP-234036, New WID: NR mobility enhancements Phase 4, Intel

[2]      RP-241515, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom