3GPP TSG-RAN WG4 Meeting #112	R4-2411356

# Maastricht, NL, Aug 19 – 23, 2024

Title: 	Discussion on RRM requirements for Rel-19 NTN phase3

Source: 	CATT

Agenda item:	8.25.4

Document for:	Discussion

- Introduction

A new RAN4 leading work item for Non-Terrestrial Networks (NTN) for NR Phase 3 was approved in RAN#102 [1]., and the WID have been revised in RAN#104[2].The latest objectives of the work item are as following:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify Orthogonal Cover Codes (OCC) for DFT-s-OFDM PUSCH at least for multiplexing 2 or 4 UEs when PUSCH repetitions are used - Specify necessary signalling - Update RF requirements accordingly, if needed - Notes for this objective: - The enhancement is not targeting improvements/impacts of MU-MIMO capability - The enhancement is not targeted to PUSCH DMRS - No enhancement for initial access - Enhancements to PRACH are not in scope. - This feature may be applicable for UEs operating in terrestrial networks based on a common design - Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Specify SIB signaling to indicate the intended service area in case the satellite footprint covers a larger area. [RAN2] - Specify the necessary signaling between CN and NG-RAN. [RAN3] - Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it. - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

This document will discuss the general RRM issues about NTN phase 3, and give our understanding and proposals.

- Discussion

Downlink coverage enhancements

RAN1 is discussing the enhancements for downlink coverage. According to the WID, if the potential extension of the SSB periodicity is considered in RAN1, RAN4 will further discuss the impact of RRM requirements in NTN operation. RAN4 should wait for more RAN1 progress.

Proposal 1: If SSB channel enhancement with SSB periodicity extension is considered in RAN1, RAN4 will discuss the impact of RRM requirements in NTN operation.

- RAN4 to wait for more RAN1 progress.

Regenerative payload

For the new scenario of regenerative payload, firstly RAN4 need to discuss whether all of existing requirements defined for NR NTN will be defined for regenerative architecture, if yes, the existing requirements can be a baseline. Then, RAN4 need to discuss what the RRM impacts on all of existing requirements defined for NR NTN, and RAN4 will define specific RRM requirements at least for the parts that have an impact due to regenerative architecture.

Proposal 2: RAN4 need to discuss whether all of existing requirements defined for NR NTN will be defined for regenerative architecture.

- If yes, the existing requirements for NR NTN can be a baseline.

Proposal 3: RAN4 need to discuss what the RRM impacts on all of existing requirements defined for NR NTN.

- RAN4 will define specific RRM requirements at least for the parts that effected by regenerative architecture.

For the time and frequency synchronization, the time and frequency pre-compensation were introduced in NR NTN. In the scenario of regenerative payload, the BS will no longer need to compensate for the delay and frequency offset of the feeder link, and RAN4 only need to consider the time and frequency pre-compensation from the UE side.

Considering the RRM impact of timing requirement in NTN,  NTA,adjcommon will be always considered as 0 when calculating the reference point for the UE initial transmit timing control requirement. Due to NTA,adjcommon is defined in TS 38.211, so it may not affect the spec of TS 38.133 in RAN4[3].

Observation 1: For time and frequency pre-compensation with regenerative payload, the BS will no longer need to compensate for the delay and frequency offset of the feeder link, and RAN4 only need to consider the time and frequency pre-compensation from the UE side.

Proposal 4: For regenerative payload, NTA,adjcommon will be always considered as 0 when calculating the reference point for the UE initial transmit timing control requirement.

- FFS: Due to NTA,adjcommon is defined in TS 38.211, so it may not affect the spec of TS 38.133 in RAN4.

In addition, RAN2 is discussing the relevent issues and the related agreements reached in RAN2#126 are as follows:

Regarding potential issues with RRC re-establishment, we wait for RAN3 input, if any Regarding potential issues with support of Inactive state, we wait for RAN3 input, if any Companies interested to support satellite switch with resync with regenerative payload are invited to bring this discussion to the RAN plenary RAN2 aims at supporting RACH-less handover for regenerative architecture. Can further discuss whether any optimization is needed RAN2 assumes that network verified UE positioning is supported for regenerative architecture

Based on the above conclusions from RAN2, we suggest:

Proposal 5: Whether to update the specific RRM requirements for RRC re-establishment with regenerative payload should wait for RAN2/RAN3 progress.

Proposal 6: Whether to update the specific RRM requirements for inactive state with regenerative payload should wait for RAN2/RAN3 progress.

Proposal 7: RAN4 should define the RRM requirements for RACH-less handover for regenerative architecture.

- The existing requirements for NR NTN can be a baseline.

- Wait for more RAN2 progress on further optimization.

Proposal 8: RAN4 should define the RRM requirements for network verified UE positioning for regenerative architecture.

- The existing requirements for NR NTN can be a baseline.

RedCap/eRedCap UE for NTN

According to the WID[2], Rel-19 will support Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands, so RAN4 need to consider the feature of Rel-17 RedCap and Rel-18 eRedCap UEs.

- General：

Firstly, it is necessary to clarify some general issue for (e)Redcap UE with FR1-NTN bands before defining RRM requirements. For the operating band, bandwidth and the number of Rx, the following agreements had already been reached in RF session in RAN4#110-bis meeting[4]:

## Sub-topic 1-4: 1Rx vs. 2Rx for (e)RedCap Agreement: Consider both 1Rx and 2Rx for requirements. ## Sub-topic 1-5: eRedCap BW reduction Agreement: Specify both eRedCap variants, i.e. with and without bandwidth reduction. ## Sub-topic 2-4: Operating bands Agreement: All the NR-NTN FR1-NTN bands will be considered.

Based on the above agreements in RF session, we suggest to follow the same principles for RRM requirements:

Proposal 9: For RRM requirements, the operating band of (e)Redcap UE with FR1-NTN bands include n256, n255 and n254 defined in Table 5.2-1 in TS 38.108.

Proposal 10: For RRM requirements, both bandwidth with and without reduction should be considered for (e)Redcap UE with FR1-NTN bands.

Proposal 11: For RRM requirements, the number of Rx considered for (e)Redcap UE with FR1-NTN bands support include both 1Rx and 2Rx.

According to WID[2], the satellite types considered for (e)Redcap UE with FR1-NTN bands include both GSO and Non-GSO. For UE capability, (e)Redcap UEs with FR1-NTN bands should support the capability of GNSS and the capability of NR-NTN operation.

For the duplex mode, since half duplex FDD(HD-FDD) has been consdered in RedCap WI, RAN4 also need to consider HD-FDD for (e)Redcap UE with FR1-NTN bands. Considering that only FDD are considered for NTN, the duplex mode considered for (e)Redcap UE with FR1-NTN bands include both full duplex(FD-FDD) and half duplex FDD(HD-FDD).

Proposal 12: For RRM requirements, the satellite types considered for (e)Redcap UE with FR1-NTN bands include both GSO and Non-GSO.

Proposal 13: (e)Redcap UEs with FR1-NTN bands should support the capability of GNSS and the capability of NR-NTN operation.

Proposal 14: For RRM requirements, the duplex mode considered for (e)Redcap UE with FR1-NTN bands include both full duplex(FD-FDD) and half duplex FDD(HD-FDD).

Since only NR SA operation mode has been supported for NTN, RAN4 also need to only define the RRM requirements in NR SA operation mode for (e)Redcap UE with FR1-NTN bands.

Proposal 15: For RRM requirements, the scenario considered for (e)Redcap UE with FR1-NTN bands only supports NR SA operation mode.

For SCS, considering the requirements for both FR1-NTN and Redcap, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

Proposal 16: For RRM requirements, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

- The overview of RRM requirements

As a general principle, RAN4 should define the requirements for (e)RedCap UE with FR1-NTN bands based on the existing requirements for FR1-NTN.

Proposal 17: A general principle is that to define the RRM requirements for (e)RedCap UE with FR1-NTN bands based on the existing requirements for FR1-NTN.

The following Table lists the requirements defined for (e)RedCap UE and for NTN.:

Table 1: The requirements currently defined for NTN and (e)RedCap

| Requirements defined for NTN | Requirements defined for RedCap |
| --- | --- |
| SA: RRC_IDLE state mobility |  |
| - Cell Re-selection for NR UE for Satellite Access - Requirements from NTN to NTN - Requirements from NTN to TN - Requirements from TN to NTN - FFS: Minimization of Drive Tests (MDT) for Satellite Access | - Cell Selection - Cell Re-selection for RedCap - NR measurements for positioning for RedCap - RSTD measurements for RedCap - PRS-RSRP measurements for RedCap - PRS-RSRPP measurements for RedCap |
| SA: RRC_INACTIVE state mobility |  |
| - Cell Re-selection - Requirements from NTN to NTN - Requirements from NTN to TN - Requirements from TN to NTN - FFS: Minimization of Drive Tests (MDT) for Satellite Access | - Cell Re-selection for RedCap - Configured Grant based Small Data Transmissions (CG-SDT) for RedCap - NR measurements for positioning for RedCap - Random access based Small Data Transmissions (RA-SDT) for RedCap |
| RRC_CONNECTED state mobility |  |
| - Handover |  |
| - NR SAN Handover - NR SAN FR1 – NR SAN FR1 Handover - NR SAN FR2-NTN – NR SAN FR2-NTN Handover (related to FR2) - FFS: NR SAN Conditional Handover - FFS: NR SAN Satellite switching with re-synchronization | - Handover for RedCap - NR FR1 - NR FR1 Handover - NR FR2- NR FR2 Handover - NR Handover to other RATs |
| - RRC Connection Mobility Control |  |
| - SA: RRC Re-establishment for Satellite Access - UE Re-establishment delay requirement - UE Re-establishment delay requirement for VSAT (related to FR2) - Random access for Satellite Access - SA: RRC Connection Release with Redirection for Satellite Access - RRC connection release with redirection to NR | - SA: RRC Re-establishment for RedCap - Random access for RedCap - SA: RRC Connection Release with Redirection for RedCap - RRC connection release with redirection to NR - RRC connection release with redirection to E-UTRAN |
| Timing |  |
| - UE transmit timing for Satellite Access - UE timer accuracy for satellite access - Timing advance for satellite access | - UE transmit timing for RedCap - UE timer accuracy for RedCap - Timing Advance for RedCap - deriveSSB-IndexFromCell tolerance for RedCap |
| Signalling characteristics |  |
| - Radio Link Monitoring for Satellite Access - Link Recovery Procedures - Active BWP switch delay - Active TCI state switching delay - Uplink spatial relation switch delay (do not apply for satellite access in FR1) - UE-specific CBW change - FFS: Pathloss reference signal switching delay | - Radio Link Monitoring for RedCap - Link Recovery Procedures for Redcap - Active BWP switch delay for RedCap - Active TCI state switching delay for RedCap - Uplink spatial relation switch delay for RedCap - UE-specific CBW change for RedCap |
| Measurement Procedure |  |
| - General measurement requirement - NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - Requirements applicability - Measurement Reporting Requirements - L1-RSRP measurement requirements - Measurement restriction for L1-RSRP measurement (SSB and CSI-RS based) - Scheduling availability of UE during L1-RSRP measurement - L1-RSRP measurement requirements for NTN band above 10GHz - Measurement restriction for L1-RSRP measurement for NTN band above 10GHz - Scheduling availability of UE during L1-RSRP measurement for NTN band above 10GHz - NR measurements for positioning - RSTD measurements (There are no requirements) - PRS-RSRP measurements (There are no requirements) - UE Rx-Tx time difference measurements | - General measurement requirement for RedCap - NR intra-frequency measurements for RedCap - NR inter-frequency measurements for RedCap - Inter-RAT measurements for RedCap - L1-RSRP measurements for Reporting for RedCap - Requirements applicability - Measurement Reporting Requirements - L1-RSRP measurement requirements - Measurement restriction for CSI-RS and SSB for L1-RSRP measurement - Scheduling availability of UE during L1-RSRP measurement - NR measurements for positioning for RedCap - RSTD measurements for RedCap - PRS-RSRP measurements for RedCap - UE Rx-Tx time difference measurements for RedCap - PRS-RSRPP measurements for RedCap - NR measurements with autonomous gaps for RedCap |

Compared with the RRM requirements defined for (e)RedCap UE and NTN in the above Table 1, the requirements highlighted in green in Table 1 should be defined for (e)RedCap UE with FR1-NTN bands.

Proposal 18: The requirements highlighted in green in Table 1 should be defined for (e)RedCap UE with FR1-NTN bands.

- The requirements highlighted in green is a common part of both NTN and (e)RedCap UE requirements.

However, there are some requirements defined for NTN requirements that (e) Redcap does not define. RAN4 need to discuss whether to define these NTN requirements for (e) Redcap with FR1-NTN bands, at least including the following requirements (highlighted in blue in Table 1):

- FFS: Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state

- FFS: NR Conditional Handover

- FFS: NR SAN Satellite switching with re-synchronization

- FFS: Pathloss reference signal switching delay

Proposal 19: For the requirements that have only been defined for NTN, RAN4 needs to discuss whether to define the following FR1-NTN requirements for (e)Redcap:

- FFS: Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state

- FFS: NR Conditional Handover

- FFS: NR SAN Satellite switching with re-synchronization

- FFS: Pathloss reference signal switching delay

Regarding the requirements that are only considered for (e)Redcap and has no relevant definitions in NTN (highlighted in yellow in Table 1, including  the requirements of CG-SDT, RA-SDT, Inter-RAT measurements and NR measurements with autonomous gaps, etc.), RAN4 will not discuss them under this objective for NTN.

Proposal 20: For the requirements that have only been defined for (e)Redcap and has no relevant definition in FR1-NTN, RAN4 will not discuss them under this objective.

- The impact of HD-FDD mode

RAN4 only considered FDD in NTN. However, FD-FDD and HD-FDD mode have been considered for (e)RedCap UE. For HD-FDD (e)RedCap UEs, they can’t transimit the uplink data and receive downlink data at the same time, so RAN4 need to discuss the imapct on RRM requirements for HD-FDD (e)RedCap UE with FR1-NTN bands. According to the RedCap WI, the relevant RRM requirements affected by HD-FDD are listed as follows:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

For the specific impact on above RRM requirements, RAN4 should consider the requirement applicability for HD-FDD (e)RedCap UE with FR1-NTN bands.

Proposal 21: RAN4 need to consider the impact of HD-FDD, and the following RRM requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN bands:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Proposal 22: RAN4 should consider the requirement applicability for HD-FDD (e)RedCap UE with FR1-NTN bands.

- The impact of 1Rx RedCap UE

In RedCap WI, considering that the worse accuracy of 1Rx RedCap UE will result in the reduction of cell coverage, many RRM requirements were relaxed for 1Rx RedCap UE. According to the requirements defined for 1Rx RedCap UE and 2Rx RedCap UE respectively in RedCap WI, we suggest that the following requirements also need to be defined for 1Rx and 2Rx (e)RedCap UEs respectively with FR1-NTN bands:

Table 2 RRM Requirements need to be defined for 1Rx and 2Rx (e)RedCap UEs respectively

SA: RRC_IDLE/ RRC_INACTIVE state mobility

- Cell Selection in RRC_IDLE state - Cell Re-selection in RRC_IDLE/ RRC_INACTIVE state

SA: RRC_CONNECTED state mobility

- NR Handover - NR FR1 - NR FR1 Handover - RRC Connection Mobility Control - SA: RRC Re-establishment - Random access - SA: RRC Connection Release with Redirection

Signalling characteristics

- Radio Link Monitoring - Link Recovery Procedures

Measurement Procedure

- NR intra-frequency measurements - NR inter-frequency measurements - L1-RSRP measurements for Reporting - NR measurements for positioning

For (e)RedCap UE with FR1-NTN bands, it is suggested to also consider the relaxation on the above RRM requirements in Table 2.

Proposal 23: The RRM requirements proposed to define respectively for 1Rx and 2Rx (e)Redcap UEs with FR1-NTN bands are listed in Table 2, like for(e)Redcap UE with TN.

Proposal 24: It is suggested to consider the relaxation on the above requirements for 1Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 25: It is suggested to reuse the existing requirements for NTN as a baseline for 2Rx (e)RedCap UEs.

- The impact of eDRX enhancement

In Rel-17 RedCap and Rel-18 eRedCap, the eDRX enhancement has been introduced, and the relevant enhancements were only have an impact on the RRM requirements for RRC_IDLE/RRC_INACTIVE state mobility. Therefore, the eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE/RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands.

Proposal 26: The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE/RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands.

Even though it is stated that Rel-19 will support Rel-17 RedCap and Rel-18 eRedCap UEs with FR1-NTN bands in WID, it does not mean that all of enhancement RRM requirements for (e)RedCap UEs must be introduced. Especially for the eDRX enhancement requirements, RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced according to the actual demand of power saving.

Proposal 27: RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced for (e)RedCap UEs with FR1-NTN bands.

- Option 1: Only introduce the eDRX enhancement for Rel-17 RedCap UEs.

- Option 2: Introduce the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs.

- The impact of bandwidth reduction

In Rel-17 RedCap, RAN1 discussed whether to allow RedCap UE and normal UE to share initial BWP. In TN network, considering the bandwidth reduction for RedCap UEs compared with normal UE, the frequency range of the initial BWP for normal UE may exceed the maximum bandwidth of RedCap UE. Therefore, RAN4 introduced the configurable RedCap specific initial BWP for RedCap UE in Rel-17. When the RedCap specific initial BWP is configured, the following specific RRM requirements were defined for (e)RedCap UEs:

- RRC Re-establishment delay TUEre-establishdelay

- RRC connection release with redirection delay Tconnection_release_redirect_NR

However, the maximum SAN channel bandwidth defined in Table 5.3.5-1 in TS 38.108 is only 30MHz in FR1, and the deployment of 30 MHz channel bandwidth for NTN SAN needs to be preceded, so the maximum SAN channel bandwidth can be regarded as 20MHz [5]. Since the maximum bandwidth of RedCap UEs is also 20MHz in FR1, RAN4 needs to discuss whether to consider the specific RRM requirements when the RedCap specific initial BWP is configured in NTN network.

Observation 2: For the case where the RedCap specific initial BWP is configured for (e)RedCap UEs in TN network, RAN4 has defined the following specific requirements:

- RRC Re-establishment delay TUE_re-establish_delay

- RRC connection release with redirection delay Tconnection_release_redirect_NR

Proposal 28: RAN4 needs to discuss whether to consider the specific RRM requirements when the RedCap specific initial BWP is configured in NTN network.

- The impact of NCD-SSB

The two types of SSB including Cell Defining SSB(CD-SSB)  and Non Cell Defining SSB (NCD-SSB) were introduced in NR. For CD-SSB, it associated with CORESET # 0, while NCD-SSB is not associated with CORESET # 0 and is not related to the configuration of broadcast system information transmission, and it is mainly used for RRM measurement to support UE mobility.

Consdering that NCD-SSB was widely used in the requirements for (e)RedCap UEs, RAN4 also needs to discuss whether to consider the impact of NCD-SSB and clarify the types of  SSB(CD-SSB or NCD-SSB) in the related requirements for (e)RedCap UEs with FR1-NTN bands.

Proposal 29: RAN4 needs to discuss whether to consider the impact of NCD-SSB for (e)RedCap UEs with FR1-NTN bands.

- FFS: Whether to clarify the types of  SSB(CD-SSB or NCD-SSB) in the related requirements.

- Specification structure：

For the specification structure, we support to define the requireents for (e)RedCap UEs with FR1-NTN in the new sections of section number with new suffix X in the specification, like current way used for NTN requirements and (e)RedCap requirements. It is more clear and simple way.

In addition, RAN4 will utilize the reference method for the requirements that can be reused, thereby reducing the redundancy of the spec to a certain extent.

Proposal 30: Use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands:

- Define them in the new sections of section number with new suffix X in the specification.

- Utilize the reference method for the requirements that can be reused.

- Conclusion

This document discussed preliminary the issues for NTN RRM requirements and presented following observations and proposals:

Proposal 1: If SSB channel enhancement with SSB periodicity extension is considered in RAN1, RAN4 will discuss the impact of RRM requirements in NTN operation.

- RAN4 to wait for more RAN1 progress.

Proposal 2: RAN4 need to discuss whether all of existing requirements defined for NR NTN will be defined for regenerative architecture.

- If yes, the existing requirements for NR NTN can be a baseline.

Proposal 3: RAN4 need to discuss what the RRM impacts on all of existing requirements defined for NR NTN.

- RAN4 will define specific RRM requirements at least for the parts that effected by regenerative architecture.

Observation 1: For time and frequency pre-compensation with regenerative payload, the BS will no longer need to compensate for the delay and frequency offset of the feeder link, and RAN4 only need to consider the time and frequency pre-compensation from the UE side.

Proposal 4: For regenerative payload, NTA,adjcommon will be always considered as 0 when calculating the reference point for the UE initial transmit timing control requirement.

- FFS: Due to NTA,adjcommon is defined in TS 38.211, so it may not affect the spec of TS 38.133 in RAN4.

Proposal 5: Whether to update the specific RRM requirements for RRC re-establishment with regenerative payload should wait for RAN2/RAN3 progress.

Proposal 6: Whether to update the specific RRM requirements for inactive state with regenerative payload should wait for RAN2/RAN3 progress.

Proposal 7: RAN4 should define the RRM requirements for RACH-less handover for regenerative architecture.

- The existing requirements for NR NTN can be a baseline.

- Wait for more RAN2 progress on further optimization.

Proposal 8: RAN4 should define the RRM requirements for network verified UE positioning for regenerative architecture.

- The existing requirements for NR NTN can be a baseline.

Proposal 9: For RRM requirements, the operating band of (e)Redcap UE with FR1-NTN bands include n256, n255 and n254 defined in Table 5.2-1 in TS 38.108.

Proposal 10: For RRM requirements, both bandwidth with and without reduction should be considered for (e)Redcap UE with FR1-NTN bands.

Proposal 11: For RRM requirements, the number of Rx considered for (e)Redcap UE with FR1-NTN bands support include both 1Rx and 2Rx.

Proposal 12: For RRM requirements, the satellite types considered for (e)Redcap UE with FR1-NTN bands include both GSO and Non-GSO.

Proposal 13: The (e)Redcap UEs with FR1-NTN bands should support the capability of GNSS and the capability of NR-NTN operation.

Proposal 14: For RRM requirements, the duplex mode considered for (e)Redcap UE with FR1-NTN bands include both full duplex(FD-FDD) and half duplex FDD(HD-FDD).

Proposal 15: For RRM requirements, the scenario considered for (e)Redcap UE with FR1-NTN bands only supports NR SA operation mode.

Proposal 16: For RRM requirements, the SCS considered for (e)Redcap UE with FR1-NTN support 15kHz, 30kHz and 60kHz.

Proposal 17: A general principle is that to define the RRM requirements for (e)RedCap UE with FR1-NTN bands based on the existing requirements for FR1-NTN.

Proposal 18: The requirements highlighted in green in Table 1 should be defined for (e)RedCap UE with FR1-NTN bands.

- The requirements highlighted in green is a common part of both NTN and (e)RedCap UE requirements.

Proposal 19: For the requirements that have only been defined for NTN, RAN4 needs to discuss whether to define the following FR1-NTN requirements for (e)Redcap:

- FFS: Minimization of Drive Tests (MDT) in RRC_IDLE state and RRC_INACTIVE state

- FFS: NR Conditional Handover

- FFS: NR SAN Satellite switching with re-synchronization

- FFS: Pathloss reference signal switching delay

Proposal 20: For the requirements that have only been defined for (e)Redcap and has no relevant definition in FR1-NTN, RAN4 will not discuss them under this objective.

Proposal 21: RAN4 need to consider the impact of HD-FDD, and the following RRM requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN bands:

- Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

- Handover interruption time

- Random access

- SA: RRC Connection Release with Redirection

- Minimum requirement for L1 indication for RLM and LR

- MAC-CE/DCI/RRC based uplink spatial relation switch delay

- Scheduling availability of UE performing intra/inter measurements

- Scheduling availability of UE during L1-RSRP measurement

Proposal 22: RAN4 should consider the requirement applicability for HD-FDD (e)RedCap UE with FR1-NTN bands.

Proposal 23: The RRM requirements proposed to define respectively for 1Rx and 2Rx (e)Redcap UEs with FR1-NTN bands are listed in Table 2, like for(e)Redcap UE with TN.

Proposal 24: It is suggested to consider the relaxation on the above requirements for 1Rx (e)RedCap UEs with FR1-NTN bands.

Proposal 25: It is suggested to reuse the existing requirements for NTN as a baseline for 2Rx (e)RedCap UEs.

Proposal 26: The eDRX enhancement introduced for (e)RedCap UEs should be considered when defining RRC_IDLE/RRC_INACTIVE state mobility requirements for (e)RedCap UEs with FR1-NTN bands.

Proposal 27: RAN4 need to discuss whether the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs should be introduced for (e)RedCap UEs with FR1-NTN bands.

- Option 1: Only introduce the eDRX enhancement for Rel-17 RedCap UEs.

- Option 2: Introduce the eDRX enhancement for both Rel-17 RedCap and Rel-18 eRedCap UEs.

Observation 2: For the case where the RedCap specific initial BWP is configured for (e)RedCap UEs in TN network, RAN4 has defined the following specific requirements:

- RRC Re-establishment delay TUE_re-establish_delay

- RRC connection release with redirection delay Tconnection_release_redirect_NR

Proposal 28: RAN4 needs to discuss whether to consider the specific RRM requirements when the RedCap specific initial BWP is configured in NTN network.

Proposal 29: RAN4 needs to discuss whether to consider the impact of NCD-SSB for (e)RedCap UEs with FR1-NTN bands.

- FFS: Whether to clarify the types of  SSB(CD-SSB or NCD-SSB) in the related requirements.

Proposal 30: Use the following principles to define the RRM requirements for (e)RedCap UEs with FR1-NTN bands:

- Define them in the new sections of section number with new suffix X in the specification.

- Utilize the reference method for the requirements that can be reused.

- Reference

- RP-234078, New WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Huawei

- RP-241667 Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT

- 3GPP TS 38.133, NR Requirements for support of radio resource management, V18.6.0

- R4-2406610, WF on NR_NTN_Ph3_UERF, Qualcomm Inc.

- 3GPP TS 38.108, NR; Satellite Access Node radio transmission and reception, V18.3.0