3GPP TSG-RAN WG4 Meeting #112				                                              R4-2411357

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor

Source:	CATT

Agenda Item:	8.15.2.1

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the topic summary [2]. In this paper, we continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing Rx beam sweeping factor (BSF) and give our views.

2 Discussion

The first issue we need to discuss is the applicability of L3 measurement delay reduction by optimizing Rx BSF. In last meeting, the applicability about the number of carriers was agreed as below. But the applicability about UE power class and the clarification about Multi-Rx operation were not concluded.

- Issue 2-1-1: Applicability requirement of L3 measurement delay reduction by optimizing Rx BSF Applicability requirement: Agreement: Baseline: L3 delay enhancements in Rel-19 by optimizing Rx BSF for UE supporting multi-rx simultaneous reception are applicable provided that: - the target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and - UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC. Note: The ‘other UE CA/DC modes (e.g., 1 or 2 FR2-1 bands CA, or FR1+FR2 CA/DC, or EN-DC)’ and/or the ‘other number of target to-be-measured carrier(s) on FR2-1 band’ can be FFS after concluding the baseline above. These extra FFS parts will NOT delay the WI completion.

For the enhancement of optimizing Rx BSF which is generally for the UE supporting multiple-Rx simultaneous reception on single carrier, we think the mechanism and conclusions in Rel-18 Multi-Rx WI can be reused as much as possible. But since the optimization in this WI is for L3 measurement while Rel-18 Multi-Rx operation is for L1, some Rel-19 further enhancements should also be allowed.

For the applied UE power class, except for PC6 which is for HST and can be discussed separately, we think there is no need to limit the applicability since generally the RRM requirements are irrelevant to power class. But we do notice that in Rel-18 Multi-Rx, all the UE capabilities are defined for PC3 only, so to promote the discussion, we can also accept to start from PC3.

Proposal 1: For enhanced Rx BSF to reduce L3 measurement delay, it is proposed to reuse the condition and definition in Rel-18 Multi-Rx as much as possible. But some Rel-19 specific enhancements are still allowed.

Proposal 2: It is proposed not to limit the applied power class for enhancement of Rx BSF.

In last meeting, there was also one clarification on WID. Based on the work item description, the enhancement of Rx BSF is for the UE supporting Multi-Rx simultaneous reception on single carrier. It means UE can receive multiple SSBs from different directions on the same target frequency layer within a SMTC window. Some companies mentioned in last meeting that it does not mean UE can process SSB from different directions in parallel and additional processing delay may be needed. In our view, the Multi-Rx operation is only about the reception not the processing, and we agree that the simultaneous reception and simultaneous processing can be discussed separately. But we think more motivations are needed to add additional processing delay and can be discussed in requirements part. Even in single Rx operation or Multi-Rx operation for L1, there may be multiple SSBs from the target frequency layer within one SMTC but no additional delay is considered for processing. We think the support of Multi-Rx operation for L3 does not change the UE processing capability and no additional processing delay is needed either.

Other clarification on WID: - FFS: - “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”

Proposal 3: The enhanced Rx BSF applies to the UE supporting Multi-Rx operation for L3 measurements which means UE supports simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window.

Proposal 4: No need to add additional processing time due to multiple SSBs within one SMTC.

The second issue to be discussed is the condition to apply L3 measurement delay reduction by optimizing Rx BSF. And the following aspects were discussed based on the approved WF [1].

Conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF: - FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature - FFS: UE’s mobility status, e.g., whether HST is precluded or not - FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - FFS: SSB processing delay/time for processing multiple beams received in a SMTC - FFS: Power consumption issue - FFS: UE has prior knowledge on the cell to be measured - FFS: Rel-19 L3 measurement with multi-Rx DL reception is irrelevant to multi-TRP operation deployment - FFS: cell-centre UE or cell-edge UE - FFS: DRX is configured or not (newly added issue in this meeting based on Xiaomi’s proposal) - FFS: Simultaneous operation between L3 and L1 measurements

Regarding the condition about Multi-Rx operation, we think the Rel-18 conclusion can be the baseline. In Rel-18 Multi-Rx, the UE is in multi-Rx operation if UE is configured with group-based beam reporting (GBBR) report. The similar condition can be used for L3 measurement, but since the current GBBR is for L1 measurement, RAN4 need to discuss whether additional conditions are needed to indicate that UE is in Multi-Rx operation for L3 measurement.

Agreements: - For fast beam sweeping, the UE is in multi-Rx operation if following condition is met: - UE is configured with group-based beam reporting (GBBR) report.

For power consumption issue, it also exists in Multi-Rx operation for L1 measurement and it was resolved by introducing the indication on UE preference as shown below.

multiRx-PreferenceFR2 Indicates the UE's preference on single FR2 Rx operation to address overheating or power saving. This field is allowed to be reported only when UE is configured with serving cells operating on FR2.

We think the same mechanism can be reused for L3 measurement and whether to reuse the existing signaling can be further discussed.

Proposal 5: The condition of Multi-Rx operation in Rel-18 can be reused, i.e.,

- the UE is in multi-Rx operation if following condition is met:

  - UE is configured with group-based beam reporting (GBBR) report.

Proposal 6: UE can indicate the preference of Multi-Rx operation for L3 measurement and further discuss whether to reuse the existing signaling.

For searcher or UE mobility status, we think the existing assumption can be applied and no need to specifically differentiate or preclude some cases. E.g., for HST, the requirements are similar as low mobility requirements but with tightened scaling factor. Since the capability for Multi-Rx and HST are separately defined, if UE can support both of the capabilities, the enhanced BSF can also be applied to HST requirements.

For the prior knowledge of target cell, we do not see the reason to introduce it as the condition on top of Multi-Rx operation. The requirements can be applied when UE needs beam sweeping. If UE has the prior knowledge of cell location or direction, UE does not need beam sweeping and there is no need to enhance the beam sweeping factor.

Proposal 7: The existing searcher assumption (i.e., 2 searchers) is applied to the requirements of enhanced BSF.

Proposal 8: The requirements of enhanced BSF can also be applied to HST if UE supports both of the capabilities.

Proposal 9: For enhanced BSF to reduce L3 measurement delay, on top of the UE capability of supporting Multi-Rx, no additional conditions of prior knowledge for target cell is needed so far.

Regarding the UE capability for L3 measurement, we think it can be defined separately with that for L1 measurement. And to reduce the complexity of discussion, we do not need to consider the simultaneous Multi-Rx operation for both L1 and L3 measurement. To achieve this goal, we think the indication on UE preference of Multi-Rx operation can be defined separately and UE can indicate different preference for L1 and L3 measurement. In such case, the flexibility on whether to use Multi-Rx operation for L1 and L3 measurement is left to UE implementation.

Proposal 10: Do not consider simultaneous Multi-Rx operation for both L1 and L3 measurement in this WI.

Regarding the applied requirements to use measurement delay reduction by optimizing Rx BSF, we think intra-frequency measurement, inter-frequency measurement and handover can be considered as first priority, and PSCell addition and SCell activation can also be considered if companies want. For the other requirements, e.g., RRC re-establishment, CGI identification, we think they can be discussed after the solutions are clear.

Proposal 11: Taking intra-/inter-frequency measurement and handover requirements as baseline to discuss the L3 measurement delay reduction by optimizing Rx BSF.

For the solutions of optimizing Rx BSF, we think the conclusions in Rel-18 Multi-Rx can be used as baseline, i.e., for UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. And the reduced Rx BSF applies to intra/ inter-frequency SSB based measurement, handover, PSCell addition and SCell activation.

For intra/inter-frequency SSB based measurement, the enhancement also applies to deactivated PSCell and SCell. And for the associated SSB in CSI-RS based measurement, since it refers to formula of SSB based measurement, the enhanced Rx BSF can also apply but no spec updates are needed. Similarly for handover, since the requirements for CHO and DAPS HO refer to the formula of SSB based measurement and handover respectively, the enhanced Rx BSF can also apply but no specification impact is expected.

Proposal 12: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

Proposal 13: The reduced Rx BSF in proposal 11 can be applied to:

- TPSS/SSS_sync_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement;

- Tsearch for FR1-FR2 HO and FR2-FR2 HO.

Proposal 14: The reduced Rx BSF in proposal 11 can be applied to the associated SSB in CSI-RS based measurement, CHO and DAPS HO with no specification impact.

3 Summary

In this paper, we have some discussions on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor, and the following proposals are given:

Proposal 1: For enhanced Rx BSF to reduce L3 measurement delay, it is proposed to reuse the condition and definition in Rel-18 Multi-Rx as much as possible. But some Rel-19 specific enhancements are still allowed.

Proposal 2: It is proposed not to limit the applied power class for enhancement of Rx BSF.

Proposal 3: The enhanced Rx BSF applies to the UE supporting Multi-Rx operation for L3 measurements which means UE supports simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window.

Proposal 4: No need to add additional processing time due to multiple SSBs within one SMTC.

Proposal 5: The condition of Multi-Rx operation in Rel-18 can be reused, i.e.,

- the UE is in multi-Rx operation if following condition is met:

  - UE is configured with group-based beam reporting (GBBR) report.

Proposal 6: UE can indicate the preference of Multi-Rx operation for L3 measurement and further discuss whether to reuse the existing signaling.

Proposal 7: The existing searcher assumption (i.e., 2 searchers) is applied to the requirements of enhanced BSF.

Proposal 8: The requirements of enhanced BSF can also be applied to HST if UE supports both of the capabilities.

Proposal 9: For enhanced BSF to reduce L3 measurement delay, on top of the UE capability of supporting Multi-Rx, no additional conditions of prior knowledge for target cell is needed so far.

Proposal 10: Do not consider simultaneous Multi-Rx operation for both L1 and L3 measurement in this WI.

Proposal 11: Taking intra-/inter-frequency measurement and handover requirements as baseline to discuss the L3 measurement delay reduction by optimizing Rx BSF.

Proposal 12: For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability.

Proposal 13: The reduced Rx BSF in proposal 11 can be applied to:

- TPSS/SSS_sync_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement;

- Tsearch for FR1-FR2 HO and FR2-FR2 HO.

Proposal 14: The reduced Rx BSF in proposal 11 can be applied to the associated SSB in CSI-RS based measurement, CHO and DAPS HO with no specification impact.

4 References

- R4-2410260	, Way Forward for [111][228] NR_RRM_Ph5, Apple, RAN4#111

- R4-2408025, Topic summary for [111][228] NR_RRM_Ph5, Apple, RAN4#111