3GPP TSG-RAN WG4 Meeting #112				                                              R4-2411358

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

Source:	CATT

Agenda Item:	8.15.2.2

Document for:	Discussion

1 Introduction

In last RAN4 meeting, the enhancement for FR2-1 SSB based measurement delay reduction was discussed. The conclusions were captured in the approved WF [1] and the open issues can be found in the topic summary [2]. In this paper, we continue the discussion on FR2-1 SSB based measurement delay reduction by optimizing CSSF outside gap in CA/DC and give our views.

2 Discussion

Based on the WID, the optimization of CSSF outside MG is for the UE not in multi-Rx simultaneous reception mode, but it needs to be clarified how to indicate the condition. We think it should be the opposite case of optimization of Rx BSF, and the conclusions in Rx BSF optimization as discussed in [3] can be the baseline, i.e., the CSSF optimization can be applied to the case when UE is not capable of Multi-Rx or not in Multi-Rx operation.

Meanwhile, we also notice and understand that the CSSF outside gap enhancement is irrelevant to Multi-Rx status since it is only related to the number of searchers and how to allocate these searchers. So we are also fine to discuss CSSF optimization independently with Multi-Rx.

Proposal 1: Clarify on the CSSF optimization with one of the following options:

- Option 1: The discussion of CSSF optimization starts from the case when UE is not capable of Multi-Rx or UE is capable of Multi-Rx but not in Multi-Rx operation.

- Option 2: Discuss CSSF optimization independently with the UE support of multi-Rx capabilities.

Regarding the applicability of CSSF optimization, we think intra-frequency and inter-frequency measurement requirements can be the baseline to discuss such enhancement. And since in existing requirements, CSSF outside gap applies for all the EN-DC, NE-DC, SA and NR-DC scenarios, if no technical issues identified, all the scenarios can be considered to discuss enhanced CSSF. If any specific conditions are needed e.g., the number of configured FR2 carriers, or CA configurations, it can be discussed in the detailed solutions.

Proposal 2: For L3 measurement delay reduction by optimizing CSSF outside gap, all the EN-DC, NE-DC, SA and NR-DC scenarios can be considered.

In last meeting, the searcher assumption was also discussed. Although it is indicated in the WID that the baseline assumption is 2 searchers, we think other options are not precluded. It can be discussed together with solution of CSSF optimization. If the solutions with additional searcher assumption are feasible and do not change/increase the existing UE implementation complexity, they should be also considered.

Proposal 3: Not to preclude the solutions based on 3 searchers assumption in current stage.

Regarding the scenarios to apply CSSF outside gap optimization, we think the intra-frequency and inter-frequency measurement without gap can be the baseline. Meanwhile, it is noticed that some enhancements on measurement without gap were introduced in Rel-18 including NeedForGap/NeedForGapNCSG related measurement without MG and inter-RAT measurement without MG. The requirements for such enhancements are based on legacy intra-/inter-frequency measurement without MG and are using the same CSSF outside gap. We think the CSSF optimization can also be applied to these requirements.

Proposal 4: The CSSF outside gap optimization applies to:

- TPSS/SSS_sync_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement without gap;

- Inter-RAT measurement without gap

Proposal 5: The applied SSB based intra-frequency and inter-frequency measurements include the cases when:

- UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR or,

- UE indicates ‘no-gap’ via NeedForGapsInfoNR

In last meeting, different solutions were raised to enhance CSSF outside gap, including adjusting searcher allocation, reducing unnecessary measurement and allowing additional searcher assumption and others. The solutions can be summarized as the following:

- Solution #1: Reduce the measurement on serving CCs

  - UE only needs to measure one serving CC per band if multiple serving CCs are in the same band, e.g.,

    - If PCC in the band, measure PCC

    - Otherwise if PSCC in the band, measure PSCC

    - Otherwise if SCC is in the band, measure the SCC with neighbor cell MO

    - Otherwise up to UE implementation

- Solution #2: Using PCC/PSCC searcher to speed up SCC measurement

  - UE can reduce the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions

- Solution #3: Prioritize some of measurement

  - NW can group among intra-frequency layers from configured MO. UE can prioritize to measure one frequency layer in each group.

- Solution #4: Three searchers assumption for UE supporting per-FR gap

  - For UE supporting per FR gap, when all MOs are to be measured outside gap, the CSSFoutsidegap can be optimized, as these MOs can share the three searchers.

- Solution #5: Others

  - Different SMTC on different CCs, minimal CC number applying CSSF,

  - Optimize the counting of layers,

  - Relax SSB measurement, Minimize the impact of CSI-RS measurement

Among all the solutions above, some further clarifications are needed. For solution #1, as discussed in last meeting, it can be implemented by network configuration, e.g., only one servingCellMO is configured on one SCC and no MOs are configured on the other SCCs, if multiple serving CCs are in the same band. Otherwise, if NW configures multiple CCs but UE only measures one of them, it needs to clarify how to interpret other CCs in this band. If it means the measurement on single CC can represent the channel quality of all the CCs in the same band, some additional conditions may be needed between these CCs.

For solution #3, it is similar as solution #1 but with more flexible grouping methods. This grouping rule should be known and aligned for both NW and UE. Based on the proposals in last meeting, the grouping is decided by NW. But it needs to be clarified how to prioritize the measurement in each group, based on NW indication or UE implementation? In our understanding, it is better to follow NW indication. But from NW side, it will increase the complexity and overhead but with no significant benefits. It is proposed to simplify the solution, e.g., merged into solution #1.

For solution #2, the searcher occupation ratio for PCC/PSCC and SCC needs to be discussed. In our understanding, it is better to define a fixed ratio in the specification rather than to dynamically indicate the ratio. But we understand such solution would extend the PCC/PSCC measurement and may impact the communication in some of scenarios. Some additional conditions are needed.

For solution #4, it is based on existing UE capability, and no additional UE implementation complexity is required. We think it is most straightforward and less impact on the specification.

For other solutions, the details are not clear and more clarifications are needed.

We think multiple solutions can be specified if they are feasible. Based on the discussions above, our first preference is solution #4 and solution #2 can also be an alternative.

Proposal 6: Multiple solutions can be considered for CSSF optimization, and option #4 and option #2 are preferred.

- Solution #4: Three searchers assumption for UE supporting per-FR gap

- Solution #2: Using PCC/PSCC searcher to speed up SCC measurement

For the feature capability, we think it depends on the specified solutions, e.g., new capability is not needed for solution #4 but may be needed for other solutions. And it can be discussed after the solutions are clear.

Proposal 7: Discuss the feature capability after the solutions are decided.

3 Summary

In this paper, we have some discussions on FR2-1 L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC, and the following proposals are given:

Proposal 1: Clarify on the CSSF optimization with one of the following options:

- Option 1: The discussion of CSSF optimization starts from the case when UE is not capable of Multi-Rx or UE is capable of Multi-Rx but not in Multi-Rx operation.

- Option 2: Discuss CSSF optimization independently with the UE support of multi-Rx capabilities.

Proposal 2: For L3 measurement delay reduction by optimizing CSSF outside gap, all the EN-DC, NE-DC, SA and NR-DC scenarios can be considered.

Proposal 3: Not to preclude the solutions based on 3 searchers assumption in current stage.

Proposal 4: The CSSF outside gap optimization applies to:

- TPSS/SSS_sync_intra and TSSB_measurement_period_intra for SSB based intra-frequency measurement without gap;

- TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter for SSB based inter-frequency measurement without gap;

- Inter-RAT measurement without gap

Proposal 5: The applied SSB based intra-frequency and inter-frequency measurements include the cases when:

- UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoNR or,

- UE indicates ‘no-gap’ via NeedForGapsInfoNR

Proposal 6: Multiple solutions can be considered for CSSF optimization, and option #4 and option #2 are preferred.

- Solution #4: Three searchers assumption for UE supporting per-FR gap

- Solution #2: Using PCC/PSCC searcher to speed up SCC measurement

Proposal 7: Discuss the feature capability after the solutions are decided.

4 References

- R4-2410260	, Way Forward for [111][228] NR_RRM_Ph5, Apple, RAN4#111

- R4-2408025, Topic summary for [111][228] NR_RRM_Ph5, Apple, RAN4#111

- R4-2411357, Discussion on FR2-1 L3 measurement delay reduction by optimizing Rx beam sweeping factor, CATT, RAN4#112