3GPP TSG-RAN WG4 Meeting #112				                                               R4-2411359

Maastricht, Netherlands, 19th – 23th August, 2024

Title:	Discussion on Fast SCell activation for UE supporting Rel-18 EMR

Source:	CATT

Agenda Item:	8.15.3

Document for:	Discussion

1 Introduction

In RAN#103 meeting, the new WI of NR Radio Resource Management (RRM) Phase 5 [1] has been approved. Based on the objectives description and TU budget, the fast SCell activation with EMR will be discussed from this meeting. The objective is described as below:

- Fast SCell activation for UE supporting Rel-18 EMR - Study and, if feasible, to reduce the SCell activation delay with valid EMR reporting - Apply fast scell activation in FR1 and FR2-1 - Note: RAN4 to start this work from Q3’2024 and aim for completion in Dec’2024. Workplan for this bullet can be discussed in May’2024

In this paper, we initially discuss the fast SCell activation with EMR and give our views.

2 Discussion

2.1 Background

In Rel-16, the UE capabilities for EMR are defined as follows and the requirements for Idle and INACTIVE Mode CA/DC Measurements were defined in clause 4.4 and 5.4 in TS 38.133.

idleInactiveNR-MeasReport-r16, idleInactiveNR-MeasReport-r17 Indicates whether the UE supports configuration of NR SSB measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9]. If this parameter is indicated for FR1 and FR2 differently, each indication corresponds to the frequency range of measured target cell.

idleInactiveEUTRA-MeasReport-r16 Indicates whether the UE supports configuration of E-UTRA measurements in RRC_IDLE/RRC_INACTIVE and reporting of the corresponding results upon network request as specified in TS 38.331 [9].

In Rel-18 mobility, the following two UE capabilities were defined for eEMR, and the capability measValidationReportEMR is for the UE capable of Rel-16 EMR while measValidationReportReselectionMeasurements is for the UE not capable of Rel-16 EMR:

measValidationReportEMR-r18 Indicates whether the UE supports measurement validation and report based on EMR measurement during connection setup/resume for fast CA/DC setup. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively. A UE supporting this feature shall also indicate support of idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16.

measValidationReportReselectionMeasurements-r18 Indicates whether the UE supports measurement validation based on reselection measurements during IDLE/INACTIVE state and reporting for fast CA/DC setup. UE shall set the capability value consistently for all FDD-FR1 bands, all TDD-FR1 bands, all TDD-FR2-1 bands and all TDD-FR2-2 bands respectively.

For the above two types of UE, taking IDLE mode as example, the UE behaviours are defined as below, i.e., when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured, UE only report the valid measurement results based on IDLE mode CA/DC measurements or cell reselection measurements. When measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is not configured, UE is not required to perform validity check for IDLE mode CA/DC measurement or cell reselection measurement and may report the measurement results as long as the results satisfy the accuracy requirements.

For a UE supporting measValidationReportEMR-r18 and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, it shall report based on the idle mode measurement which is specified in clause 4.4, and according to the measurement reporting requirements specified in clause 4.7.3. For a UE supporting measValidationReportReselectionMeasurements-r18, it shall report based on the idle mode measurement specified in the clause 4.2.2, and according to the measurement reporting requirements specified in the clause 4.7.3. The measurement results are considered valid if the following conditions are met for the validity check: - the measurements are performed before msg1 transmission for RRC resume/setup request within the last: - measIdleValidityDuration-r18 seconds for carriers configured in measIdleCarrierListNR-r16 or measIdleCarrierListEUTRA-r16, and/or - measReselectionValidityDuration-r18 seconds for carriers configured in measReselectionCarrierListNR-r18, - the measurement results satisfy measurement accuracy requirement at the measurement instance. Otherwise, the measurement results are considered invalid. The UE shall not report invalid measurement results when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured. If the measIdleValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers in measIdleCarrierListNR-r16 and measIdleCarrierListEUTRA-r16, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance. If the measReselectionValidityDuration-r18 is not configured, the UE is not required to perform validity check for carriers configured in measReselectionCarrierListNR-r18, and the UE may report measurement results given the measurement results satisfy measurement accuracy requirement at the measurement instance.

2.2 Applicability of fast SCell activation

Based on the objectives of Rel-19 RRM phase 5, the discussion is based on Rel-18 eEMR, i.e., the discussion should be applied to the UE supporting measValidationReportEMR or measValidationReportReselectionMeasurements. Meanwhile, since the target is to reduce the SCell activation delay with valid measurement report, the requirements to be defined should be applied to the scenarios when valid results are reported, i.e., when measIdleValidityDuration-r18 and/or measReselectionValidityDuration-r18 is configured and the conditions for the validity check are met.

And it needs to be clarified that this WI is not intended to change the definition, conditions or requirements defined in Rel-18, but to use the valid results derived according to Rel-18 validity check to improve the SCell activation.

Proposal 1: Do not change the Rel-18 eEMR definition in this fast SCell activation discussion.

Proposal 2: The fast SCell activation delay requirements are defined for the case when:

- The UE supports Rel-18 eEMR and is configured with validity duration,

  - the UE supporting measValidationReportEMR-r18 and configured with measIdleValidityDuration-r18 by higher layers, or

  - the UE supporting measValidationReportReselectionMeasurements-r18 and configured with measReselectionValidityDuration-r18 by higher layers.

- And the UE has reported valid results on the SCell to be activated before SCell activation command.

2.3 RRM requirements

In current specification, the following SCell activation delay requirements are defined:

- SCell activation delay requirements - 8.3.2 SCell Activation Delay Requirement for Deactivated SCell - 8.3.4 Direct SCell Activation at SCell addition - 8.3.5 Direct SCell Activation at Handover - 8.3.6 Direct SCell Activation at RRCResume - 8.3.7 SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells - 8.3.9 Direct SCell Activation of Multiple Downlink SCells at SCell addition - 8.3.10 Direct SCell Activation of Multiple Downlink SCells at Handover - 8.3.11 Direct SCell Activation of Multiple Downlink SCells at RRC Resume - 8.3.12 SCell Activation Delay Requirement for Deactivated PUCCH SCell - 8.3.13 SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells - 8.3.16 Fast SCell Activation Delay Requirement for Deactivated SCell - 8.3.17 SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation. - 8.3.18 SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting

For the enhancement of SCell activation delay reduction with valid EMR reporting, we suggest taking Rel-15 SCell activation, i.e., clause 8.3.2, as baseline and discuss whether to apply to other SCell activation delay requirements after the mechanism is clear.

Proposal 3: RAN4 to discuss the fast SCell activation delay with valid EMR reporting using Rel-15 SCell activation as baseline.

In the current specification, the SCell activation delay requirements are defined for known case and unknown case, and the known condition is defined as below for FR1 and FR2:

SCell in FR1 is known if it has been meeting the following conditions: - - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - - the UE has sent a valid measurement report for the SCell being activated and - - the SSB measured remains detectable according to the cell identification conditions specified in clause 9.2 and 9.3. - - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell identification conditions specified in clause 9.2 and 9.3. Otherwise SCell in FR1 is unknown. For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - - the UE has sent a valid L3-RSRP measurement report with SSB index, and - - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell identification conditions specified in clauses 9.2 and 9.3, and the TCI state is selected based on one of the latest reported SSB indexes. Otherwise, the first SCell in FR2 band is unknown.

For the case when UE is performing SCell activation with valid EMR reporting, we think it can also be regarded as known case and the existing delay requirements can be used as baseline. RAN4 need to update the known condition to clarify the case with valid EMR reporting. For example, the known condition for FR1 can be updated to include the following case:

SCell in FR1 is known if it has been meeting the following conditions: - - During the period equal to max(5*measCycleSCell, 5*DRX cycles) for FR1 before the reception of the SCell activation command: - - the UE has sent a valid measurement report for the SCell being activated according to the reporting requirements in 4.7.3 or 5.8.3 and - - the SSB measured remains detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement condition specified in 4.4. - the SSB measured during the period equal to max(5*measCycleSCell, 5*DRX cycles) also remains detectable during the SCell activation delay according to the cell reselection conditions specified in 4.2 or the CA/DC measurement condition specified in 4.4.

And the known condition for FR2 can be updated to include the following case:

For the first SCell activation in FR2 bands, the SCell is known if it has been meeting the following conditions: - - During the period equal to 4s for UE supporting power class 1/5 and 3s for UE supporting power class 2/3/4 before UE receives the last activation command for PDCCH TCI, PDSCH TCI (when applicable) and semi-persistent CSI-RS for CQI reporting (when applicable): - - the UE has sent a valid L3-RSRP measurement report according to the reporting requirements in 4.7.3 or 5.8.3, and - - SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation - - During the period from L3-RSRP reporting to the valid CQI reporting, the reported SSBs with indexes remain detectable according to the cell reselection conditions specified in 4.2 or the CA/DC measurement condition specified in 4.4, and the TCI state is selected based on one of the latest reported SSB indexes.

Proposal 4: For SCell activation with valid Rel-18 eEMR reporting, existing SCell activation delay requirements for known case can be used as baseline.

Proposal 5: RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command.

3 Summary

In this paper, we have some discussions on fast SCell activation with EMR, and the following proposals are given:

Proposal 1: Do not change the Rel-18 eEMR definition in this fast SCell activation discussion.

Proposal 2: The fast SCell activation delay requirements are defined for the case when:

- The UE supports Rel-18 eEMR and is configured with validity duration,

  - the UE supporting measValidationReportEMR-r18 and configured with measIdleValidityDuration-r18 by higher layers, or

  - the UE supporting measValidationReportReselectionMeasurements-r18 and configured with measReselectionValidityDuration-r18 by higher layers.

- And the UE has reported valid results on the SCell to be activated before SCell activation command.

Proposal 3: RAN4 to discuss the fast SCell activation delay with valid EMR reporting using Rel-15 SCell activation as baseline.

Proposal 4: For SCell activation with valid Rel-18 eEMR reporting, existing SCell activation delay requirements for known case can be used as baseline.

Proposal 5: RAN4 to update the known condition for SCell activation to include the case when UE has valid EMR reporting before SCell activation command.

4 References

- RP-240830, New WID: WI resulting from discussion on RRM enhancements, vivo (Moderator), RAN#103

- R4-2404368, Work plan for R19 RRM Phase 5, Apple, CATT RAN4#110bis