3GPP TSG-RAN WG4 Meeting #112				                                               R4-2411360

Maastricht, Netherlands, 19th – 23th August, 2024

Title:	Discussion on RRM requirements for R19 NES

Source:	CATT

Agenda Item:	8.21.2

Document for:	Discussion

1 Introduction

In RAN#102 meeting, the new WI of enhancements of network energy savings [1] has been approved and the WID was updated in RAN#103 and RAN#104 meeting [2][3]. The objectives are copied as below:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown

Based on the RAN4 TU budget, the RRM discussion will start from this meeting. In this paper, we initially discuss the potential RRM impact for the above objectives and give our views.

2 Discussion

2.1 On-demand SSB SCell activation

For the objective of on-demand SSB SCell activation, RAN1 has discussed the scenarios, measurements and signaling, and the following agreements were reached:

RAN1#116bis and RAN1#117 meetings: - Scenarios: - For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2: SCell is configured to a UE but before the UE receives Scell activation command (e.g., as defined in TS 38.321) - Scenario #2A refers to - “When UE receives Scell activation command (e.g., as defined in TS 38.321)” - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - Measurement: - For a cell supporting on-demand SSB Scell operation, - L1 and/or L3 measurement based on on-demand SSB is supported for the cell. - FFS further details on L1 and/or L3 measurement - At least support L1 measurement based on on-demand SSB - Signaling: - For a cell supporting on-demand SSB Scell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell.

Based on the agreements above and the discussion in RAN1, the on-demand SSB can be triggered before UE receives SCell activation command, when UE receives SCell activation command, before SCell activation is completed, when SCell activation is completed and after SCell activation is completed. And the current SCell may or may not have always-on SSB. There will be many cases with the different combinations above. And RAN1 is discussing the applied scenarios and cases. From RAN4 perspective, we can follow RAN1 conclusions to discuss the requirements on agreed cases and to wait for more progress on other cases.

Proposal 1: RAN4 to define on-demand SSB based SCell activation requirements for the following cases as agreed in RAN1:

  - Scenario #2 and Case #1

  - Scenario #2 and Case #2

  - Scenario #2A and Case #1

  - Scenario #2A and Case #2

For the requirements to be defined, the first one is SCell activation delay requirements. In current specification, the following SCell activation delay requirements are defined:

- SCell activation delay requirements - 8.3.2 SCell Activation Delay Requirement for Deactivated SCell - 8.3.4 Direct SCell Activation at SCell addition - 8.3.5 Direct SCell Activation at Handover - 8.3.6 Direct SCell Activation at RRCResume - 8.3.7 SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells - 8.3.9 Direct SCell Activation of Multiple Downlink SCells at SCell addition - 8.3.10 Direct SCell Activation of Multiple Downlink SCells at Handover - 8.3.11 Direct SCell Activation of Multiple Downlink SCells at RRC Resume - 8.3.12 SCell Activation Delay Requirement for Deactivated PUCCH SCell - 8.3.13 SCell activation delay Requirement for Deactivated PUCCH SCell with Multiple SCells - 8.3.16 Fast SCell Activation Delay Requirement for Deactivated SCell - 8.3.17 SCell Activation Delay Requirement for Deactivated SCell with the L3 reporting during activation. - 8.3.18 SCell Activation Delay Requirement for Deactivated SCell with Multiple Downlink SCells with L3 reporting

For the support of on-demand SSB SCell activation, we suggest taking Rel-15 SCell activation delay requirements, i.e., clause 8.3.2 without SSB-less part, as baseline and discuss whether to apply to other SCell activation delay requirements after the mechanism is clear.

Proposal 2: RAN4 to discuss on-demand SSB SCell activation delay requirements using Rel-15 SCell activation as baseline.

Besides the SCell activation delay requirements, if on-demand SSB is triggered before UE receives SCell activation command or after SCell activation is completed, since L3 and L1 measurements are supported for on-demand SSB based on RAN1 agreements, the measurement requirements of deactivated SCell based on on-demand SSB can also be discussed.

And for the scenarios that on-demand SSB is triggered before UE receives SCell activation, if the measurement based on on-demand SSB was reported before SCell activation command, it can be regarded as known case.

Proposal 3: RAN4 to discuss measurement requirements of deactivated SCell based on on-demand SSB.

Proposal 4: For the scenarios that on-demand SSB is triggered before UE receives SCell activation, RAN4 to discuss known condition of SCell activation based on on-demand SSB measurement.

For the scenarios that on-demand SSB is triggered when or after UE receives SCell activation, since the starting time of on-demand SSB is configurable, there may be time gap between SCell activation command and the start of on-demand SSB. But in the existing SCell activation requirements, the time for waiting first SSB is already defined and can be still applied for on-demand SSB. The time gap between signaling for SCell activation and for on-demand SSB triggering is already included in this waiting time.

Proposal 5: For the scenarios that on-demand SSB is triggered when or after UE receives SCell activation, the existing SCell activation delay requirements can be reused.

Regarding the impact of always-on SSB, if no always-on SSB on the SCell, the existing SCell activation requirements can be reused by clarifying that the SSB used in activation procedure is on-demand SSB. If there is always-on SSB on the cell, we think the always-on SSB can also be used and further discuss how to handle the different configurations e.g., periodicity of two types of SSB.

Proposal 6: For Case #1 (i.e., No always-on SSB on the cell, the existing SCell activation delay requirements can be reused by clarifying that the SSB used in activation procedure is on-demand SSB.

Proposal 7: For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), further discuss how to handle the different configurations e.g., periodicity of two types of SSB.

2.2 On-demand SIB1 in idle/inactive state

For on-demand SIB1 in idle/inactive mode, based on the discussion of RAN1/2, the procedure is basically as below:

- UE obtains UL WUS configuration（dedicate PRACH resource）from NES cell or cell A

- UE transmit UL WUS to target cell (NES cell or cell A)

- UE receives on-demand SIB1 from NES cell or cell A

And based on the different choices of cell in each step, there can be multiple scenarios:

- On target cell of UL WUS transmission:

  - Option 1: UE transmits UL WUS to NES Cell

  - Option 2: UE transmits UL WUS to Cell A

- On configuration provision for UL WUS transmission

  - Option A: UE obtains the UL WUS configuration from NES Cell

  - Option B: UE obtains the UL WUS configuration from Cell A

- On receiving of SIB1

  - Option X: UE receives on-demand SIB1 from NES Cell

  - Option Y: UE receives on-demand SIB1 from Cell A

In RAN1#117 meeting, it has been agreed to prioritize Case 2 and Case 3:

- Case 1: Option 1+A+X

- Case 2: Option 1+B+X

- Case 3: Option 2+B+Y

But there are still many issues e.g., UL-WUS configuration, trigger condition and UE identification etc. discussing in RAN1/2 with no conclusions, and it is not clear how the procedure impact measurement configurations. From our perspective, no RRM impact is identified so far and RAN4 need to wait for further progress from other groups.

Proposal 8: No RRM impact is identified for on-demand SIB1 and RAN4 to wait for further progress from other groups.

2.3 Adaptation of common signal/channel transmissions

For the adaptation of common signal/channel transmissions, RAN1 discussion includes SSB adaptation, PRACH adaptation and paging adaptation. From RAN4 perspective, no RRM impact is identified so far for PRACH and paging adaptation and RAN4 can wait for further progress from other groups.

Proposal 9: No RRM impact is identified for PRACH adaptation and paging adaptation, and RAN4 to wait for further progress from other groups.

For SSB adaptation, RAN1 has the following agreements:

Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

Based on the above agreement, the SSB periodicity or other SSB configurations may be adapted by gNB triggering. And from RAN4 perspective, this may impact the measurement requirements if it occurs within the measurement period. But since many issues e.g., which cell the adaptation can be applied, what other configurations can be adapted have no conclusions in RAN1, it is better for RAN4 to wait for further progress from other groups.

Proposal 10: RAN4 to discuss the impact on measurement period requirements due to SSB adaptation.

3 Summary

In this paper, we have some initial discussions on enhancement of network energy savings, and the following proposals are given:

Proposal 1: RAN4 to define on-demand SSB based SCell activation requirements for the following cases as agreed in RAN1:

  - Scenario #2 and Case #1

  - Scenario #2 and Case #2

  - Scenario #2A and Case #1

  - Scenario #2A and Case #2

Proposal 2: RAN4 to discuss on-demand SSB SCell activation delay requirements using Rel-15 SCell activation as baseline.

Proposal 3: RAN4 to discuss measurement requirements of deactivated SCell based on on-demand SSB.

Proposal 4: For the scenarios that on-demand SSB is triggered before UE receives SCell activation, RAN4 to discuss known condition of SCell activation based on on-demand SSB measurement.

Proposal 5: For the scenarios that on-demand SSB is triggered when or after UE receives SCell activation, the existing SCell activation delay requirements can be reused.

Proposal 6: For Case #1 (i.e., No always-on SSB on the cell, the existing SCell activation delay requirements can be reused by clarifying that the SSB used in activation procedure is on-demand SSB.

Proposal 7: For Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), further discuss how to handle the different configurations e.g., periodicity of two types of SSB.

Proposal 8: No RRM impact is identified for on-demand SIB1 and RAN4 to wait for further progress from other groups.

Proposal 9: No RRM impact is identified for PRACH adaptation and paging adaptation, and RAN4 to wait for further progress from other groups.

Proposal 10: RAN4 to discuss the impact on measurement period requirements due to SSB adaptation.

4 References

- RP-234065, New WID: Enhancements of network energy savings for NR, Ericsson (Moderator), RAN#102

- RP-240170, Revised WID for Enhancements of network energy savings for NR, Ericsson, RAN#103

- RP-241650, Revised WID: Enhancements of network energy savings for NR, Ericsson, Apple, RAN#104