3GPP TSG-RAN WG4 Meeting #112                                 R4-2411361

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on simulation assumptions for LP-WUR measurement

Source: 	CATT

Agenda Item:	8.22.4.1

Document for:	Discussion

1. Introduction

To define the LP-WUR measurement requirements, the simulation work is needed. In last RAN4 meeting, the simulation assumption for LP-WUR measurement was discussed and an example table was captured in the approved WF [1] for information which can be the baseline to further discuss.

In this contribution, we have some discussions on the simulation assumption for LP-WUR measurement and give our proposals.

2. Discussion

Since it was agreed in previous RAN4 meeting to start the simulation from FR1, and RAN1 didn’t discuss the signal design for FR2 so far, the discussion in this part only includes the assumption for FR1. And both LP-SS and SSS based measurements are included based on previous agreements.

In previous release, the SS-RSRP and SS-RSRQ measurement has been simulated and the simulation assumption was defined in [3] which can be the baseline for LP-WUR measurement.

Scenarios

Based on the following RAN1 agreements, the measurement metrics include RSRP and RSRQ, and the definition is same as the definition of SS-RSRP and SS-RSRQ.

Measurement metrics: - RAN1#116： - Support LP-RSRP and LP-RSRQ - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - LP-RSRQ=LP-RSRP/LP-RSSI - RAN1#117： - For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. - For LP-RSSI definition for LP-RSRQ, the following is agreed - Option 1: LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. - Note: Above does not constrain LP-SS sequence design for OOK - Working Assumption - From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively. - Above is applicable for both time-domain processing or frequency-domain processing - Above does not imply that RAN1 will introduce LP-SSS-RSRP and LP-SSS-RSRQ in the specifications

Similar as SS-RSRP and SS-RSRP measurement simulation, the following measurements are to be studied:

- RSRP based on LP-SSS,

- RSRQ based on LP-SSS,

- RSRP based on LP-SS,

- RSRQ based on LP-SS.

And in the simulation, two cells are assumed, interfering Cell 1 (unknown to the UE) and measured Cell 2 (known to the UE).

Proposal 1: The following measurements are to be simulated:

- LP-RSRP

- LP-RSRQ

- LP-SSS-RSRP

- LP-SSS-RSRQ

Simulation assumption

For SSS based measurement, the simulation assumption for SS-RSRP/SS-RSRQ can be reused except for the SINR. The same SINR can be used for LP-SS based and LP-SSS based measurement.

Regarding SNR values for LP-SS design, RAN1 has the following agreements:

- RAN1#117： - For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

The above values can be used as the SINR value in RAN4 simulation by taking the interference cell into account.

Proposal 2: The following values agreed in RAN1 can be used as SINR value in RAN4 simulation:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

Proposal 3: Use the same SINR values for LP-SS based and LP-SSS based measurement.

For LP-SS design, RAN1 has the following agreements:

Bandwidth: - RAN1#116bis： - For OOK-4 with M >1, support M=2 & M=4 (working assumption) for LP-WUS. - FFS whether value of M depends on SCS - FFS M=1 for OOK-4 - RAN1#117： - Update agreement in last meeting as below: - From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between = 11 and 12 PRBs - FFS the number of PRBs for 15kHz - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz - FFS: Whether the above is applicable to FR2 LP-SS sequence: - RAN1#117： - Further down-select the number of binary LP-SS sequences for the ‘ON-OFF’ pattern: - 3 - 4 - 8 - 16 - For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following: - Gold sequence - M sequence - Computer searched sequence - FFS: the length of LP-SS sequence

According to the RAN1 discussion and agreements, there are still many aspects not decided for LP-SS design. And RAN4 need to wait for more progress to derive the simulation assumption.

Proposal 4: RAN4 to wait for more progress on LP-SS design, e.g., sequence, waveform and periodicity etc. From RAN1 to derive the simulation assumption.

Proposal 5: The simulation assumption can be defined as the following tables:

Table 1: General parameters

| Parameters | LP-SSS based | LP-SS based |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | FR1: 2.6 GHz |  |
| System bandwidth | 10MHz, 20 MHz |  |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No/Yes |  |
| DRX | No |  |
| BS transmit antennas | 1 tx or single layer transmissions |  |
| UE receive antennas | 1 rx |  |
| Data and control channel subcarrier spacing | The same as SS block | The same as LP-SS |
| Number of measurement samples | [5] | [3, 5] |
| Propagation conditions | AWGN, TDL-C (300 ns delay spread, 100 Hz), TDL-A (30 ns delay spread, 5Hz),TDL-B (100 ns delay spread, 200Hz) |  |
| UE speed | FR1: 30 km/h |  |
| SS burst set/LP-SS configuration |  |  |
| Subcarrier spacing | 15 kHz, 30 kHz |  |
| Number of SS/LP-SS blocks per set | [1] |  |
| LP-SS/SSB burst periodicity | [160 ms] | [320 ms] |
| SSS/LP-SS block BW | 20PRBs for 15kHz and 30kHz | 11PRBs for 30kHz FFS for 15kHz |
| Actual SS/LP-SS block transmissions | Always transmitted | Always transmitted |
| SSS/LP-SS sequences | Company report | TBD |
| LP-SS ‘ON-OFF’ pattern | - | TBD |
| Data Modulation | QPSK | TBD |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| PBCH and DMRS power offset with respect to LP-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to LP-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Waveform |  | OOK-1; OOK-4 with M = 2,4; OFDM; | Same as Cell 1 |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Frequency Offset relative to UE frequency reference | Hz | [TBD] | [TBD] |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | [CP/2] |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | [3 ms] |
| SNR | dB |  |  |
| Es/IoT | dB | N/A | -3dB, -0.5dB, 2dB |
| NOTE: the companies are encouraged to state channel model parameters together with the results, the parameters are to be further discussed and aligned |  |  |  |

Table 3: UE-specific parameters

| [Receiver Filter] | [5th Order Butterworth with 4.32MHz bandwidth] |
| --- | --- |
| [Receiver ADC bit width] | [4/8-bitADC] |
| [Receiver Sampling Rate] | [3.84/7.68MHz] |

Performance Metrics

The simulation results are to be provided for LP-RSRP, LP-RSRQ, LP-SSS-RSRP and LP-SSS-RSRQ.

For different estimation time periods, the CDF curves and 5th, 50th, and 95th %-iles are to be provided for (without RF impairment margin):

- Delta measurement = (estimated measurement – ideal measurement)  [dB];

- Delta cell quality based on N SS block measurements which are L1 filtered but not L3 filtered = (estimated - ideal) [dB], where N=1 (for K=1) and N=K/2 (for K>1).

3. Conclusion

In this paper, we provide our views on the simulation assumption for LP-WUR measurement. From the discussion we have derived the following observations and proposals:

Proposal 1: The following measurements are to be simulated:

- LP-RSRP

- LP-RSRQ

- LP-SSS-RSRP

- LP-SSS-RSRQ

Proposal 2: The following values agreed in RAN1 can be used as SINR value in RAN4 simulation:

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

Proposal 3: Use the same SINR values for LP-SS based and LP-SSS based measurement.

Proposal 4: RAN4 to wait for more progress on LP-SS design, e.g., sequence, waveform and periodicity etc. From RAN1 to derive the simulation assumption.

Proposal 5: The simulation assumption can be defined as the above tables.

4. Reference

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo, RAN4#111

- R4-2408026, Topic summary for [111][229] NR_LPWUS, vivo, RAN4#111

- R4-1806551, Link-level simulation assumptions for SS block measurements, Ericsson, RAN4 #87