3GPP TSG-RAN WG4 Meeting #112                                 R4-2411362

Maastricht, Netherlands, 19th – 23rd August, 2024

Title: 	Discussion on RRM requirements for LP-WUS/WUR

Source: 	CATT

Agenda Item:	8.22.4.2

Document for:	Discussion

1. Introduction

In last RAN4 meeting, the potential RRM requirements for R19 LP-WUS/WUR were discussed and the conclusions were captured in the approved WF [1]. And the detailed issue list can be found in the summary [2].

In this contribution, we have some further discussions on RRM requirements for LP-WUS/WUR and give our proposals.

2. Discussion

Cases/states to be considered for MR and LR

In order to further reduce the UE power consumption of terminals, Rel-18 proposed the concept of Low power wake up signal (LP-WUS), introducing a low-power receiver (LP-WUR) to receive LP-WUS, and then triggering MR to wake up from the ultra-low power state for data transmission. In Rel-19 discussion, RAN2 and RAN4 need to study UE serving cell RRM measurement offloaded from MR to LP-WUR. To align the understanding on the offloading procedure, in last meeting, the different states of MR and LR were discussed and the following 5 cases/scenarios were considered:

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |
| #5 Exit case (legacy case) | On （relaxed） | On | Off |

It was agreed to discuss the RAN4 requirements first for the case #1 and FFS for other cases. In our understanding, case #3 in which the MR measurement behaviour for serving cell and neighbour cell are aligned can also be considered on top of case #1, and the other cases can be deprioritized.

For case #5, it is the legacy case in which LR is turned off. In such case, no matter whether and how to relax the serving cell measurement, all the discussions are irrelevant to LP-WUR which means it is out of this WI scope. Even if LR measurement is turned on to relax the serving cell measurement, we think the power saving gain would be very limited.

For case #4, it is not the typical case and was raised to mainly consider the high priority layers. We think if the entry condition for fully offloading has been met and the MR serving cell measurement has been turned off, it would be not necessary to perform cell reselection and no need to measure neighbor cell even it is high priority. Case #2 may be possible and it can be the extreme relaxation of neighbor cell in case #3, but the benefits compared to case #3 are not significant and we would suggest deprioritizing it.

In summary, RAN4 can discuss the requirements starting from case #1 and case #3. Case #2 and case #4, as the particular scenarios of case #3, can be further considered only if the real demand is proposed.

Proposal 1: RAN4 to prioritize case#1 and case #3 for defining the requirements.

Criteria (entry/exit conditions)

The discussion of criteria of entry/exit conditions includes the conditions for LP-WUS monitoring, LP-WUR measurement and MR RRM measurement relaxation. Firstly, RAN4 need to align the understanding on the three procedures and our understandings on the entry of three procedures are as below:

- LP-WUR measurement: LR starts serving cell measurement and meets the measurement requirements regardless MR status.

- MR measurement relaxation: MR performs relaxed measurement and meets relaxed requirements when LR is turned on, i.e., the case #3 above.

- LP-WUS monitoring: LR starts monitoring LP-WUS. MR may stop the legacy PO monitoring before LR receives LP-WUS indicating wake-up.

Proposal 2: RAN4 to firstly align the understanding on the meaning of entry/exit for LP-WUS monitoring, LP-WUR measurement and MR RRM measurement relaxation.

For the entry/exit condition of LP-WUS monitoring, RAN1 had the following working assumption in RAN1#116bis meeting:

RAN1#116bis meeting: Working Assumption - From RAN1 perspective, for the entry/exit conditions for LP-WUS monitoring in IDLE/inactive mode, - The UE may start LP-WUS monitoring if - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB, and/or - FFS other conditions, and if any, whether all or one or some of the conditions need to be satisfied - FFS the serving cell measurement metrics - The entry/exit thresholds can be configured separately for different types of LR - It is left to RAN2 discussion whether the threshold(s) are always configured by the gNB. - Note: This may be revisited based on the RAN2/RAN4 discussion.

And RAN2 had the following agreements in RAN2#126 meeting:

- RAN2#126 meeting: - Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed. - Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

We think we can follow the RAN1/2 conclusions and no other conditions are needed. Based on RAN1/2 conclusions, RAN4 can further discuss the signal types and measurement metrics used to evaluate the entry/exit condition.

For the entry/exit conditions for LP-WUR measurement, we think when to turn on the LR and start LP-WUR measurement is UE implementation, and what we should consider is when UE can relax or stop the MR measurements. And even when the measurement is performed by MR, UE is not mandatory to turn off LR and stop LR measurement. Based on such understanding, we think we do not need to define the entry/exit condition for LP-WUR measurement.

For the entry/exit conditions for MR measurement relaxation, we think the similar approach as LP-WUS monitoring can be used, i.e, when the MR serving cell measurement is above the threshold.The different part with LP-WUS monitoring is that some relaxation criteria have been defined in the existing specification. When LR measurement is introduced, whether the legacy criteria are still needed should be discussed. And to guarantee the system performance, additional conditions on LR measurement may also help. So the entr/exit condition for MR RRM relaxation can be defined as:

  - For the entry/exit conditions for MR measurement relaxation in IDLE/INACTIVE mode,

    - The MR enter the relaxation mode and meet the relaxed requirements if

      - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB,

      - FFS whether other conditions e.g., Rel-16 ‘not at cell edge’ and ‘low mobility’ criteria are needed.

      - FFS whether the conditions of LR measurement quality are needed.

    - The MR exit the relaxation mode and meet the normal requirements if

      - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB.

Regarding the entry/exit conditions for fully offloading case, i.e., case #1, RAN2 is discussing the criterion and had the following conclusions in RAN2#126 meeting:

- RAN2#126 meeting: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell.

We think RAN4 can leave the criterion discussion to RAN2 and focus on the LP-WUR measurement requirements discussion.

Proposal 3: For the entry/exit condition for LP-WUS monitoring, follow RAN1/2 conclusions, i.e.,

    - The UE may start LP-WUS monitoring if

      - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

    - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

    - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

      - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

Proposal 4: Do not define entry/exit conditions for LP-WUR measurement.

Proposal 5: The entry/exit conditions for MR measurement relaxation are defined as:

  - For the entry/exit conditions for MR measurement relaxation in IDLE/ INACTIVE mode,

    - The MR enters the relaxation mode and meets the relaxed measurement requirements if

      - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB,

      - FFS whether other conditions e.g., Rel-16 ‘not at cell edge’ and ‘low mobility’ criteria are needed.

      - FFS whether the conditions of LR measurement quality are needed.

    - The MR exits the relaxation mode and meets the normal measurement requirements if

      - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB.

Proposal 6: The criterion discussion for fully offloading case (i.e., case #1) is left to RAN2 and RAN4 focus on the LP-WUR measurement requirements.

In the previous meeting, RAN4 reached the following agreements for LP-WUR measurement:

Agreement: - At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following: - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - Other related requirements are FFS

And for the measurement metrics, RAN1 had the following agreements:

RAN1#116bis meeting: - Support LP-RSRP and LP-RSRQ - LP-RSRP is the linear average of received power of LP-SS in OOK ON symbols. - LP-RSRQ=LP-RSRP/LP-RSSI - LP-RSSI is the linear average of total received power in ON and OFF LP-SS OOK symbols. RAN1#117 meeting: - For LP-SS based LP-RSRQ, LP-RSRP and LP-RSSI are measured within the same bandwidth. - From RAN1 perspective, for the RRM measurement metrics based on SSS for OFDM-based LP-WUR, use the same definition of SS-RSRP and SS-RSRQ for LP-SSS-RSRP and LP-SSS-RSRQ, respectively.

According to the above agreements, LP-WUR serving cell measurement can be based on LP-SS and/or PSS/SSS at Idle/Inactive state, so we believe that both LP-SS and PSS/SSS based measurement for different types of LR can be considered when evaluating the above conditions for LP-WUS monitoring and MR measurement relaxation. But whether to define different thresholds for different signal types can be further discussed based on the simulation of LP-SS and SSS based measurement. And for the metrics to be used for entry/exit condition, we think both RSRP and RSRQ should be used, i.e., the serving cell measurement is above the threshold means that measured RSRP > Threshold1 and measured RSRQ > Threshold2.

Proposal 7: Both LP-SS and SSS based measurement can be used for LP-WUR to evaluate the entry/exit condition. Whether to define different conditions can be discussed after performing the simulation.

Proposal 8: Both RSRP and RSRQ measurements are used for LP-WUR and MR to evaluate the entry/exit condition.

LP-WUR measurement requirements in RRC_IDLE/INACTIVE state

According to the existing cell reselection requirements, the UE shall measure the SS-RSRP and SS-RSRQ level of the serving cell and evaluate the cell selection criterion S defined in TS 38.304 [1] for the serving cell within a certain cycle. But for fully offloading case, the MR measurements are stopped and no neighbour cell measurement is performed. In such case, whether the evaluation of cell selection criterion is still needed should be discussed. If needed, the evaluation would be based on LR measurement and the evaluation results may be one of the exit conditions for fully offloading case.

Proposal 9: RAN4 to discuss whether to define evaluation requirements for cell selection criterion based on LR measurement for fully offloading case.

Besides the above requirements, we need to define measurement period requirements for LR to evaluate the entry/exit condition for LP-WUS monitoring and/or MR measurement relaxation. In previous meeting, it was agreed both LP-SS and SSS based serving cell measurement requirements need to be defined. Since the LP-SS design is still discussing in RAN1, RAN4 can discuss the SSS based measurement requirements firstly and wait for more progress from RAN1 for LP-SS.

For the SSS based measurement, we think the existing intra-frequency cell reselection requirements can be used as baseline. And the side condition can be further discussed following RAN1 conclusions on the SNR.

Proposal 10: RAN4 discuss LP-WUR measurement requirements starting from SSS based measurement and wait for more progress from RAN1 for LP-SS based measurement.

Proposal 11: For SSS based LP-WUR measurement, the existing intra-frequency cell reselection requirements can be used as baseline with the side condition to be further discussed.

For the target SNR of LP-WUR measurement, RAN1 has the following conclusion:

- RAN1#117： - For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB

We think RAN4 can use the above values as SINR to perform the simulation and decide whether to use them as side condition of the LP-WUR measurement based on the simulation results.

Proposal 12: The following values agreed in RAN1 can be used to perform simulation and RAN4 to decide whether to use all of them as side condition based on the results.

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

LP-WUR measurement accuracy requirements

In the previous meeting, RAN4 discussed the accuracy for LP-WUR requirements and the related agreement is copied as follows:

Agreement: RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples. - RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement. - For information only: - RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - Options for further discussion (the options are only for information purpose) - Option 1): Define the dedicated accuracy requirement in the performance section, FFS whether/how it can be verified in the test case. - Option 2): No dedicated accuracy requirement in the performance section, and reflect the accuracy performance as a margin in the core requirement. - Option 2a): FFS accuracy requirement, and reflect the accuracy performance in delay requirement in core part. - Other options are not precluded.

For whether/how to define the accuracy requirement, considering that the LP-WUR RRM measurement results of serving cell may be used in the entry/exit conditions, we believe that the measurement accuracy is very important to guarantee the network performance, we prefer to use the legacy measurement accuracy for CONNECTED mode in Clause 10.1.2 TS 38.133 as baseline and discuss the further updates based on simulation results. Regarding how to define the accuracy requirements, we think it can be defined in dedicated section to make it clear. Whether and how to reflect it in the core requirements e.g., introduce additional margin on the threshold can be further discussed after the simulation.

Proposal 13: RAN4 to use the legacy measurement accuracy for CONNECTED mode in Clause 10.1.2 TS 38.133 as baseline.

Proposal 14: Define LP-WUR measurement accuracy in dedicated section. Whether/how to reflect it in the core requirements can be further discussed after the simulation by taking the results into account.

MR RRM relaxation for serving cell/neighbour cell

In Rel-16/17 PowSav and RedCap WI, various RRM measurement relaxation schemes in IDLE/INACTIVE mode on neighbouring cell were introduced. For this WI, further relaxation or even no measurement by MR could be considered while serving cell measurements are offloaded to LP-WUR.

As discussed above, the condition for MR RRM relaxation is when MR serving cell measurement is above a threshold. We think this can be one of the relaxed conditions in parallel with Rel-16 not-at-cell edge or low mobility criterion. So for the neighbor cell measurement relaxation, the existing relaxed requirements can be used as baseline. For the serving cell measurement, there are no relaxed measurement requirements before, but we think the principle when defining neighbor cell measurement relaxation can be reused, i.e., to introduce a relaxation factor based on legacy requirements.

Proposal 15: For MR neighbor cell measurement relaxation, the existing relaxed requirements in 4.2.2 can be used as baseline.

Proposal 16: For MR serving cell measurement relaxation, a scaling factor can be introduced on existing serving cell measurement requirements in 4.2.2.2. Further discuss whether to use same scaling factor as neighbor cell measurement.

LP-WUS operation in RRC_CONNECTED state

Based on the WID as copied below, no LP-WUR measurement is requested and the UE RRM/RLM/BFD/CSI measurements are performed by MR. So no LP-WUR measurement requirements are needed for CONNECTED mode.

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

RAN1 has discussed LP-WUS operation in RRC_CONNECTED state, and has the following agreements:

- RAN1#116: - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance - Minimum time gap between LP-WUS reception and MR to start PDCCH monitoring is introduced considering at least following： - LP-WUS processing time - MR transition time for ramp up - Time/frequency synchronization of MR - RAN1#117: - For RRC CONNECTED mode, support UE capability report for determination of minimum time gap between LP-WUS reception and MR to start PDCCH monitoring.

From the agreements, we can see that the only requirements aspect is the time gap between LP-WUS reception and PDCCH monitoring. And this has been defined as UE capability in RAN1. RAN1 will decide the candidate values and no RAN4 impact is expected so far.

Proposal 17: For LP-WUS operation in RRC_CONNECTED state, no RAN4 impact is expected so far.

3. Conclusion

In this paper, we provide our views on RRM core requirements for R19 LP-WUS/WUR. From this discussion we have derived the following observations and proposals:

Proposal 1: RAN4 to prioritize case#1 and case #3 for defining the requirements.

Proposal 2: RAN4 to firstly align the understanding on the meaning of entry/exit for LP-WUS monitoring, LP-WUR measurement and MR RRM measurement relaxation.

Proposal 3: For the entry/exit condition for LP-WUS monitoring, follow RAN1/2 conclusions, i.e.,

    - The UE may start LP-WUS monitoring if

      - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB

    - If UE starts LP-WUS monitoring, it may stop the legacy PO monitoring before UE receives LP-WUS indicating wake-up

    - The UE monitors the legacy PO (and may monitor PEI) and may stop LP-WUS monitoring if

      - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB

Proposal 4: Do not define entry/exit conditions for LP-WUR measurement.

Proposal 5: The entry/exit conditions for MR measurement relaxation are defined as:

  - For the entry/exit conditions for MR measurement relaxation in IDLE/ INACTIVE mode,

    - The MR enters the relaxation mode and meets the relaxed measurement requirements if

      - the serving cell measurement performed by the MR is above entry threshold(s), if configured by the gNB,

      - FFS whether other conditions e.g., Rel-16 ‘not at cell edge’ and ‘low mobility’ criteria are needed.

      - FFS whether the conditions of LR measurement quality are needed.

    - The MR exits the relaxation mode and meets the normal measurement requirements if

      - the serving cell measurement performed by the LR is below exit threshold(s), if configured by the gNB.

Proposal 6: The criterion discussion for fully offloading case (i.e., case #1) is left to RAN2 and RAN4 focus on the LP-WUR measurement requirements.

Proposal 7: Both LP-SS and SSS based measurement can be used for LP-WUR to evaluate the entry/exit condition. Whether to define different conditions can be discussed after performing the simulation.

Proposal 8: Both RSRP and RSRQ measurements are used for LP-WUR and MR to evaluate the entry/exit condition.

Proposal 9: RAN4 to discuss whether to define evaluation requirements for cell selection criterion based on LR measurement for fully offloading case.

Proposal 10: RAN4 discuss LP-WUR measurement requirements starting from SSS based measurement and wait for more progress from RAN1 for LP-SS based measurement.

Proposal 11: For SSS based LP-WUR measurement, the existing intra-frequency cell reselection requirements can be used as baseline with the side condition to be further discussed.

Proposal 12: The following values agreed in RAN1 can be used to perform simulation and RAN4 to decide whether to use all of them as side condition based on the results.

- SNR=-3dB for NF of LR = NF of MR+ 8dB

- SNR= -0.5dB for NF of LR = NF of MR+ 5dB

- SNR=2dB for NF of LR = NF of MR+ 2dB

- Note 1: The NF of MR is assumed as 7dB

Proposal 13: RAN4 to use the legacy measurement accuracy for CONNECTED mode in Clause 10.1.2 TS 38.133 as baseline.

Proposal 14: Define LP-WUR measurement accuracy in dedicated section. Whether/how to reflect it in the core requirements can be further discussed after the simulation by taking the results into account.

Proposal 15: For MR neighbor cell measurement relaxation, the existing relaxed requirements in 4.2.2 can be used as baseline.

Proposal 16: For MR serving cell measurement relaxation, a scaling factor can be introduced on existing serving cell measurement requirements in 4.2.2.2. Further discuss whether to use same scaling factor as neighbor cell measurement.

Proposal 17: For LP-WUS operation in RRC_CONNECTED state, no RAN4 impact is expected so far.

4. Reference

- R4-2410296, WF for RRM core requirements for LP-WUS/WUR, vivo, RAN4#111

- R4-2408026, Topic summary for [111][229] NR_LPWUS, vivo, RAN4#111