3GPP TSG-RAN WG4 Meeting #112				                                              R4-2411376

Maastricht, Netherlands, 19th – 23rd August, 2024

Title:	Discussion on maintenance issues for Rel-18 gap enhancements

Source:	CATT

Agenda Item:	5.15.1

Document for:	Discussion

1 Introduction

In last meeting, the WI of further enhancements on NR and MR-DC measurement gaps and measurements without gaps was completed and two WFs were approved in [1][2]. But there are still some maintenance issues to be decided. In this paper, we discuss some of the remaining issues and present our views.

2 Discussion

Clarification on the gap applicability of positioning measurement

In the current gap related requirements, there are some applicability rules about positioning measurement, e.g., only per-UE gap can be configured in some cases or some gap patterns can only be used when positioning measurements are configured. Some examples are provided as highlighted below:

If the UE is configured via LPP [34] to measure PRS for any RSTD, PRS-RSRP, UE Rx-Tx time difference measurement and PRS-RSRPP measurement defined in TS 38.215 [4], in order for the requirements in clauses 9.9.2, 9.9.3, 9.9.4 and 9.9.6 to apply, the network must provide - - a single per-UE measurement gap pattern for concurrent monitoring of all positioning frequency layers and intra-frequency, inter-frequency and/or inter-RAT frequency layers of all frequency ranges, or - - if UE supports independent measurement gap patterns for different frequency ranges for PRS measurement, i.e. supporting independentGapConfigPRS-r17, per-FR measurement gap pattern for the frequency range for concurrent monitoring of all positioning frequency layers and intra-frequency, inter-frequency cells and/or inter-RAT frequency layers in the corresponding frequency range. Table 9.1.2-2: Applicability for Gap Pattern Configurations supported by the E-UTRA-NR dual connectivity UE or NR-E-UTRA dual connectivity UE Measurement gap pattern configuration; Serving cell; Measurement PurposeNote 5; Applicable Gap Pattern Id. Per-UE; E-UTRA + FR1, or; non-NR RAT Note1,2; 0,1,2,3. Measurement gap; E-UTRA + FR2, or E-UTRA + FR1 + FR2; FR1 and/or FR2 Note 6,7; 0-11, 24, 25. non-NR RATNote1,2 and FR1 and/or FR2 Note 6,7; 0, 1, 2, 3, 4, 6, 7, 8,10, 24. E-UTRA and, FR1 if configured; non-NR RAT Note1,2; 0,1,2,3. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR1 only Note 6,7; 0-11,24,25. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR2 only Note 6,7; No gap. Per-FR; FR2 if configured; 12-25. measurement gap; E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR1 Note 6,7; 0, 1, 2, 3, 4, 6, 7, 8,10,24. FR2 if configured; No gap. E-UTRA and, FR1 if configured; FR1 and FR2 Note 6,7; 0-11,24,25. FR2 if configured; 12-25. E-UTRA and, FR1 if configured; non-NR RAT Note1,2 and FR2 Note 6,7; 0, 1, 2, 3, 4, 6, 7, 8,10,24. FR2 if configured; 12-25. E-UTRA and, FR1 if configured; Non-NR RAT Note1,2 and FR1 and FR2 Note 6,7; 0, 1, 2, 3, 4, 6, 7, 8,10,24. FR2 if configured; 12-25. Note: In E-UTRA-NR dual connectivity mode, if GSM or UTRA TDD or UTRA FDD inter-RAT frequency layer is configured to be monitored, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. In NR-E-UTRA dual connectivity mode, if UTRA FDD inter-RAT frequency layer is configured to be monitored for SRVCC, only measurement gap pattern #0 and #1 can be used for per-FR gap in E-UTRA and FR1 if configured, or for per-UE gap. NOTE 1: In E-UTRA-NR dual connectivity mode, non-NR RAT includes E-UTRA, UTRA and/or GSM. In NR-E-UTRA dual connectivity mode, non-NR RAT means E-UTRA, and UTRA for SRVCC. NOTE 2: Void NOTE 3: When E-UTRA inter-frequency RSTD measurements are configured and the UE requires measurement gaps for performing such measurements, only Gap Pattern #0 can be used. NOTE 4: For UE supporting supportedGapPattern-NRonly-NEDC or measGapPatterns-NRonly-ENDC-r16 but not supporting supportedGapPattern for the corresponding gap patterns among GP2-11, the corresponding gap patterns are not applicable to measurement of non-NR RATs as defined in NOTE 1. NOTE 5: Inclusion of positioning measurements: Measurement purpose which includes E-UTRA measurements includes also E-UTRA RSRP and E-UTRA RSRQ measurements for E-CID. NOTE 6: Measurement gap patterns #24 and #25 can be requested [2] only when the UE is configured at least with any of RSTD, UE Rx-Tx, PRS-RSRP or PRS-RSRPP measurements requiring such gaps and can only be used during the corresponding positioning measurement period NOTE 7: Inclusion of positioning measurements for measurement gaps: Measurement purpose which includes any of FR1 and FR2 measurements includes also RSTD, UE Rx-Tx, PRS-RSRP and PRS-RSRPP measurements..

For the gap applicability of positioning measurement in current specification, all the existing positioning measurement types including RSTD, UE Rx-Tx, PRS-RSRP and PRS-RSRPP measurements are mentioned. But in Rel-18, two new CPP measurements (i.e., RSCP and RSCPP) are introduced and the gap based requirements are defined. We understand the two measurements should also be included in the applicability of gap configurations. And the required updates are included in our companion CR [3].

Proposal 1: CPP measurement should be introduced to the applicability of gap configurations.

Clarification on the UE behavior when not capable of dynamic collision

In last meeting, the UE capability on the support of dynamic collision is defined for concurrent gap with Pre-MG as below:

| Index | Feature group | Components | Prerequisite feature groups | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- | --- |
| 32-2 | Support for dynamic collisions | Support RRM requirements for handling dynamic collisions between a Pre-MG and another measurement gap or Pre-MG. | 32-1 | UE is not expected to meet RRM requirements for dynamic collisions |

And the UE behavior when it supports dynamic collision is defined, but the requirements when not capable of this capability need to be clarified. We understand UE would follow the legacy collision and priority rule regardless the Pre-MG status.

Proposal 2: For UE not supporting dynamic collision for concurrent gap with Pre-MG, the legacy collision and priority rule would apply regardless of the Pre-MG status.

3 Summary

In this paper, we discuss some of the remaining issues for measurement gap enhancement, and the following proposals are given:

Proposal 1: CPP measurement should be introduced to the applicability of gap configurations.

Proposal 2: For UE not supporting dynamic collision for concurrent gap with Pre-MG, the legacy collision and priority rule would apply regardless of the Pre-MG status.

4 References

- R4-2410376, WF on NR_MG_enh2_part1, MediaTek, RAN4#111

- R4-2410406, WF on NR_MG_enh2_part2, Intel, RAN4#111

- R4-2411375, CR on Rel-18 gap enhancements, CATT, RAN4#112