# 3GPP TSG-RAN WG4 Meeting #112	R4-2411381

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.19.2

Source:	Apple

Title:	Clarification on default assumptions for MU-MIMO with advanced receiver

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#111 the performance part of the Rel-18 WI on NR demodulation performance enhancements was completed.  In this paper we would like to clarify some default assumptions in RAN4 for advanced receiver for MU-MIMO.

# 2. Discussion

For advanced receiver for MU-MIMO in Rel-18 RAN4 had agreed to some default assumptions for some of the information on the co-scheduled UE(s) configuration. The information needed and default assumptions were discussed and agreed in [1] , [2] and summarized in the table below:

Table 1: Summary of RAN4 assumptions for advanced receiver for MU-MIMO

| Information | RAN4 Default assumption (If N/A, how could be obtained by the UE) | Signalling if RAN4 default assumption not valid |
| --- | --- | --- |
| The DMRS port information for the co-scheduled UE | N/A | N/A |
| PRB bundling size for the co-scheduled UE Frequency domain resource allocation for the co-UE within each PRG of the target UE | For the target and any co-scheduled UEs in different CDM groups and with the same DMRS sequence, the target UE assumes the precoding and resource allocation of the co-scheduled UE are the same in the PRG-level grid configured to the target UE when PRG=2 or 4. | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not |
| DMRS power boosting for the co-scheduled UE | Same as target UE | N/A covered in RAN1 assumption |
| Time domain resource allocation information of the co-scheduled UE | Same as target UE | Introduce dedicated RRC signalling to indicate whether the default assumptions valid or not |
| Frequency domain resource allocation for the co-UE across different PRGs of the target UE: | N/A (Obtained by UE blind detection) | N/A |
| CSI-RS location of co-scheduled UE (Only required for R-ML) | UE assumes the target PDSCH is not overlapped with the CSI-RS of the co-scheduled UE | N/A |

Based on agreements in RAN4 we sent LS to RAN2 to introduce the RC based NWA and we have the following in TS 38.331 spec:

For precoding granularity and resource allocation RAN1 has introduced a limitation since Rel-15 on co-scheduled UE(s) on DMRS ports within the same CDM group.

The RRC based NWA covers the default assumption validity on precoding and resource allocation for DMRS ports in different CDM groups.

In RAN4 discussion, RAN1 limitation and RAN2 NWA signaling the assumption has only focused on PRG granularity=2,4, but there was no discussion on the assumption when precoding granularity is set to wideband.

- For assumptions of precoding granularity RAN1 spec is limited to PRG=2,4 for target and co-schedule UE same CDM group.

- RAN4 discussion/ agreements in WF as well as RAN2 RRC based NWA is also limited to PRG=2,4

- There is no assumption for wideband precoding.

We need to discuss the default assumptions in RAN4 when UE is configured with wideband precoder. We propose to use the same assumption as PRG=2, 4 for wideband precoding. If target UE is configured with wideband precoding, the default assumption for advanced receiver for MU-MIMO is that the precoding is the same for the co-scheduled UE(s) in same or different CDM groups. Also, RAN4 should send LS to RAN2 to update the RRC based NWA IE AdvancedReceiver-MU-MIMO field description precodingAndResourceAllocation for accordingly.

- Extend the default assumption on precoding granularity to wideband precoding for advanced receiver for MU-MIMO.

- If the target UE is configured with wideband precoding, the default assumption for advanced receiver with MU-MIMO is that the co-scheduled UE is also configured with wideband precoding.

- Send LS to RAN2 to update the field precodingAndResourceAllocation in IE AdvancedReceiver-MU-MIMO.

TP for RAN2 LS

In the current RRC based assistance signaling for advanced receiver for MU-MIMO the field for precodingAndResourceAllocation only covers PRG=2,4. RAN4 has discussed the default assumptions and agreed that default assumption of same precoding granularity for target and co-scheduled UE(s) shall also be if the target UE is configured with wideband precoding. The assumption is for when the DMRS ports of target and co-scheduled UE are in the same or different CDM groups.

RAN4 would like to request RAN2 to update the IE accordingly.

In addition, we could also seek clarification from RAN1 on UE assumption on precoding granularity for co-scheduled UEs in same CDM group if the DMRS ports of target UE and co-scheduled UE(s) are in the same CDM group and target UE is configured with wideband precoding.

- Send LS to RAN1 to seek clarification on UE assumption of precoding granularity for co-scheduled UEs when target UE is configured for wideband precoder.

TP for RAN1 LS

Based on the text in TS 38.212 the UE can assume that the precoding granularity and resource allocation for any co-scheduled UE are aligned with the target UE at PRG-level when PRG=2, 4 when the co-scheduled UE’s DMRS ports are in the same CDM group as the target UE. RAN4 would like to request clarification on the assumption if target UE is configured with wideband precoding.

# 3. Conclusion

In this paper, we clarify some default assumptions in RAN4 for advanced receiver for MU-MIMO. Our observations and proposals are captured below:

- For assumptions of precoding granularity RAN1 spec is limited to PRG=2,4 for target and co-schedule UE same CDM group.

- RAN4 discussion/ agreements in WF as well as RAN2 RRC based NWA is also limited to PRG=2,4

- There is no assumption for wideband precoding.

- Extend the default assumption on precoding granularity to wideband precoding for advanced receiver for MU-MIMO.

- If the target UE is configured with wideband precoding, the default assumption for advanced receiver with MU-MIMO is that the co-scheduled UE is also configured with wideband precoding.

- Send LS to RAN2 to update the field precodingAndResourceAllocation in IE AdvancedReceiver-MU-MIMO.

- Send LS to RAN1 to seek clarification on UE assumption of precoding granularity for co-scheduled UEs when target UE is configured for wideband precoder.

# Reference

- R4-2309892, “WF on advanced receiver for MU-MIMO scenario”, China Telecom.

- R4-2313993, “WF on advanced receiver for MU-MIMO scenario”, China Telecom.