# 3GPP TSG-RAN WG4 Meeting #112	R4-2411384

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.27.1

Source:	Apple

Title:	On MIMO evolution RRM requirements maintenance

Release:	Rel-18

Document for:	Discussion

# 1. Introduction

In RAN4#111 RRM impacts for R18 MIMO evolution were discussed and way forward [1] was agreed.  In this contribution we present our views on RRM requirements maintenance for MIMO evolution for 2TA and unified TCI framework with mTRP.

# 2. Discussion

## 2 TA with multi-DCI multi-TRP

The reference timing for 2 TA has been discussed in previous meetings. The issues related to 2 TA were:

- Applicability of the requirements for UL signals

- Reference point definition for PUCCH/PUSCH/SRS

- Reference point for PDCCH order triggered PRACH for intra-cell and inter-cell cases

The above issues were discussed in previous meetings with no conclusion.

At a high level our proposals for addressing the issues with 2 TA are:

- For multi-DCI multi-TRP with 2TA the timing requirements are applicable to PUCCH/PUSH/SRS and PDCCH ordered RACH for CFRA

- The reference point for PUCCH/PUSCH/SRS for each TAG is the first detected path in the reference cell associated with the activated DL or joint TCI state having the same TAG as the UL signal.

- RAN4 shall define the reference point for PDCCH order RACH for CFRA for inter-cell and intra-cell scenario.

In this meeting we have a CR [2] based on offline discussion and co-sourced by multiple companies and should bring a conclusion to these open issues.

We have timing error and gradual timing adjustment requirements defined for multi-DCI multi-TRP operation with 2 TA. The gradual timing adjustment requirement in §7.1.2.1 is defined as:

The gradual timing adjust requirements for 2 TA are applicable for each TAG independently. The UE applies gradual timing adjustment until the timing till transmission timing error between UE and the reference timing is within ±Te for each TAG. For multi-DCI multi-TRP operation with 2TA there is also MTTD requirement applicable to the pair of TAGs.

The gradual timing adjustment requirements are applicable per TAG and the MTTD requirements are applicable for the pair of TAGs.

- For multi-DCI multi-TRP operation with 2TA the UE has gradual timing adjust requirements applicable to each TAG.

- For multi-DCI multi-TRP operation with 2TA the UE has MTTD requirements applicable to pair of TAGs.

The UE is expected to meet both the requirements for gradual timing adjustment and MTTD for the pair of TAGs. The gradual timing adjust is applied until the timing error with respect to reference timing is within ±Te for each TAG. At the same time MTTD requirement shall be met. It is possible that with gradual timing adjustment applied on each TAG independently the TTD between the pair of TAGs exceeds MTTD requirement and could impact the UL transmission.

- While the UE is applying gradual timing adjustment for each TAG, the TTD for the pair of TAGs could exceed the MTTD requirement.

There should be a limit on the gradual timing adjustment to ensure that the MTTD is not exceeded. Without the limit the gradual timing adjustment independently on each TAG might impact UL transmission for multi-DCI multi-TRP with 2TA. Hence, we propose to limit the gradual timing adjustment until the TTD between pair of TAGs is within the MTTD requirement.

- For multi-DCI multi-TRP with 2TA the gradual timing adjustment is applied until the timing error for each TAG is within ±Te or the TTD between pair of TAGs doesn’t exceed the MTTD requirement for 2TAs.

Our proposal is captured in our companion CR [3].

There would be a similar issue with legacy gradual timing adjustment requirements and MTTD requirements for CA, which can be discussed once we conclude on addressing the issue for mTRP requirements.

- Similar issue with legacy gradual timing adjustment and MTTD for CA to be discussed once RAN4 concludes on this issue for mTRP requirements.

## Multi-DCI based mTRP

For multi-DCI multi-TRP TCI state switch we agreed on delay requirement for overlapping or adjacent SSB.

One open issue for multi-DCI multi-TRP is UL TCI state switching requirements is the requirements with 2TA:

Issue 1-2-2: For mDCI mTRP, how to specify UL TCI state switching requirements for eUTCI if UE supporting two TAs (RTD<CP and RTD>CP)?

< Way forward >

- Option 1 (Apple, Samsung)

  - For joint TCI state, no additional DL RS tracking time for UL TCI state switching.

  - For separate UL TCI state, If the DL beams are changed as well and DL TCI is not in the active list, the previous DL timing cannot be used. Additional DL RS tracking time for UL TCI state switching is needed as:

    - Known case: THARQ + 3Nslotsubframe,µ + TOk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+NM*( Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms)

    - Unknown case: THARQ + 3Nslotsubframe,µ + TL1-RSRP + TOuk-ref (Tfirst-SSB-DLRef + OL*T SSB-DLRef + 2ms)+ Tfirst-PL-RS  + 4*Ttarget_PL-RS + 2ms

    - TOk-ref = 1 if there is no active DL TCI-State for DL timing reference associated with the same coresetPoolIndex

  - For other cases of separate UL TCI state, no additional DL tracking is needed.

- Option 2: (Huawei, MediaTek, Ericsson)

  - No additional DL RS tracking time for UL TCI state switching

The UL TCI state is only for beam indication, but for 2TA operation the UL timing reference is different for each TAG. Based on discussion on the UL timing reference the timing reference for UL with 2 TA is defined per TAG and is the DL timing reference of the reference cell for activated DL or joint TCI state with the same TAG as the UL signal. If the UE has a DL timing reference associated with the UL TCI state, then no additional time is needed for time tracking.

- For 2TA case UE has 2 TAG and 2 DL reference timing associated with each TAG.

- In case the UL TCI state has a DL timing reference associated with the TAG, then no additional time is needed for time tracking.

In case for TAG2 the UE is configured with separate UL and DL TCI state with single activated TCI, and both the DL and UL TCI state switch, then the UL signal has no DL timing reference for TAG2. In this case we have 2 options:

- Option 1: Additional time for time tracking based on the DL Reference RS is needed for UL TCI state switch.

- Option 2: Do not define requirements for this case

- In case both DL and UL TCI state switch, and there is DL TCI state in the active TCI state list associated with the TAG, the following options are possible:
Option 1: Additional time for time tracking is needed for UL TCI state switch.
Option 2: No requirements are applicable for this case

For the case of separate UL TCI state switch if no DL TCI is in the active TCI state list associated with the TAG of the target UL TCI state, choose from one of the two options above.

- For multi-DCI multi-TRP with 2TA for separate UL TCI state switch, if no DL TCI is in the active TCI state list associated with the TAG of the target UL TCI state choose one of the 2 options -
    Option 1: Additional time for DL timing reference tracking is needed for the UL TCI state switch
    Option 2: No requirements are applicable for this case

# 3. Conclusion

In this paper, we provide our views on open issues on RRM requirements for mTRP extension to unified TCI framework. Our observations and proposals are captured below:

2TA with multi-DCI  multi-TRP

- For multi-DCI multi-TRP with 2TA the timing requirements are applicable to PUCCH/PUSH/SRS and PDCCH ordered RACH for CFRA

- The reference point for PUCCH/PUSCH/SRS for each TAG is the first detected path in the reference cell associated with the activated DL or joint TCI state having the same TAG as the UL signal.

- RAN4 shall define the reference point for PDCCH order RACH for CFRA for inter-cell and intra-cell scenario.

- For multi-DCI multi-TRP operation with 2TA the UE has gradual timing adjust requirements applicable to each TAG.

- For multi-DCI multi-TRP operation with 2TA the UE has MTTD requirements applicable to pair of TAGs.

- While the UE is applying gradual timing adjustment for each TAG, the TTD for the pair of TAGs could exceed the MTTD requirement.

- Similar issue with legacy gradual timing adjustment and MTTD for CA to be discussed once RAN4 concludes on this issue for mTRP requirements.

- For multi-DCI multi-TRP with 2TA the gradual timing adjustment is applied until the timing error for each TAG is within ±Te or the TTD between pair of TAGs doesn’t exceed the MTTD requirement for 2TAs.

Multi-DCI mTRP

- For 2TA case UE has 2 TAG and 2 DL reference timing associated with each TAG.

- In case the UL TCI state has a DL timing reference associated with the TAG, then no additional time is needed for time tracking.

- In case both DL and UL TCI state switch, and there is DL TCI state in the active TCI state list associated with the TAG, the following options are possible:
Option 1: Additional time for time tracking is needed for UL TCI state switch.
Option 2: No requirements are applicable for this case

- For multi-DCI multi-TRP with 2TA for separate UL TCI state switch, if no DL TCI is in the active TCI state list associated with the TAG of the target UL TCI state choose one of the 2 options -
    Option 1: Additional time for DL timing reference tracking is needed for the UL TCI state switch
    Option 2: No requirements are applicable for this case

# Reference

- R4-2410317, “WF on RRM requirements for NR_MIMO_evo_DL_UL”, Samsung.

- R4-2413010, “CR to TS 38.133 on UL Transmit timing for MIMO Evolution.”, Ericsson, Qualcomm, Apple, Nokia, Vivo.

- R4-2411385, “CR on gradual timing adjustment for 2TA”, Apple.