3GPP TSG-RAN WG4 Meeting #112						R4-2411392

Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.18.3

Source:	Apple

Title:	On RRM Requirements for MIMO Evolution

Release:	Rel-19

Document for:	Discussion

# 1. Introduction

In R19 we have a new WI for MIMO evolution [1]. In the first meeting for RRM core requirements discussion we present our views on impact to RRM requirements from NR MIMO Phase 5.

# 2. Discussion

The objectives of the work item are listed below:

- UE Initiated beam management

- CSI enhancements for up to 128 CSI-RS ports

- CJT enhancements

- Non-coherent UL codebook for 3-antenna port transmissions

- Asymmetric DL sTRP/UL mTRP

We discuss below the impact to RRM requirements based on each of the features introduced in this WI.

UE Initiated Beam Management

For this feature RAN1 will specify the following enhancement:

The UE initiated/event-driven beam management aims at reducing the overhead/latency for beam measurement reporting. The UE reports a measurement based on a trigger event rather than a pre-configured measurement report. The events considered in RAN1 discussion are:

- Event-1: Quality of the current beam is worse than a certain threshold.

- Event-2: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the current beam.

- Event-3: Quality of a new beam is better than a certain threshold.

- Event-4: Quality of the current beam is worse than a threshold 1, and quality of at least one new beam is better than a threshold 2.

- Event-5: Absolute value of the difference between the quality of the current beam and the quality of at least one new beam is lower than a threshold.

- Event-6: When the current beam is not in the best K>1 beams (out of configured beams for measurement and reporting).

- Event-7a: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the worst quality.

- Event-7b: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the RS derived from the activated TCI state with the best quality.

- Event-8: Quality of M>1 new beams, such as L1-RSRP, become a threshold value better than the current beam.

- Event-9: Quality of at least one new beam, such as L1-RSRP, becomes a threshold value better than the configured reference RS (can be SSB or CSI-RS).

Of the events above, RAN1 has only discussed details for Event-2 so far, some of them are descoped while some of them might be discussed in future meetings.

For Event-2 there are 2 possible reporting modes –

For beam report transmission procedure for UE-initiated beam reporting, following modes are supported:

- Mode A  - dynamically scheduling UCI by gNB

- Mode B - UCI in pre-configured resource(s) for second UL channel

Since the UE initiated beam reporting requires L1 measurement and reporting, we expect to see RRM core requirement impact from this feature.

- UE initiated/event-triggered beam reporting has impact to RRM core requirements.

CSI enhancements for up to 128 CSI-RS ports

For supporting > 32 TX ports RAN1 shall specify the following:

To support larger number of TX ports, RAN1 shall specify enhancement to Type I and Type II codebook and CRI reporting. There is no RRM impact we foresee from this enhancement.

- No impact to RRM core requirements with CSI enhancements for up to 128 CSI-RS ports.

CJT enhancements

For CJT deployments RAN1 is introducing CJT calibration reporting – delay offset, frequency offset, phase offset reporting. The reporting range for the offsets are:

Frequency Offset: [0 , AFO]; AFO={0.1ppm, 0,2ppm}

Delay Offset: [0 , AD]; AD={0.5CP, CP}

Phase: [0 , 2];

The reporting range and resolution for the reported quantities are very small. It would not be feasible to define any meaningful accuracy requirements for the newly introduced offset reporting for CJT.

- The delay, frequency and phase offset range and resolution are small to define any accuracy requirements.

The offset measurements would follow the reporting delay for aperiodic measurement report. Hence, we don’t see any RRM core requirements impact with CJT enhancements.

- No impact to RRM core requirement impact with CJT calibration reporting enhancement.

RAN4 could potentially define mapping tables for the delay quantities of not covered in RAN1 spec in performance part.

- RAN4 to discuss introducing mapping tables for the delay, frequency, phase offsets in performance part.

Non-coherent UL codebook for 3-antenna port transmissions

For 3TX support RAN1 is introducing non-coherent UL codebook. Here is no RRM impact to introduction of 3TX codebook.

- No impact to RRM core requirement impact with 3TX codebook enhancement.

Asymmetric DL sTRP/UL mTRP

For asymmetric DL sTRP/ UL mTRP RAN1 is introducing a pathloss offset for the UL only TRP transmission. The pathloss offset can be updated by MAC -CE like TCI state. We introduced pathloss RS switching requirements in Rel-16 for eMIMO, the pathloss offset update would be very similar to that. Based on progress so far, it is not clear if there is any new impact to RRM core requirements.

- RAN4 is introducing a path loss offset to support UL on TRP transmission.

- The pathloss offset can be activated by MAC-CE similar to pathloss RS

- Pathloss offset update requirement would be similar to pathloss switching requirements introduced in eMIMO

There is also a discussion whether 2TA support is in the scope of this feature. In case it is we anticipate impact to RRM core requirements and needs further discussion.

- It is under discussion if 2TA is supported for asymmetric DL sTRP/ UL mTRP.

- There could be RRM impact with support of 2TA for asymmetric DL sTRP/ UL mTRP.

Our assessment of the impact to RRM requirements can be summarized below:

| Feature | RRM Impact |
| --- | --- |
| UE Initiated Beam management | RRM core requirement impact with event triggered measurement report |
| CSI support for up to 128 CSI-RS ports | No RRM impact |
| Reporting enhancement for CJT deployments | No RRM core requirement impact Discuss if Delay offset, Frequency offset Phase offset mapping tables to be introduced in performance part |
| Non-coherent UL codebook to for 3TX | No RRM impact |
| Asymmetric DL sTRP/UL mTRP | No RRM impact yet, other than requirements similar to Pathloss switching Wait for progress in RAN1 to understand impact to RRM |

Based on the above observations, we propose to further discuss the following requirements in RRM:

- RRM requirements for UE initiated beam management

- RRM requirements for Asymmetric DL sTRP/UL mTRP with 2TA based on progress in RAN1

Further discuss RRM requirements for UE initiated beam management

RAN4 to discuss RRM requirements for Asymmetric DL sTRP/UL mTRP with 2TA based on progress in RAN1

# 3. Conclusion

In this paper, we provide our views on impact to RRM requirements from NR MIMO Phase 5. Our observations and proposals are captured below:

- UE initiated/event-triggered beam reporting has impact to RRM core requirements.

- No impact to RRM core requirements with CSI enhancements for up to 128 CSI-RS ports.

- The delay, frequency and phase offset range and resolution are small to define any accuracy requirements.

- No impact to RRM core requirement impact with CJT calibration reporting enhancement.

- RAN4 to discuss introducing mapping tables for the delay, frequency, phase offsets in performance part.

- No impact to RRM core requirement impact with 3TX codebook enhancement.

- RAN4 is introducing a path loss offset to support UL on TRP transmission.

- The pathloss offset can be activated by MAC-CE similar to pathloss RS

- Pathloss offset update requirement would be similar to pathloss switching requirements introduced in eMIMO

- It is under discussion if 2TA is supported for asymmetric DL sTRP/ UL mTRP.

- There could be RRM impact with support of 2TA for asymmetric DL sTRP/ UL mTRP.

- Further discuss RRM requirements for UE initiated beam management

RAN4 to discuss RRM requirements for Asymmetric DL sTRP/UL mTRP with 2TA based on progress in RAN1

# Reference

- RP-240087, “WID revision: NR MIMO Phase 5”, Samsung.