3GPP TSG-RAN WG4 Meeting #112                                         	                     R4-2411393

# Maastricht, Netherlands, August 19 – August 23, 2024

Agenda item:	8.1.1.3.2

Source:	Apple Inc.

Title:	Views on Maximum Number of MIMO Layers for 6Rx UEs

Release:	Rel-19

Document for:	Discussion

# Introduction

A new WID on RF enhancements for NR FR1/FR2 and EN-DC has been created for Release 19. The WID covers three main areas: High power UE (HPUE) for CA in terrestrial network (TN), power boosting and/or MPR reduction, and 6Rx for handheld and FWA UEs. The main objectives for 6Rx, as highlighted in the WID [1], are captured below:

Specify the core requirements to enable 6Rx for higher frequency bands (>2.5GHz) targeting at support of handheld UE for NR FR1 single carrier scenario

  - Example bands: n41, n77/n78, n79, n104

  - Support 4 MIMO layers at least, and study the gain and feasibility and if feasible, support 6 MIMO layers

  - Specify the Rx requirements including reference sensitivity requirements for support 6Rx

    - Note: the specified requirements can be applicable to both handheld UE and FWA devices

  - Specify the requirements to support SRS antenna switching including t1r6, t2r6, t3r6, t4r6 depending on UE capability

  - Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding solution

Study the issue of insertion loss imbalance across SRS ports, and if justified, specify the corresponding

In this discussion paper, we share our views on the maximum number of MIMO layers for 6Rx handheld UEs. Enabling 6Rx for large form-factor handheld UEs has the potential of improving DL coverage and throughput. However, it is important to understand if supporting 6 layers vs. 4 layers can produce practical throughput gains. We continue discussing our views below, including supporting simulation results.

# Discussion

One of the main objectives of this WID is 1) to support 4 MIMO layers as a baseline, and 2) study the gain and feasibility of supporting 6 MIMO layers with a 6Rx UE. Increasing the number of antennas to 6Rx for larger form-factor UEs may have the potential of improving DL coverage. However, having more antennas in the same panel in a limited area will make things challenging with respect to MIMO operation. In general, achieving full rank requires good antenna isolation and low correlation, which could be difficult to maintain in these circumstances.

In addition to this, another point worth discussing is the assumptions on Tx EVM commonly considered. For 256QAM a common assumption is 3% EVM. We believe that a discussion on whether to tighten this common assumption should start after 1) simulation parameters are agreed, and 2) interested companies report their simulation results and conclusions.

Observation #1: Common assumptions for Tx EVM are 6% for QPSK/16QAM and 3% for 256QAM.

Proposal #1: Postpone discussion on whether to tighten BS EVM requirements after simulation assumptions are agreed and interested companies have reported simulation results and conclusions.

On the other hand, as per the WID, the specified requirements can be applicable to both handheld UE and FWA devices. Our interpretation is that RAN4 will only define one set of requirements, and these requirements should be applicable for both types of form factors. As per current specification, it’s not a common practice to make a distinction between types of UE devices. This also means that in principle, the 6 MIMO layers capability cannot be precluded from one type of device.

Observation #2: As per WID, the specified requirements can be applicable to both handheld UE and FWA devices.

Proposal #2: RAN4 should initially define a single set of demodulation requirements that apply to both handheld UE and FWA devices.

One potential outcome of this study could certainly be that performance gains of 6 layers vs 4 layers only materialize under low Rx antenna correlation, possibly depending on the exact simulation assumptions being made. Under this scenario, RAN4 could take into consideration that a lower correlation is only realistically practical for FWA devices, even though FR1 demodulation requirements are conducted requirements (antenna correlation would not be perceived). Making 6 layers support an optional feature may help fulfil the purpose stated in the WID.

Proposal #3: RAN4 to confirm that 6-Layer support is an optional feature.

On another point, as discussed in the previous WF, the most important assumption that companies need to agree is the assumed MIMO correlation. We believe that for a practical analysis, and without relying in too many implementation details, we decided to reuse the existing 3GPP MIMO correlation models. With this, we could observe an order of magnitude of the relative performance gains, if any, and get a good idea of whether refining the antenna correlation model can really change the outcome of the study. For example, if initial simulation results already show that performance gains are unlikely even under a conservative MIMO correlation model, this would save valuable standardization time and resources.

    - Overall, propose the simulation assumptions stated below, as a starting point.

Proposal #4: RAN4 to consider the following baseline PDSCH simulation assumptions.

Table 1: Baseline simulation assumptions for PDSCH performance

| Parameter |  | Value |
| --- | --- | --- |
| Duplex mode |  | FDD/TDD |
| TDD pattern |  | 7D1S2U S=6D:4G:4U |
| SCS(kHz)/Bandwidth (MHz) |  | TDD: 30/40 |
| MIMO layer and Antenna configuration |  | Rank 4: 8x6 Rank 6: 8x6 |
| Propagation condition and antenna correlation |  | ULA MediumB ( = 0.3, = 0.005154) ULA MediumA ( = 0.3, = 0.3874) |
| MCS |  | 64QAM Table: MCS17, MCS19 256QAM Table: MCS21, MCS23, MCS25 |
| PDSCH configuration | Mapping type | Type A |
|  | k0 | 0 |
|  | Starting symbol (S) | 2 |
|  | Length (L) | 12 |
|  | PRB bundling type | Static |
|  | PRB bundling size | 2 |
| PDSCH DMRS configuration | DMRS Type | Type 1 |
|  | Number of additional DMRS | 1 |
|  | Maximum number of OFDM symbols (maxLength) for DL | Double-symbol |
| TRS configuration |  | Symbol#{5, 9} |
| PT-RS |  | Not configured |
| NZP-CSI-RS configuration |  | row 6 |
| Overhead |  | 0 |
| N1 and N2 configurations for 8Tx cases |  | Use (N1,N2) = (4,1), (O1, O2) = (4,1) |
| Coodebook for PDCCH for PDSCH tests with 4Tx and 8Tx |  | Keep same number of Tx for PDSCH and PDCCH during PDSCH test. Set “codebookMode” to 1 |
| Special slot (S slot) scheduling |  | Not schedule PDSCH in special slot |
| Number of HARQ Processes |  | FDD: 4, TDD: 8 |
| HARQ ACK/NACK bundling |  | Not configured |
| Maximum HARQ transmissions |  | 1 |
| Redundancy version coding sequence |  | {0} |
| PDSCH & PDSCH DMRS Precoding configuration |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with PRB bundling granularity |
| Tx EVM (Explicitly modeled in the simulation) |  | 64QAM: 6% 256QAM: 3% |
| Test metric |  | SNR@70% max Throughput |

(Other common parameters, refer to TS 38.101-4: Table 5.2-1: Common test parameters for PDSCH)

Taken from the simulation parameters above, we have run some initial simulation results.

Figure 1: Simulation results for 6Rx UE comparing Rank 4 and Rank 6 under MediumB correlation in TDLA30-10 channel

In Figure 1 we compare Rank 4 and Rank 6 under TDLA30-10, MediumB correlation, displaying absolute throughput as the metric. We consider the MediumB antenna correlation model as an initial and quite conservative approach of what we would expect a handheld UE would resemble. Under this assumption, we already observe that performance gains can only be seen at very low MCS. At this low MCS, we believe it is unlikely that network deployments would schedule such high ranks such like Rank 4 and Rank 6.

Observation #3: MediumB correlation can be used as a very conservative antenna correlation model of the actual antenna correlation that a handheld UE could achieve.

Observation #4: Simulation results under our assumed MediumB correlation show that Rank 6 outperforms Rank 4 only at very low MCS where such high-rank transmission is not intended.

In addition, we want to see at which point a Rank 4 transmission under MediumB correlation can achieve full throughput. Figure 2 we shows this below based on relative throughput.

Figure 2: Relative throughput comparison for Rank 4 transmission under MediumB correlation in TDLA30-10 channel

As shown, we see that even for Rank 4 the full throughput can only be achieved at very low MCS. However, it is worth noticing that the assumed receiver at this point is a linear MMSE-IRC receiver.

Observation #5: Under these assumptions, Rank 4 transmission under MediumB correlation achieves full throughput only at very low MCS.

Proposal #5: RAN4 to limit to 4 layers the number of MIMO layers in 6Rx unless other simulation assumptions are agreed.

Finally, for informational purposes, we also present simulation results considering this time no antenna correlation in the figure below. This case would resemble a FWA device without form factor constraints.

Figure 3: Simulation results comparing Rank 4 and Rank 6 under Low correlation in TDLA30-10 channel

As show, removing antenna correlation from the problem increases the MCS values at which we can expect reasonable performance. In the figure above, we restrict ourselves to MCSs taken from the 64QAM table only. Here we see that even MCS23 cannot achieve full throughput below the 30dB mark for Rank 6 transmission, which further supports our conclusion. However, other simulation assumptions may show more positive results.

# Conclusions

Our observations and proposals are summarized below:

Observation #1: Common assumptions for Tx EVM are 6% for QPSK/16QAM and 3% for 256QAM.

Proposal #1: Postpone discussion on whether to tighten BS EVM requirements after simulation assumptions are agreed and interested companies have reported simulation results and conclusions.

Observation #2: As per WID, the specified requirements can be applicable to both handheld UE and FWA devices.

Proposal #2: RAN4 should initially define a single set of demodulation requirements that apply to both handheld UE and FWA devices.

Proposal #3: RAN4 to confirm that 6-Layer support is an optional feature.

Proposal #4: RAN4 to consider the following baseline PDSCH simulation assumptions (table above).

Observation #3: MediumB correlation can be used as a very conservative antenna correlation model of the actual antenna correlation that a handheld UE could achieve.

Observation #4: Simulation results under our assumed MediumB correlation show that Rank 6 outperforms Rank 4 only at very low MCS where such high-rank transmission is not intended.

Observation #5: Under these assumptions, Rank 4 transmission under MediumB correlation achieves full throughput only at very low MCS.

Proposal #5: RAN4 to limit to 4 layers the number of MIMO layers in 6Rx unless other simulation assumptions are agreed.

# References

- RP-240720 “New WID: UE RF enhancements for FR1/FR2 and EN-DC, Phase 4”, by RAN4 chair (Huawei), 3GPP TSG-RAN Meeting #103, Maastricht, Netherlands, March 18-21, 2024