3GPP TSG-RAN4 Meeting  #112	R4-2411398

# Maastricht, NL, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4660 | rev | - | Current version: | 17.14.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) CR on active TCI state list update delay - R17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2024-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There is ambiguity/inaccuracy in the current wording in active TCI state list update delay requirement. In particular, it is unclear if a UE should expect when the network transmits PDCCH, as PDCCH is up to network scheduling and the network could transmit PDCCH before the UE completes active TCI state list update. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the wording to the following: “If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for activation/deactivation of UE-specific PDSCH TCI state as defined in clause 6.1.3.14 of TS 38.321 [7] at slot n, UE shall be able to receive PDCCH, which schedules PDSCH with the new target TCI state, at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length, and the UE is not expected to receive such PDCCH before that. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3.” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is confusion in interpreting the requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.10.6, 8.10A.6, 8.10B.6, 8.10C.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

### 8.10.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for activation/deactivation of UE-specific PDSCH TCI state as defined in clause 6.1.3.14 of TS 38.321 [7] at slot n, UE shall be able to receive PDCCH, which schedules PDSCH with the new target TCI state, at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length, and the UE is not expected to receive such PDCCH before that. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10.3.

# <End of Change #1>

# <Start of Change #2>

### 8.10A.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH, which schedules PDSCH with the new target TCI state at the first slot that is after n + 3Nslotsubframe,µ + (THARQ +TOk*(Tfirst-SSB + TSSB-proc+TSSB*LMAC,known)) / NR slot length, and the UE is not expected to receive such PDCCH before that. Where THARQ, Tfirst-SSB, TSSB-proc  , TSSB, LMAC,known and TOk are as defined in clause 8.10A.3.

# <End of Change #2>

# <Start of Change #3>

### 8.10B.6	Active TCI state list update delay

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH, which schedules PDSCH with the new target TCI state, at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length, and the UE is not expected to receive such PDCCH before that. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10B.3.

# <End of Change #3>

# <Start of Change #4>

### 8.10C.6	Active TCI state list update delay

Upon receiving PDSCH carrying MAC-CE active TCI state list update at slot n, UE shall be able to receive PDCCH, which schedules PDSCH with the new target TCI state, at the first slot that is after n+ THARQ +3Nslotsubframe,µ +TOk*(Tfirst-SSB + TSSB-proc) / NR slot length, and the UE is not expected to receive such PDCCH before that. Where THARQ, Tfirst-SSB, TSSB-proc and TOk are defined in clause 8.10C.2.

# <End of Change #4>