3GPP TSG-RAN WG4 Meeting #112		R4-2411400

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	5.13.1

Source:	Apple

Title:   	On RRM core requirement maintenance for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

In this contribution, we focus the discussion on the following issues with an aim to improve specification text or FFS:

- When is UE considered to be in multi-RX operation

- Maximum receive timing difference requirement

- Measurement restriction for sDCI and mDCI

# 2	Discussion

## 2.1 When is UE considered to be in multi-RX operation

As captured in the moderator summary [1]:

Issue 1-1-1: When is UE considered to be in multi-rx operation - Proposals - Proposal 1: Keep the agreed definition - Given UE indication of its preference of multi-RX/single-RX operation has been allowed, it is OK to take the current definition of multi-RX operation - Proposal 2: Add following additional condition - P2a: - UE sent a recent valid Rel-17 group-based beam reporting (GBBR). - If UE recently reported ‘Not valid’ for one of the RSRP for a beam pair, this means UE is allow to fallback to single panel for the later reception QCL-ed to that beam pair. - P2b: Rel-17 group-based beam reporting (GBBR) is activated/triggered by the network.

There was discussion on whether the previously agreed definition of multi-RX operation should be revised. In the end, the definition was kept unchanged due to the lack of consensus to change. As it stands, the definition is as follows:

- For fast beam sweeping, the UE is in multi-Rx operation if following condition is met:

  - UE is configured with group-based beam reporting (GBBR) report.

We are OK with the current definition. As a result, when the CR was drafted for 38.133 on applicability of requirements multi-Rx operation in FR2-1, the following wording was used (copied below)

To have accurate specification text, we think the above wording needs to be improved for the following reason. RAN4 has agreed to use UAI to allow the UE to indicate its preference of multi-RX or single-RX operation. Furthermore, neither the network behavior nor UE behavior after UAI indication to the network has been defined. Therefore, it is at least unclear if the requirement would apply to the case where the network configures GBBR to the UE and then the UE indicates via UAI that it prefers to fall back from multi-RX operation to single-RX operation.

Observation 1: Neither the network behavior nor UE behavior after UAI indication to the network has been defined. Therefore, it is at least unclear if the requirement would apply to the case where the network configures GBBR to the UE and then the UE indicates via UAI that it prefers to fall back from multi-RX operation to single-RX operation.

As such, we propose to modify the text a bit to ensure there is no misunderstanding about the requirement applicability.

Proposal 1: It is proposed to change in “3.6.19       Applicability of requirements for multi-Rx operation in FR2-1” from “The requirements related to the support of [reducedRxBeamNum] is applicable when the network configures the UE with a CSI report containing groupBasedBeamReporting-v1710.” To “The requirements related to the support of [reducedRxBeamNum] is applicable when the network has configured the UE with a CSI report containing groupBasedBeamReporting-v1710 and the UE has not sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.”

## 2.2 Maximum receive timing difference requirement

In 38.133, the maximum receive timing difference requirement for multi-RX operation is captured as follows:

There is an FFS on Note 1 in the table. The reason to have such as FFS in our understanding is to consider the case where there are different SCSes used. In R18, the targeted scenario is intra-cell multi-TRP, where a UE can receive PDSCH or perform measurements simultaneously. In such a scenario, it can be assumed that the two involved TRPs would deploy the same SCS in the DL. Then the only issue is if the SSB SCS and data SCS would be the same or different. In our understanding, it is possible to have different SSB SCS (120/240kHz) and date SCS (60/120kHz). Therefore, it is reasonable to consider the maximum of SSB SCS and data SCS. At the same time, it is recognized that it is challenging to maintain MRTD < CP length of 240kHz SCS from network deployment perspective. We are open to further discussions. We make proposal 2 below.

Proposal 2: Considering that SSB SCS (120/240kHz) and date SCS (60/120kHz) can be different, it is proposed that the MRTD is smaller than the CP length corresponding to MAX (SSB SCS, data SCS). It is recognized that it is more challenging to maintain MRTD < CP length of 240kHz SCS from network deployment perspective.

## 2.3 measurement restriction for sDCI and mDCI

After last meeting, the issue 1-3-2 was still open. There are two cases for consideration in our view, mTRP sDCI and mTRP mDCI. We understand the analysis in [1], and agree to remove the condition of “The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol” from measurement restriction relaxation requirement for mTRP sDCI case. However, for mTRP mDCI, we have one example that,

The TRP1 PDSCH #1 TCI is indicated by PDCCH#1, and use the corresponding beam for reception, however, on slot#2 from TRP2, the beam used to buffer the part after PDCCH of slot #2 could be based on the RAN1 definition as: the UE may assume that the DM-RS ports of PDSCH associated with a value of coresetPoolIndex of a serving cell are quasi co-located with the RS(s) with respect to the QCL parameter(s) used for PDCCH quasi co-location indication of the CORESET associated with a monitored search space with the lowest controlResourceSetId among CORESETs. In our view, there is no restriction that this by-default beam must be paired with the beam of PDSCH#1 which is indicated by PDCCH#1. We assumed that if PDSCH are scheduled on overlapped symbols, network will somehow help to figure out the beam pair for PDSCH reception at UE (at least to make sure it won’t use un-paired beam for overlapping data scheduling). Thus, for mDCI mTRP case, if there are no PDSCH overlapped with CSI-RSs for L1, it can still cause measurement restriction.

Proposal 3: For sDCI mTRP case, remove the following conditions for measurement restriction relaxation:

- The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol

Proposal 4: For mDCI mTRP case, conditions that measurement restriction for CSI-RS based L1 measurements can be relaxed for multi-Rx are:

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON

- The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol.

- One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH

- Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

- UE is activated with multi-Rx operation

# 3	Conclusions

In this contribution, we make the following proposals.

Observation 1: Neither the network behavior nor UE behavior after UAI indication to the network has been defined. Therefore, it is at least unclear if the requirement would apply to the case where the network configures GBBR to the UE and then the UE indicates via UAI that it prefers to fall back from multi-RX operation to single-RX operation.

Proposal 1: It is proposed to change in “3.6.19       Applicability of requirements for multi-Rx operation in FR2-1” from “The requirements related to the support of [reducedRxBeamNum] is applicable when the network configures the UE with a CSI report containing groupBasedBeamReporting-v1710.” To “The requirements related to the support of [reducedRxBeamNum] is applicable when the network has configured the UE with a CSI report containing groupBasedBeamReporting-v1710 and the UE has not sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.”

Proposal 2: Considering that SSB SCS (120/240kHz) and date SCS (60/120kHz) can be different, it is proposed that the MRTD is smaller than the CP length corresponding to MAX (SSB SCS, data SCS). It is recognized that it is more challenging to maintain MRTD < CP length of 240kHz SCS from network deployment perspective.

Proposal 3: For sDCI mTRP case, remove the following conditions for measurement restriction relaxation:

- The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol

Proposal 4: For mDCI mTRP case, conditions that measurement restriction for CSI-RS based L1 measurements can be relaxed for multi-Rx are:

- Both CSI-RSs are not in any CSI-RS resource set with repetition ON

- The two CSI-RS resources and both PDSCHs are overlapped on the same OFDM symbol.

- One CSI-RS has same QCL source as the active TCI state of one PDSCH, and the other CSI-RS has same QCL source as the active TCI state of the other PDSCH

- Resources of the active TCI states for the two PDSCHs have been reported as a resource group in Rel-17 group-based RSRP report.

- UE is activated with multi-Rx operation

# 4	References

R4-2408000, "Topic summary for [111][203] FR2_multiRx_part1," Moderator (Ericsson)