3GPP TSG-RAN4 Meeting  #112	R4-2411401

# Maastricht, NL, August 19th – 23rd, 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4662 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on multi-RX core performance maintenance |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR2_multiRX_DL-Core |  |  |  |  |  | Date: |  |  | 2024-08-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | First, in 3.6.19 Applicability of requirements for multi-Rx operation in FR2-1 it is at least unclear if the requirement would apply to the case where the network configures GBBR to the UE and then the UE indicates via UAI that it prefers to fall back from multi-RX operation to single-RX operation, since neither the network behavior nor UE behavior after UAI indication to the network has been defined. Second, in Table 7.6.8-3: Maximum receive timing difference requirement for UE supporting multi-RX, there is FFS on whether CP length depends on SCS. This FFS needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | First, in Clause 3.6.19, Change the wording from “The requirements related to the support of [reducedRxBeamNum] is applicable when the network configures the UE with a CSI report containing groupBasedBeamReporting-v1710.” To “The requirements related to the support of [reducedRxBeamNum] is applicable when the network has configured the UE with a CSI report containing groupBasedBeamReporting-v1710 and the UE has not sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.” Second, in Table 7.6.8-3, change Note 1 to “CP length is determined by the maximum of SSB SCS and data SCS.” Third, the capability IEs are updated following RAN2 IE names. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | There is confusion in interpreting the requirement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.6.19, 7.6.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of Change #1>

### 3.6.19	Applicability of requirements for multi-Rx operation in FR2-1

The requirements related to the support of [fastBeamSweepingMultiRx-r18 and FG 30-1] are applicable to a PCell, PSCell, or SCell, provided the UE is configured with a single serving cell (PCell, PSCell, or SCell) in FR2-1.

The requirements related to the support of [fastBeamSweepingMultiRx-r18] is applicable when the network has configured the UE with a CSI report containing groupBasedBeamReporting-v1710 and the UE has not sent UAI indicating its preference of single-RX operation since it was most recently configured with such CSI reporting.

# <End of Change #1>

# <Start of Change #2>

A UE supporting [FG 30-1 or fastBeamSweepingMultiRx-r18] shall be capable of handling at least a relative receive timing difference between slot timing of different TCI states on the same carrier at the UE receiver as shown in Table 7.6.8-3.

Table 7.6.8-3: Maximum receive timing difference requirement for UE supporting multi-RX

| Frequency Range | Maximum receive timing difference |
| --- | --- |
| FR2-1 | CP lengthnote 1 |
| Note 1: CP length is determined by the maximum of SSB SCS and data SCS. |  |

# <End of Change #2>