3GPP TSG-RAN WG4 Meeting #112		R4-2411402

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	5.13.2

Source:	Apple

Title:   	On RRM performance requirement maintenance for NR FR2 multi-Rx chain DL reception

Document for:	Discussion

# 1	Introduction

At the last meeting, the big CR on performance requirements was agreed [1]. Meanwhile, there are still some open issues on the two/three AoA setups. In this contribution, we continue discussing the issue to finalize the corresponding specification text.

- Definition of qualified AoA pairs

- How to ensure such pairs are found in RF tests

# 2	Discussion

## 2.1 2 AoAs for simultaneous reception with QCL Type-D

The current specification text is as follows [1]:

### A.3.15.5 Setup 5: 2 AoAs for simultaneous reception with QCL Type-D There are 2 active probes in the test. The DL signals, and noise if applicable, transmitted from the two active probes. The 2 AoAs [(AoA1 and AoA2)] for simultaneous reception with different QCL-typeD is from the set of [qualified AoA pairs] according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 for power class 3 supporting simultaneous reception from multiple directions, the angular separation between the directions (AoAs) of the 2 active probes is declared from Table 7.3K.3-1 in clause 7.3K.3 of TS 38.101-2 [19] and [shall not be changed for each test iteration]. [The relative angular offset between the directions of the AoA pair is based on the UE’s declared [AoA separation and UE] orientation as defined in clause 7.3K.3 of TS 38.101-2.] [One of the AoAs (AoA1) needs to satisfy the spherical coverage requirement in Table 7.3.4.3-1 of TS 38.101-2]. Editor’s note: FFS how single AoA RF test and 2 AoA RF test could ensure this. ### A.3.15.6 Setup 6: 3 AoAs for simultaneous reception with different QCL Type-D There are 3 active probes in the test and the DL signals and noise are transmitted from the three active probes. Out of the three AoA, one AoA [(AoA1)] is aligned to a direction which is from the set of directions corresponding to the EIS spherical coverage percentile of the DUT as defined in clause 7.3.4 of TS 38.101-2 [19] for UE power class 3 and other 2 AoAs [(AoA2, AoA3)] are from the set of [qualified AoA pairs] according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 for power class 3 supporting simultaneous reception from multiple directions. The relative angular offset between the directions of the AoA pair is based on the UE’s declared [AoA separation and UE] orientation as defined in clause 7.3K.3 of TS 38.101-2 and [shall not be changed for each test iteration].

•	Definition of qualified AoA pairs

As there is no such a term as “qualified AoA pairs” defined in 38.101-2, it may be subject to different interpretation even with the modifier “according to the spherical coverage requirement for simultaneous reception from multiple directions as defined in clause 7.3K.3 of TS 38.101-2 for power class 3 supporting simultaneous reception from multiple directions.” Therefore, it is better to avoid using the word “qualified”. Based on the current text in 7.3K.3 of TS 38.101-2, we propose to use the following wording:

Proposal 1: It is proposed to use the wording “the AoA pairs, i.e., (AoA1, AoA2), that can support 2 AoA reception.”

Note that the AoA pairs that can support 2 AoA reception requires further clarification, as shown in the discussion below.

How to ensure such pairs are found in RF tests

In our understanding based on the discussion with TE vendors, the test setup for the RF requirement would look like the following, where the probes are placed on the XZ plane and with an angular offset from the Z axis of 0/30/90/150 degrees.

Given an AoA separation as declared by the UE, say 30 degrees, the two probes, denoted by P0 and P30 would be used. In the two AoA setup, the number of AoA1s (say represented by probe P0) that would satisfy the legacy or single AoA spherical coverage requirement can be obtained in the legacy single AoA RF test. Let us denote all such AoA1s as Set 1. Similarly, the number of AoA pairs (AoA1 an AoA2, represented by P0 and P30) that can support 2 AoA reception can be obtained in the 2 AoA RF test. Note in RF test, pair (AoA1, AoA2) and pair (AoA2, AoA1) are different, or more specifically, one is with +AoA offset (AoA2 – AoA1) and the other with -AoA offset (AoA1 – AoA2). Since it is possible that one pair may pass the throughput test but not the other, we need to be precise in which pair we indicate in the test. Let us denote all such AoA pairs, i.e., (AoA1, AoA2), as Set 2. Then in the RRM test, we can choose one AoA pair, (AoA1, AoA2), where AoA1 belongs to Set 1 and (AoA1, AoA2) belong to Set 2.

In the rare case where no (AoA1, AoA2) pairs are found, we can reverse the order of (AoA1, AoA2) to (AoA2, AoA1) and repeat the above procedure.

Observation 1: An AoA pair, i.e., (AoA1, AoA2), where AoA1 satisfies the legacy or single AoA spherical coverage requirement, and (AoA1, AoA2) supports 2 AoA reception, can be obtained in the single AoA and 2 AoA RF tests.

The above info is for RAN4 understanding, and there is no need to capture it in 38.133. The detailed handling can be left to RAN5 when they specify the test procedure. Therefore, it is proposed to change the Editor’s note to:

Proposal 2: The Editor’s note is changed to “The chosen AoA pair in the test, i.e., (AoA1, AoA2), is up to RAN5.”

We provide a corresponding CR in a companion contribution.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: It is proposed to use the wording “the AoA pairs, i.e., (AoA1, AoA2), that can support 2 AoA reception.”

Observation 1: An AoA pair, i.e., (AoA1, AoA2), where AoA1 satisfies the legacy or single AoA spherical coverage requirement, and (AoA1, AoA2) supports 2 AoA reception, can be obtained in the single AoA and 2 AoA RF tests.

Proposal 2: The Editor’s note is changed to “The chosen AoA pair in the test, i.e., (AoA1, AoA2), is up to RAN5.”

# 4	References

R4-2408282, "Big CR to TS 38.133 on performance requirements for NR FR2 multi-Rx chain DL reception," vivo, Ericsson