3GPP TSG-RAN4 Meeting  #112	R4-2411404

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	8.6.2

Source:	Apple

Title:	On methods for reducing the number of UE Rx chains for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

In this contribution, we share our initial views on the methods of reducing the number of UE Rx chains and the means for a UE to inform the network.

# 2	Discussion

Prerequisite conditions

First, to allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, as analyzed in our companion contribution [2], the blocker power level in the gap needs to be manageable to the UE. In other words, the blocker power level needs to be at an appropriate level that the UE can handle with common RF chain. It can either be a specified value in the spec or based on UE capability reporting. The Rx chain configuration/reconfiguration may be based on some assistant information by UE measurement of the blocker.

Proposal 1: To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, the blocker power level in the gap needs to be at an appropriate level that the UE can handle with common RF chain.

Second, the UE needs to be able to switch between common RF chain and separate RF chain. During such RF chain switching, the UE may experience interruption in its TX and RX. Such impact should be further investigated in RAN4, including the interruption time. Such interruption may affect current RRM requirement.

Finally, once an interruption time is determined, a discussion on how often Tx/Rx switching the network is allowed to schedule should follow. For example, a minimum time between switching events could be standardized, in order to reduce the reconfiguration effort and extra power consumption that this mechanism can bring to the UE.

Proposal 2: The impact of UE switching between common RF chain and separate RF chains, including potential interruption time and cadence should be studied. If there is RRM impact, RAN4 should consider it.

Signaling aspects

Besides the UE ability to switch, there needs to be signaling support for UE to inform the network of appropriate CA configuration it can support after switching to common RF chains allowed by adjusted RF requirements. Usually, the CA configurations supported by a UE are reported to the network when the UE camps on a cell and receives capability inquiry from the network. In the case of RF chain switching, however, the UE may need to update its supported CA configuration after it is configured with some CA combination by the network through RRC. An alternative way is that the UE reports to network that it is capable of Rx chain sharing for the supported band combination, either by additional signaling on top of existing band combination reporting or by defining a dedicated band combination reporting for UE supporting Rx chain sharing. This would require RAN2 discussion.

There are two options as to when to trigger RAN2 discussion:

- Option 1: To trigger the RAN2 discussion at this meeting. Therefore, the RAN4 discussion on RF requirements and RAN2 discussion on signaling can progress in parallel

- Option 2: Wait until RAN4 has concluded the RF requirement impact analysis including the blocker power level

Given the target completion time of this SI is June 2025, Option 1 is preferred.

Proposal 3: Between the following two options, Option 1 is preferred.

•	Option 1: To trigger the RAN2 discussion at this meeting. Therefore, the RAN4 discussion on RF requirements and RAN2 discussion on signaling can progress in parallel

•	Option 2: Wait until RAN4 has concluded the RF requirement impact analysis including the blocker power level

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: To allow the UE to support the two non-contiguous CCs with common/shared RF chains instead of separate RF chains, the blocker power level in the gap needs to be at an appropriate level that the UE can handle with common RF chain.

Proposal 2: The impact of UE switching between common RF chain and separate RF chains, including potential interruption, should be studied. If there is RRM impact, RAN4 should consider it.

Proposal 3: Between the following two options, Option 1 is preferred.

•	Option 1: To trigger the RAN2 discussion at this meeting. Therefore, the RAN4 discussion on RF requirements and RAN2 discussion on signaling can progress in parallel

•	Option 2: Wait until RAN4 has concluded the RF requirement impact analysis including the blocker power level

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

R4-2411405, "On UE RF requirements for fragmented carriers," Apple