3GPP TSG-RAN4 Meeting  #112	R4-2411405

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	8.6.3

Source:	Apple

Title:	On UE RF requirements for fragmented carriers

Document for:	Discussion

# 1	Introduction

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

The objective of this study is as follows:

-	Identify methods for reducing the number of UE Rx chains (e.g. from separate RF chains per CC to shared RF chains ) needed for single DL band with frequency span ≤ 100 MHz, containing two non-contiguous CCs within a CA combination for the inter-operator co-located scenario, considering:

-	Which RF requirements could be adjusted for the inter-operator co-located BS scenario, e.g. existing UE RF requirements such as ΔRIBNC, ACS and in-band blocking [RAN4];

- Other requirements are not precluded, if identified

-	The ability to semi-statically switch hardware resources (i.e. Rx chains) [RAN4, RAN2 – See note 2];

-	Up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs [RAN4];

-	Determine a reasonable level for the power spectral density difference between carriers of co-located adjacent channel operators for study [RAN4]

-	Impacts on DL performance [RAN4] ;

-	Means for a UE to inform the network of appropriate CA configuration it can support with adjusted RF requirements [RAN4, RAN2 – See note 2].

At RAN#104, a R19 SI on fragmented carriers was approved [1], with the following objective:

In this contribution, we share our views on UE RF architecture and analyze the impact on the RF requirements, including the blocker power level.

# 2	Discussion

Common RF chain vs. separate RF chains

As the goal of the SI is to study under what conditions a UE can reduce its number of RF chains to support two non-contiguous CCs within a CA combination, it is necessary to establish a common understanding of common/shared RF chain vs. separation RF chains. In Fig. 1, both architectures are shown.

- Common RF chain

- Separate RF chains

Figure 1. Two UE RF architectures (a) Common RF chain (b)Separate RF chains

Note that in the separate RF chain architecture, there is usually an eLNA before the splitting. Of course, there could be architectures of separate RF chains where there is no shared eLNA.

Proposal 1: It is proposed to clarify that the UE architecture that has a common eLNA followed by separate RF chains is considered one of separate RF chains.

Receiver sensitivity

In current CA receiver sensitivity requirement, there is ΔRIBNC to allow relaxation due to TX noise at RX band, i.e., TX noise at SCC, which assumes UE architecture of separate RF chain.

Fig. 2 below (copied from [2]) shows for CA_n25(2A), PCC is closer to SCC in the CA receiver sensitivity requirement setting than the usual TX and RX in a single carrier case.

Figure 2. CA receiver sensitivity setting for band n25

With common RF chain architecture as shown in Fig. 1(a), the TX noise at SCC from the PCC remains the same as the analog BB filter cannot provide any help. However, besides TX noise, there is TX leakage at TX band that may benefit from analog filtering before it goes into digital BB. In this case, since common RF chain architecture uses a wide analog BB filter to cover the overall frequency span from the lower edge of CC1 to the upper edge of CC2, as compared to the separate RF chain architecture where two separate narrow BB filters to cover CC1 and CC2, there is less filter rejection. As a result, the SCC performance may take some hit. This performance hit is expected to be less severe than the TX noise at SCC. More detailed analysis is needed to quantize the SCC performance degradation.

Proposal 2: ΔRIBNC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination.

ACS

In current ACS requirement, P_interference = P_wanted + ACS – SNR. Depending on the CC bandwidth, the ACS requirement ranges from 26 - 33dB, meaning P_interference is 24.5 – 31.5dB higher than P_wanted, assuming the interference is 5MHz wide.

In real co-located deployment, the power difference between blocker (i.e. CC from operator B) and wanted signal (i.e., CCs from operator A) is expected to be lower. However, with the loss of any analog BB filter rejection for in-gap ACS requirement and reduced rejections for out-gap ACS requirement, meeting the requirement is much harder and likely to eat into the UE implementation margins. This would justify limiting the PSD difference between blocker and CCs. In other words, the blocker power level in the ACS requirement needs to be adjusted.

Observation 1: With the loss of any analog BB filter rejection for in-gap ACS requirement and reduced rejections for out-gap ACS requirement, meeting the ACS requirement is much harder and likely to eat into the UE implementation margins. This would justify limiting the PSD difference between blocker and CCs. In other words, the blocker power level in the requirement needs to be adjusted.

In-band blocking

Similarly to the analysis for ACS, it is believed that the blocker level in the in-band blocking requirement needs to be adjusted.

It is important to point out that in the common RF chain case, the image of the blocker may fall onto one of the two CCs generating severe interference that cannot be rejected by any filtering. Similar interference issue takes place for ACS requirement, but the most severe case is in-band blocking case 2, where the power difference between blocker and wanted signal is the largest.

Observation 2: With the loss of any analog BB filter rejection for in-gap and reduced rejections for out-gap, meeting the in-band blocking requirement is much harder and likely to eat into the UE implementation margins. Especially, the image of the blocker in Case 2 would cause the most severe interference because of the large power difference between the blocker and wanted signal. Therefore, the blocker power level in the requirement needs to be adjusted.

In-gap blocker power level

From the analysis of both ACS and in-band blocking requirement, it is justified to limit the blocker power level. Furthermore, the blocker would impact RX dynamic range. For example, consider the extreme case where the gap is 13 times wide than a 5MHz channel, like CA_n3(2A). If we assume the blocker is 65MHz wide and there is 6dB PSD difference between the blocker and one of the two non-contiguous CCs, the blocker power is 17dB higher than the CC power, which would affect the ADC setpoint and the receiver performance. Therefore, it is necessary to limit not only the PSD difference between the blocker and CCs, but also the gap length between the two CCs. Some UE capability reporting could also be considered for UE to report its capability in handling blocker level and the maximum frequency span per shared RF chain. Note also that it was agreed in the SID that there is up to 6 dB DL received power spectral density imbalance between the two non-contiguous CCs.

Proposal 3: It is proposed to set the in-gap blocker PSD to the same as that of the higher PSD CC and limit the gap length.

# 3	Conclusions

In this contribution, we make the following proposals and observations.

Proposal 1: It is proposed to clarify that the UE architecture that has a common eLNA followed by separate RF chains is considered one of separate RF chains.

Proposal 2: ΔRIBNC may need to be updated to reflect the filter rejection reduction of common RF chain compared to separate RF chains for each intra-band non-contiguous CA band combination.

Observation 1: With the loss of any analog BB filter rejection for in-gap ACS requirement and reduced rejections for out-gap ACS requirement, meeting the ACS requirement is much harder and likely to eat into the UE implementation margins. This would justify limiting the PSD difference between blocker and CCs. In other words, the blocker power level in the requirement needs to be adjusted.

Observation 2: With the loss of any analog BB filter rejection for in-gap and reduced rejections for out-gap, meeting the in-band blocking requirement is much harder and likely to eat into the UE implementation margins. Especially, the image of the blocker in Case 2 would cause the most severe interference because of the large power difference between the blocker and wanted signal. Therefore, the blocker power level in the requirement needs to be adjusted.

Proposal 3: It is proposed to set the in-gap blocker PSD to the same as that of the higher PSD CC and limit the gap length.

# 4	References

RP‑241669, "New SID: Study on NR FR1 DL Fragmented Carriers," MediaTek

RP‑241257, "Views on SI of Fragmented Carriers in Rel-19," Huawei, HiSilicon

RP-240498, "Way Forward on Fragmented Carriers in Rel-19," TELUS