3GPP TSG-RAN WG4 Meeting #112		R4-2411406

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	8.19.3

Source:	Apple

Title:   	On UE RRM requirement

Document for:	Discussion

# 1	Introduction

In the latest R19 SBFD WI [1], there is RAN4 objective on UE CLI measurement and reporting, as shown below:

- L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Notes: Following are assumed for L1 based UE-to-UE CLI measurement and reporting - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs - Periodic and semi-persistent reporting can be considered in addition to aperiodic reporting - For the new reporting quantities, at least wideband reporting is supported - The existing CSI processing unit, CPU occupation rule and timeline for L1 beam reporting are reused for L1 UE-to-UE CLI measurement and reporting as starting point

In this contribution, we share our initial views on the corresponding RAN4 RRM requirement.

# 2	Discussion

Measurement quantities

It was agreed to introduce L1-based UE-to-UE CLI measurement and reporting to provide fast CLI mitigation by BS scheduling, as current L3-based CLI measurement and reporting incurs long delay. To this end, new measurement quantifies such as L1-SRS-RSRP and L1-CLI-RSSI will likely be introduced.

Proposal 1: RAN4 is to focus the discussion on RRM requirement for L1-SRS-RSRP and L1-CLI-RSSI, pending further updates from RAN1 if other measure quantities will be specified.

Measurement resources

As captured in the updated WID, the agreement reached in RAN1 is that periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource. In our understanding, this is an enhancement to R16 CLI where resources are only periodic. This would affect measurement period and accuracy requirement as how many samples are needed to achieve what level of accuracy.

Number of measurement samples

The basic question RAN4 needs to ask and conclude is the measurement one-shot (requiring only one sample) or multi-shot (requiring multiple samples)? Or this is something that is configurable? As the main motivation in adopting L1-based CLI measurement and reporting is to reduce measurement delay, it seems that one-shot measurement is more likely. However, as we understand, there is a tradeoff between measurement delay and measurement accuracy. In particular, one-shot measurement may be more susceptible to fast fading.

On the other hand, in R16 L3-based CLI RAN4 agreed to use three samples for SRS-RSRP measurement since more samples do not provide much better accuracy. Therefore, to reduce delay compared to L3-based measurement, it seems reasonable to assume L1-based measurement is one-shot.

Proposal 2: RAN4 is to discuss and decide how many samples are needed for L1-SRS-RSRP.

UE measurement timing

When an aggressor UE transmits SRS for a victim UE to measure, what is the transmit timing for the aggressor UE and what is the receive timing for the victim UE? In other words, how does the victim UE know when to start measuring? This issue is still open in RAN1. RAN4 needs such info when deciding the requirement such as measurement accuracy.

Observation 1: RAN4 needs to wait for RAN1 decision on UE measurement timing before deciding the measurement accuracy requirement.

# 3	Conclusions

In this contribution, we make the following proposals.

Proposal 1: RAN4 is to focus the discussion on RRM requirement for L1-SRS-RSRP and L1-CLI-RSSI, pending further updates from RAN1 if other measure quantities will be specified.

Proposal 2: RAN4 is to discuss and decide how many samples are needed for L1-SRS-RSRP.

Observation 1: RAN4 needs to wait for RAN1 decision on UE measurement timing before deciding the measurement accuracy requirement.

# 4	References

RP-241614, "Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)," Huawei