# 3GPP TSG-RAN WG4 Meeting #112	R4-2411407

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.15.2.1

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#111, which includes open issues for further discussion. In this contribution, we continue the discussion for FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor.

# Discussion

UE power class

FFS the following options and moderator option: - Option 1 (Apple, OPPO, LGE, vivo, HW, Samsung): - RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Option 1a (LGE): - but RAN4 should consider if other power classes could apply the outcome of the WI discussion - Option 2 (CATT): - It is proposed not to restrict the number of configured carriers and applied power class. - Option 3 (Ericsson): - We can have generic requirements for all power classes, maybe PC6 can be precluded, if no specific use cases for some power classes are pursued. - Option 4 (Moderator) - Baseline: RAN4 to consider UE supporting FR2-1 power class 3 as first priority. - Note: whether other power classes could apply the outcome of the WI discussion can be FFS after concluding on PC3. These extra FFS parts will NOT delay the WI completion.

We propose to focus on PC3 UE as first priority for the following two reasons:

- Multi-RX requirements, both RF and RRM, are specified for PC3 UE only in R18. Since this WI is focused on RRM only without the objective to define RF requirements for other power classes, even if we develop RRM requirements for other power class UEs, they cannot be deployed without the corresponding RF requirements.

- FR2 deployment is best suited for fixed or low-mobility UEs. For fixed UEs, the need for L3 measurements is quite limited. The only exception is PC6, which was specified for high-speed trains for CPEs mounted on the train. If there is a clear need, it can be considered later on.

Proposal 1: It is proposed to focus on PC3 UE as first priority.

Other clarification on WID

- FFS: - “For UE supporting multiple-Rx simultaneous reception for L3 delay enhancement” means UE supporting “simultaneous reception of multiple SSBs from different directions of the same target frequency layer inside a SMTC window. But it does not mean “UE can process multiple SSBs from different directions of the target frequency in parallel.”

We agree with this understanding because the processing requires BB resources which may or may not be available. That is why two searchers were assumed in NR when developing the RRM requirements. That said, considering the agreed baseline application scenario as below:

•	The target carrier(s) to be measured: only one carrier in the single FR2-1 band is configured for L3 SSB measurement and

•	UE serving carrier(s): UE is configured with single carrier on FR2-1 band, i.e. FR2-1 PCell without CA/DC.

it can be assumed that both searchers are available for processing SSBs received simultaneously from different directions. However, if other scenarios than the baseline one are considered later, additional processing time is needed.

Proposal 2: It is proposed to allow additional processing time for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement if there is only one searcher available for the processing.

Conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF:

- FFS: multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature - Option 1 (Apple, CATT, ZTE, vivo, CTC, Intel): multi-Rx simultaneous reception of UE is in active mode, which is expected to follow the one specified in Rel-18 for multi-Rx simultaneous reception feature. - Option 1a (ZTE): The UE is in multi-Rx operation if following condition is met: UE is configured with group-based beam reporting (GBBR) report - Option 1b (Intel): The UE is considered activated in multi-Rx simultaneous reception mode when the UE is configured with group-based beam reporting. The UE is considered activated for L3 reporting when the GBBR is configured not long prior to the expected L3 reporting. - Option 2 (LGE): The L3 measurement delay enhancement requirements for UE supporting multi-Rx simultaneous reception can apply if multiple panels are activate and SSBs in a SMTC window can be measured with multiple beams. - Option 3 (CMCC): the conditions for UE to apply L3 measurement delay reduction by optimizing Rx BSF is that multi-Rx simultaneous reception of UE is in active mode, as for whether the condition is same as that for Rel-18 multi-Rx simultaneous reception can be further discussed. - Option 4 (HW, Nokia, Ericsson): Do not reuse the same applicable conditions specified in Rel-18 multi-Rx - Option 4a (QC) : R19 Rx BSF is independent to R18 multi-Rx simultaneous reception feature includes conditions and requirement. - Option 5 (MTK): The conditions in R18 multi-Rx WI do not limit the discussion on having further conditions in R19 for UE supporting multi-Rx to enhance FR2-1 SSB based L3 measurement delay

In order to support Rx BSF reduction for L3 measurement, UE needs to activate multi-panels for supporting simultaneous dual Rx beam reception. We hope this is the common understanding in RAN4.

Since it is important for a UE to save its power, it is not advisable to have the UE always operate with two/more activated panels. Therefore, there is a need to define a mechanism to activate/de-active the multi-RX operation. In Rel-18 multi-Rx discussion, the general understanding is that a multi-Rx capable UE is operating in multi-RX mode if the network has configured the UE with GBBR reporting and the UE has not sent UAI indicating its preference of single-RX operation since it was most recently configured with such GBBR reporting.

At the last meeting, some companies also mentioned if other conditions or scenarios shall be considered to support this enhancement, e.g., when the UE is with low mobility or at cell edge. We are open to such discussions, but would like to obtain more details on the need and how to define such conditions precisely in the specification. It would be useful for RAN4 to establish a understanding that L3 measurement delay reduction is not the only reason to activate multi-RX operation, though it could be one of the reasons.

Proposal 3: For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded.

Scenarios to use L3 measurement delay reduction by optimizing Rx BSF

FFS on the justifications to decide which of the following scenarios shall be considered. • Encourage companies to discuss the expected benefit to considering BSF reduction for the following scenarios. - FFS on following scenarios to decide which ones are considered to use L3 measurement delay reduction by optimizing Rx BSF:

In WID description, it was stated that,

- FR2-1 SSB based L3 measurement delay reduction for connected mode

  - For UE supporting multiple-Rx simultaneous reception on single carrier:

    - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing:

      - Rx beam sweeping factor

We think the RAN4 work shall start with the most baseline aspect, i.e., L3 measurement delay, and after RAN4 had some conclusions on the solutions then it can be extended to other scenarios, e.g., handover, PSCell addition, RRC Re-establishment/RRC Connection Release with Redirection, SCell activation, SCG activation, CGI reading, and the associated SSB synchronization in CSI-RS L3 measurement. Thus, we propose to prioritize scenarios of following ones as staring point for study and then after RAN4 has conclusion on solutions, we can extend to other scenarios:

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

Proposal 4: Prioritize following scenarios as starting points to use L3 measurement delay reduction by optimizing Rx BSF:

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

Proposal 5: After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios, the solutions(s) can be extended to other scenarios, e.g., handover, PSCell addition, RRC Re-establishment/RRC Connection Release with Redirection, SCell activation, SCG activation, CGI reading, and the associated SSB synchronization in CSI-RS L3 measurement.

Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

- Option 1 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Option 2 (CMCC): - For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing RX beam sweeping factor, and the Rel-18 RX beam sweeping factor reduction can be used as baseline. - Option 2a (CTC) - Rx beam sweeping factor methods in Rel-18 multi-Rx/ eFeRRM WI can be considered as baseline for FR2-1 SSB based L3 measurement delay reduction. - Option 3 (Nokia): - RAN4 to discuss the set of values for reduced L3 beam sweeping factor. - Option 4 (Ericsson): - RAN4 checks the applicability of reducing total number of Mmeas_period by optimizing Rx beam sweeping factor in multiple measurement rounds, one example is full Rx beam sweeping factor in first round and reduced Rx beam sweeping factor in later round. - RAN4 checks the applicability of applying reduced Rx beam sweeping factor for L3 measurements provided any other operation with legacy Rx beam sweeping before it, e.g., different Rx beam sweeping factor in Tpss/sss_sync and Tssb_measurement_period.

For SSB based L3 measurement for RRM, the beam sweeping is always assumed on the whole spherical area to detect the potential neighbor cells’ SSBs, and on those SSBs for L3 measurement, the scheduling restriction is applied. As defined in current RRM requirement below, different UE power class may have different sample number for FR2-1, but the beam sweeping factor is always considered for these delay requirements.

However, since the multi-Rx chain UE now has capability of simultaneous reception on different AoAs, it would be possible to perform Rx beam sweeping in parallel on different panels at UE and the corresponding L3 detection/measurement delay requirement can be reduced significantly. In addition, such enhancement can also avoid scheduling restriction on some of the L3 measurement occasions. As shown in the following figure 2 as an example, the rough Rx beam sweeping on right panel will not cause interruption to the data/control channel reception on the left panel.

Figure 2. Example for L3 measurement delay reduction by using multi-Rx

However, the only difficult part is that network is not aware of which L3 measurement occasions can be used for scheduling. We do observe the benefit to enhance the throughput for the UE to mute the scheduling restriction on some of the L3 measurement occasions, e.g., in the figure 2 case, 50% of throughput loss can be saved compared with legacy requirement. Since L3 measurement is periodic and quite static (RRC configuration based), RAN4 can further discuss how to indicate the scheduling restriction information to network, and network can use the some of the L3 measurement occasions for scheduling. Details of indication and signaling can be FFS, e.g., UE may indicate a periodic pattern to network to tell which L3 occasion can have no scheduling restriction, but we also agree that when the scheduling active TCI is changed, UE may need to indicate a new pattern then.

In last meeting, companies provided some initial ideas about the Rx BDF reduction, and our view is quite similar as option 2.

Proposal 6: For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline.

Feature capability of L3 measurement delay reduction by optimizing Rx BSF

- Option 1 (Nokia, vivo): - RAN4 to introduce a new individual capability for L3 beam sweeping factor reduction due to multi-Rx operation. - Recommended WF - Moderator note: to discuss if option 1 can be agreed.

Even though it’s too early to discuss the capability at this stage, we share the same view with companies that a new capability is needed for this L3 Rx BSF reduction feature. The main reason is: different from R18 FR2 multi-Rx WI and R18 FR2 SCell activation enhancement, this R19 feature shall be used for both L3 serving and neighbor cell measurement. We are also fine to delay this capability discussion to the end of the core part as legacy procedure.

Proposal 7: RAN4 is to introduce a new individual capability for L3 BSF reduction due to multi-Rx operation in R19. The detailed definition can be postponed to the end of the core part discussion.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: It is proposed to focus on PC3 UE as first priority.

Proposal 2: It is proposed to allow additional processing time for UE supporting multiple-Rx simultaneous reception for L3 delay enhancement if there is only one searcher available for the processing.

Proposal 3: For power saving purpose, there is a need to have a mechanism to activate/de-activate L3 fast beam sweeping. The R18 mechanism (i.e., multi-RX operation definition and UAI indication of preference) can be considered as a baseline, while other conditions are not precluded.

Proposal 4: Prioritize following scenarios as starting points to use L3 measurement delay reduction by optimizing Rx BSF:

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter

- SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter,  TSSB_time_index_inter and TSSB_measurement_period_inter

Proposal 5: After RAN4 has conclusion(s) on the solution(s) of L3 measurement delay reduction for the above baseline scenarios, the solutions(s) can be extended to other scenarios, e.g., handover, PSCell addition, RRC Re-establishment/RRC Connection Release with Redirection, SCell activation, SCG activation, CGI reading, and the associated SSB synchronization in CSI-RS L3 measurement.

Proposal 6: For UE supporting multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline.

Proposal 7: RAN4 is to introduce a new individual capability for L3 BSF reduction due to multi-Rx operation in R19. The detailed definition can be postponed to the end of the core part discussion.

# References

[1] R4-2410260, WF for [110bis][230] NR_RRM_Ph5, Moderator (Apple), RAN4#111