3GPP TSG-RAN WG4 Meeting #112                                                            R4-2411409

Maastricht, NL, 19th August- 23th August, 2024

Source:	Apple

Title:	AI/ML Testability and interoperability issues for beam management

Agenda Item:	8.17.2

Release:            Rel-19

Document for:	Discussion

Introduction

Rel-18 Study Item was approved on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface with the target to study the 3GPP framework for AI/ML for air-interface corresponding to each targeted use cases (i.e., CSI feedback enhancement, beam management, and positioning accuracy enhancements for different scenarios) regarding aspects such as performance, complexity, and potential specification impact [1] [2].

According to latest SID in [2], RAN4 is required to study the interoperability and testability aspects for each use case: Specifically, RAN4 is expected to study the requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable.

The current agreements on how to perform the RAN4 study on general issues for AI/ML, and issues related to interoperability/testing have been captured in the latest TR [3]

In this contribution, we provide our viewpoints on some of the interoperability and testability aspects for beam management.

Discussion

## 2.1 Framework for Defining Requirements in Tests for AI/ML

From RAN1 evaluation results there are many implementation factors that could influence the AI/ML model performance. Diverse company-specific assumptions and implementations regarding model structure and parameters often lead to significant performance variations. To ensure consistency across companies' results, incorporating a reference AI/ML decoder model becomes critical in defining performance benchmarks for the UE side encoder.

From a UE processing capability limitation perspective, RAN4 will need to find a measure to access and evaluate complexity. There should be some limitations and complexity considerations when RAN4 decides on the requirements.

As an example, from RAN1 performance evaluations for the single sided BM use case, the complexity of AI/ML models reported across the companies can vary drastically as it is shown in the figure below from TR:

Fig 1: Complexity of AI/ML models from evaluation results 
in terms of FLOPs and number of parameters for BM cases

Accordingly, the performance variations across companies as outlined in Section 6.3.2.1.1 from the TR can vary from 60% accuracy to 90% accuracy where most of the performance variation could be attributed to model complexity and other differences in simulation assumptions (training procedures, training data, etc)

One question that arises is: What should be the granularity for defining the parameters to specify the reference AI/ML model for BM use case? In addition to model parameters, other factors that could affect performance include the model output type and the associated types of training data. For example, significant performance variations across companies are reported for the BM use case in section 6.3.2.3 of the TR. These variations depend on the output type, such as labels (classifier network) or L1-RSRP (CNN or MLP). For training input types for BM we could also differentiate between wide beams (SSB based, wide Tx beams) vs narrow beams (CSI-RS based, narrow Tx beams) types of set B of beams.

Similarly to the list of parameters specified for the two-sided model in CSI use case, RAN4 should investigate the set of parameters and granularity for the other use cases. (BM and positioning)

For example, for the BM case we could start with the following parameters:

- General Complexity

  - Number of parameters

  - Memory Size

  - FLOPS

- Training output type (labels vs L1-RSRP, Classifier vs MLP)

- Training Input type (narrow beams (CSI-RS) vs wide beams (SSB) Tx beams)

We provide a table with the proposed parameters for specifying reference model for BM use case. (table 1)

Proposal 1: RAN4 should study the specification of reference AI/ML models, training procedure, and training data for defining performance requirements for BM use case while considering limitations on model complexity. We provide a reference table for initiating the discussion.

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Model architecture parameters | Model type | CNN(case 1), MLP (case 1_, RNN (Case 2), LSTM (Case 2) |
|  | Fixed point representation | Int8, int16, floating point etc |
| Model Training related parameters | Training procedure | FFS (e.g Initialization method, training duration, training completion criteria, etc) |
|  | Loss function | NMSE, Cross Entropy etc. |
|  | Training datasets | Channel model (SLS,Uma, etc) number of set -B/ set-A beams, specific TX and RX codebooks. Antena port layout Best Rx beam selection SSB Wide Tx beams vs CSI-RS Narrow beams |
|  | Impairments | RSRP measurements with impairments: Impairment types affecting set B measurements and set A measurements, quantization, etc |
|  | Hyperparameters | Learning rate, batch size, regularization techniques and strength, optimization algorithm, max epochs,etc. |
|  | Cross-validation details | Dataset splits for training/testing/validation |
| Complexity | Model Parameter Size |  |
|  | Model Memory Size |  |
|  | FLOPS |  |
| Scalability | Supported set A | 32, 64 beams |
|  | Supported set B pattern | 1/2,1/4, 1/8 etc |
|  | Time Window configuration (BM case 2) |  |
|  | Number of future predicted instances (BM case 2) |  |
| Performance Metric | Beam ID, RSRP accuracy | Top-K beams, values of K? |

Table 1: Parameters for Specifying Reference Model for BM use cases

## 2.2 Beam Prediction Testability Discussion

According to the discussion during RAN4#111 [ R4-2410571] the following agreements have been reached:

Agreement:

Maximum number of set B Tx beams that test system should be able to emulate: [8-16]

Maximum number of set A Tx beams that test system should be able to emulate:  [64-128]

FFS on AoAs

UE rotation during the test: FFS

UE rotation/repositioning between different tests: Yes

Issue 2-5: Test setup needs

To verify the accuracy of the L1-RSRP prediction for BM case 1 and case 2, we will need to introduce enough randomness in the spatial and temporal domain.  As it is shown in Fig 2, the propagation environment characterized by the different AoAs and AoDs, effectively sample the radiation beam patterns across the mainlobe and sidelobes of each of the beams in set B/set A at Tx and Rx antenna arrays. Fig 2 shows the radiation pattern of one beam inside set B or set A. The spatial mapping from set B to set A is learnt through all possible samplings of set B Tx beam patterns according to the AoDs along with samplings of the Rx beams across AoAs. AI/ML can learn the mapping from set B samples to set A samples by exploiting the correlations of the radiation patterns. For this learning to be complete we should sample the radation patterns at many points in spatial domain. This could be achieved by introducing randomness in AoA/AoD domain since the channel response is determined by the Tx beam response at the direction of the AoD.

In the presence of multiple AoAs and AoDs, the resulting L1-RSRP is the superposition of these spatial beam samples at both Rx and Tx. The spatial mapping of set B L1-RSRP to set A L1-RSRP is learnt by proving enough random data  to capture the spatial correlations across different angles of departures and arrivals as well as the different superpositions of those angles. The correlations inferred will depend on the specific mainlobe as well as the sidelobe shapes of the setA/B patterns which will have to configured in the test and emulated by the test equipment. Therefore, the AI/ML model should be trained and tested with enough randomness.

Observation 1: The testing environment for Beam Management (BM) case 1 and BM case 2 should replicate conditions that sufficiently capture the correlations of the transmit (Tx) and receive (Rx) beam patterns across the entire spectrum of propagation conditions in both spatial (angles of arrival and departure - AoAs and AoDs) and temporal domains.

Observation 2:  The following conditions will introduce randomness and variations in propagation conditions across both time and spatial domains for the computation of L1-RSRP

- Different AoDs with respect to the Tx antenna array

- Different AoAs with respect to the Rx antenna array

- Different superpositions of {AoA,AoD} pairs

- Fading/Variation in time domain ( different {AoA,AoD} pairs per resolvable delay bin path)

- UE movement (including rotation)

It is expected that during deployment in the field, all the aforementioned randomness and variations will be inherent in the real propagation environment. Therefore, the AI/ML model should be tested under similar conditions, otherwise, the UE could easily pass the test and fail during real deployment scenarios.

Observation 3:  During real-world deployment, UE will encounter random radio propagations characterized by variations in both spatial and temporal domains (fading). Testing UE under similar conditions is important to reflect the realities of deployment accurately.

Fig. 2: Beam pattern spatial domain sampling across AoA/AoD

The baseline measurement setup for RRM characteristics and SNR referenece point for RRM testing procedure based  on TR 38.810 [4] for the study on test methods specifically for FR2 UE beam management is shown in Fig 3.

Fig. 3:  UE RRM test methodology for FR2 Beam Management

For the testing goals of BM AI/ML use case as described in the paragraph above, there are two main limitations with the existing test methods:

- Limitation on the number of simultaneously active AoAs (=2)

- Limitation of the relative angular separation between AoA pairs: 30°, 60°, 90°, 120° and 150°.

There are questions regarding how to meet the testing goals and RAN4 requirements using the FR2 OTA chamber, particularly with the necessity of sweeping beam patterns across Set-A and Set-B, but with the limitations of only two Angle of Arrivals (AoAs) and coarse angular separation. Moreover, for BM-Case2, to evaluate temporal DL Tx beam prediction, there should be some variation in beam direction over time, including UE rotation.

Observation 4: The following questions need to be answered to evaluate the feasibility of the FR2 OTA-based test procedure:

- How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs?

- What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam?

- How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)?

- How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction?

Based on the above discussion and observatios we have the following proposal:

Proposal 2: RAN4 should investigate the feasibility of the current FR2 OTA-based test procedure to capture random fading in both spatial and temporal domains, similar to CDL, Uma, etc. Additionally, consider the incorporation of UE rotation to help model randomness from the perspective of the Angle of Arrival (AoA)

Proposal 3:  If the current FR2 OTA-based test procedure proves to be not feasible, consider using CDL and Uma channels for testing in the BM AI/ML use case upon TE approval for feasibility

During RAN4#110-bis a spatial model was proposed to be implemented by the channel emulator as shown in Fig 4.

Fig. 4:  Channel Emulator Based Spatial channel for testing BM

The following parameters that can be programmed:

Tx codebook and antenna pattern. The Tx gain is known given the codebook and antenna pattern (across 2 polarizations) The AoD angles from the Tx side. We assume that the AoA angles are fixed and they are part of the probes in the MIMO OTA testing setup. The coupling factors that couple the AoDs clusters to the AoAs clusters Gc  AoDs represent the first stage of clusters and AoAs represent the last stage of clusters. Any intermediate clusters and interactions can be abstracted by the coupling factors Gc,ij(AoAi,AoDj). The different AoDs that couple to AoA_i are denoted as AoDij.  A Tx beam (from set A or set B) is denoted as wk. A UE RX beam is denoted as um. The RX codebook is denoted as U.

By taking into consideration the testing set-up shown in Fig 4, the RSRP for a particular Tx beam is derived as shown below:

Fig. 5:  Proposed BM-Case 1 spatial prediction testing setup ( multiple static channels)

For testing the RSRP accuracy for BM-Case1 spatial prediction we can configure multiple static configuration tests, where each static scenario corresponds to a set of parameters for generating the channel realizations. For each test we configure UE to perform measurements on the set A and we treat those measurements as the genie measurements. The whole procedure is shown in Fig 5.

Proposal 4: For BM-Case 1 spatial prediction and for verifying RSRP accuracy we propose to use the testing setup and channel emulator functionality as described in Fig 5. (multiple static configurations)

For testing the RSRP accuracy for BM-Case2 temporal prediction we can configure a non-static configuration test, where the set of parameters for generating the channel realizations vary over time.Therefore, the channel realization vary due to inherent propagation conditions that should alter the optimum Tx beam over time and introduce some randomnsee and correlation over time to be captured by the AI/ML. Similar to BM-Case 1 testing procedure, we configure UE to perform measurements on the set A and we treat those measurments as the genie measurements. The whole procedure is shown in Fig 6.

Fig. 6:  Proposed BM-Case 2 temporal prediction testing setup ( non- static channels)

Proposal 5:  For testing the beam prediction/RSRP accuracy for BM-Case 2 temporal prediction we propose the framework described in this section and in visualized I Fig 6, though a non static configuration of the channel emulator.

## 2.3  KPIs/Test Metrics for BM Use case

In TR 38.843 the following agreements are captured  for KPIs/Test metrices for BM use case.

Both spatial-domain DL beam prediction and temporal DL beam prediction are considered.

For metrics for beam management requirements/tests, the following test metrics are identified and could be considered

-	Option 1: RSRP accuracy

-	Option 2: Beam prediction accuracy

-	Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam"

-	Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams"

-	Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams"

-	Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

-	Related measurement accuracy can be considered to determine x

-	Option 4: combinations of above options

The overhead/latency reduction should be considered for the requirements as the side condition.

For option 1, RSRP prediction accuracy focus is about the RSRP difference for the same beam. RSRP prediction accuracy is more similar as legacy RSRP measurement accuracy definition:

Absolute L1-RSRP accuracy = measured L1-RSRP of beam index n - ideal absolute L1-RSRP of beam index n

RSRP prediction accuracy metric is defined in clause 6.3.1 in TR 38.843:

For AI/ML models, which provide L1-RSRP as the model output, the accuracy of predicted L1-RSRP is to be evaluated. Companies optionally report average (absolute value)/CDF of the predicted L1-RSRP difference, where the predicted L1-RSRP difference is defined as the difference between the predicted L1-RSRP of Top-1 predicted beam and the ideal L1-RSRP of the same beam.

From RAN4#110-bis the following agreement has been reached:

Companies are invited to provide inputs/proposals to refine the definition of RSRP accuracy. Hold on the discussions for concrete test metrics until RAN1 had conclusions on the schemes.

From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity. Following this perspective and in accordance with legacy schemes, a minimum RSRP accuracy test should be applied to all reported/predicted beams. Additionally, in legacy systems, there is no testing to confirm if the reported beam powers correspond to the best genie beam indices. Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams.

Observation 5: From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity

Observation 6: Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams

In line with the legacy approach, we aim to ensure RSRP accuracy for all predicted beams. When using neural networks for regression, we can rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Observation 7: When using neural networks for regression, we rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

Proposal 6: We propose that RSRP accuracy be defined as the difference between the predicted RSRP and the measured (genie) RSRP associated with the same Tx beam. Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR.

When considering the necessity of testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially in light of our definition of RSRP accuracy.

Proposal 7: When considering the necessity of additionally testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially in light of our definition of RSRP accuracy.

## 2.4  Datasets for training/testing

For training the AI/ML models for beam management we need to agree on what the source of the training data is.  In RAN4#111 the following agreement was reached:

Issue 2-6:	Datasets for training/testing

- Proposals

  - Option 1: Training Data set to be specified in RAN4(directly or through some algorithm )

  - Option 2: Training data set to be left to implementation (companies can generate it based on knowledge of the test environment)

  - Option 3: others

- Recommended WF

  - To be discussed

Agreement:

- Vendors may take into account the test environment/conditions defined by RAN4 when training the UE

  - sufficient test environment/conditions should be defined to enable vendors to create the data needed for training

- FFS on proposals to augment training data to avoid overfitting of UE models to the test environment

- RAN4 to strive to make the test conditions similar to field deployment conditions

We need to agree if the goal of the training data is to train the model for testing purposes only or for training the model for real deployment as well.

If the training data will be used to train one model (used in testing and deployment) it would be hard to gurantee similar performance (testing environment and in the field) since Tx codebooks could be different from vendor to vendor.

Proposal 8: Regarding the source of training data, RAN4 should agree that the training dataset is for testing purposes only. In real-world scenarios, a different training dataset could be obtained to train a vendor-specific model, unless vendors are willing to share their codebooks. In such a case, we could define a mixed training dataset from multiple vendors. Further feasibility studies (FFS) are needed to determine if this approach is viable.

Since the training data are uniquely affected by the specific UE HW they should be left for implementation from each UE vendor, but the channel conditions/configuration could be defined by RAN4 and used in the testing environment.

Proposal 9: For the source of the training data for BM, RAN4 should define the channel conditions and configurations to be used in the testing environment. Additionally, UE-generated training data should take into account each unique hardware implementation (Option 3, combination of Option 1 and 2).

Our proposal for generating the training dataset for beam management is shown in Fig 7.

Fig. 7:  Option 3 for generating the training dataset for beam management

## 2.5 Measurement Acuracy for BM

In RAN4#111 the issue of measurement accuracy was discussed as follows:

Measurement accuracy

Several companies brought up the need to improve measurement accuracy (e.g. RSRP measurement accuracy) to enable better AI/ML performance

Issue 2-8:	Measurement error impact

- Proposals

  - Option 1: RAN4 should study the impact of measurement error, companies should bring proposals for the next meeting on how to proceed with such a study

  - Option 2: RAN4 should postpone the discussion on the impact of the measurement error for now

  - Option 3: No need to evaluate the measurement error impact

- Recommended WF

  - To be discussed

In case the training data are based on real measurements there is a tradeoff between data quality and model generalization: In both BM-Case 1 and BM-Case 2 sub-use cases, L1-RSRP measurements serve as inputs and outputs for the AI-ML model. However, these measurements are susceptible to errors due to RF impairments,  thermal noise, quantization noise and other non-ideal components at the UE receiver. The range of measurement errors for FR2 is defined by current L1-RSRP requirements outlined in Clauses 10.1.20 of TS 38.133.

For both the Base Station (BS) and UE side models, these measurement errors impact the input L1-RSRPs for SetB beams. Simultaneously, during the training phase, measurement errors (and quantization errors) also influence the output because the labels/RSRPs are determined based on the non-ideal L1-RSRP of SetA beams. This interplay of errors affects both input and output data in the ML model training and inference processes. We would want to avoid the scenario where the AI/ML model attempts to fit the noise or impairment errors.

- Observation 8: The source of training data for beam management will play a crucial role in AI/ML BM performance and in the generalization performance in real deployment

Moreover, there is variability among the magnitude of errors. Such a concern arises when we consider the trade-off between data quality and model generalization. For instance, consider two UEs: UE 1 positioned at the center of the serving cell and UE 2 at the cell's edge (low SNR). We opt to use ground truth to retain "good data" (from UE 1 due to favorable Signal-to-Noise Ratio or SNR) while discarding "bad data" (from UE 2 due to poor SNR), undoubtedly enhancing the model's training performance but trading off generalization performance for users with low SNR conditions. Deploying this model in real-world scenarios where radio conditions (measurments at low SNR) and obstacles, such as Non-Line-of-Sight (NLOS) elements, significantly differ from the controlled test environment, can result in degraded model performance. This discrepancy jeopardizes the model's ability to generalize well, undermining the efficacy of RAN4 testing goals.

Observation 9: For training data based on real measurements, the quality of  training data depends on RF impairments, and other noise sources. There is tradeoff between training data quality and generalization performance. With training data collection from the field (across varying SNR conditions), both set-B and set-A beams will be affected ny impairments

The performance of the AI/ML model will depend upon the following factors:

- Measurement accuracy of RSRP

- Assumption on the side conditions, example SNR levels

- Training data assumptions (e.g. what was the impairment model assumed during training)

In order to investigate the performance degradation that results from measurement accuracy we propose to specify an impairment model with its associated parameters so that companies are providing comparable simulation results. We can consider the parameters in the impairment model specified in Fig 8 as a baseline.

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Training Data for BM | Training datasets | System level channel model (SLS,Uma, etc) number of set -B/ set-A beams, specific TX and RX codebooks. Antena port layout Best Rx beam selection SSB Wide Tx beams vs CSI-RS Narrow beams Tx Codebook Tx antenn array dimensions Antenna Element Spacings |
|  | Impairment Model to be added to set B and set A measurmenets | Impairment types affecting set B measurements and set A measurements, quantization, SNR levels, RF impairmenets, other side conditions etc |

Fig. 8:  Proposed parameters to generate training data with impairment model for BM

Proposal 10: To investigate performance degradation resulting from measurement accuracy, we propose specifying an impairment model along with its associated parameters. This ensures that companies provide comparable simulation results, which will guide the decision on changing RSRP accuracy requirements.

If the outcome of simulating the performance from the impairment model results in degraded performance, then we can consider the following options:

- Make the AI/ML adaptable to changing SNR conditions, for example to maintain performance the number of set B beams can increase, also the number of K in top-K could increase to compensate for degraded performance.

- Change the site conditions, (like SNR, etc) under which the AI/ML operation can be supported. Below a threshold SNR point, legacy procedures should be employed.

- If the site conditions significantly change to support AI/ML functionality, then we could consider tighter accuracy requirements

Proposal 11: If the outcome of simulating the performance with the impairment model results in degraded performance, then we can consider the following options:

- Adjusting RSRP Requirements and Enhance Measurement Accuracy: Reevaluate and potentially modify the current RSRP accuracy requirements to better align with the observed performance under the impairment model.

- Adopting Compensation Mechanisms: Make the AI/ML adaptable to changing SNR conditions, for example to maintain performance the number of set B beams can increase, also the number of K in top-K could increase to compensate for degraded performance. (change model, address scalability of AI/ML model)

- Change the side conditions, (like SNR, etc) under which the AI/ML operation can be supported. Below a threshold SNR point, legacy procedures should be employed.

## 2.6 Generalization issues for BM

For AI/ML based BM solution, generalization poses one of the main challenges for RAN4 testing. Generalization issue includes the following main aspects:

Changing radio conditions

If an AI/ML functionality/model has been trained primarily on a dataset representing specific radio conditions, it may encounter diminished performance when faced with varying channel conditions in real-world environments. For example different propagation environments characterized by different angular/delay spread (LOS/NLOS) can alter the spatial mapping from set B to set A of beams learnt by AI/ML. Similarly the spatial mapping between set B and set A can be impacted by the carrier frequency/band of the measurements.

Changing configurations/parameters settings (conditions and additional conditions)

- The performance of different AI/ML applications is greatly influenced by the level of generalization achieved  which depends on the configuration and parameter settings utilized for training dataset creation. For instance, in the context of beam management in AI/ML, configurations (conditions and additional conditions) need to consider various beam sets and codebooks, along with variations in the number of wide and narrow beams (Fig 9), different beam widths, Tx angles, a diverse grid of set B beam configurations,  and measurements across multiple frequencies or bands.

Fig. 9:  Various beam sets and codebooks across different cells

Observation 10: To guarantee that the UE operates within acceptable margins, it's essential to subject it to various radio conditions and additional conditions for testing and generalization validation in RAN4

For the generalization verification aspects, the following contents are agreed to be captured in TR 38.843 [1]:

The goals of generalization test are to verify whether the minimum level of performance of AI/ML functionality/model can be achieved/maintain under the identified scenarios and/or configurations, while the performance won’t be significantly degraded in other scenarios and/or configurations. The following aspects should be considered for generalization/scalability related testing: - details about the scenarios and/or configurations for test and the corresponding AI/ML models/functionality - what the minimum level performance for each identified scenario and/or configuration is - what the significant degradation for other scenarios and/or configurations is It should also be considered that generalization and/or scalability related requirements for different scenarios/ configurations can be implicitly handled in the test case definition. As for the handling of generalization tests, the following option is considered as baseline: Signalling based LCM procedures and performance monitoring are considered in dedicated test cases and are excluded in tests verifying generalization. RAN4 may define multiple tests with different conditions. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test) - Specified UE configuration includes functionality and/or model ID if defined

The discussion concerning "details about the scenarios and/or configurations for testing and the corresponding AI/ML models/functionality" closely aligns with the RAN1 agreement regarding AI/ML functionality and models. These are proposed for the identification of AI/ML functionality/model, as outlined in the following excerpt from the agreed RAN1 section of TR 38.843:

<RAN1 Agreement, captured from TR38.843 v1.3.0> For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models, functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. ... For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models, model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side.

A model associated with configuration/condition associated with UE capability and additional conditions is shown in Fig 10.

Fig. 10:  A model associated with configurations/conditions associated with UE capability and additional condition

Our view on the "details about the scenarios and/or configurations for testing and the corresponding AI/ML models/functionality" is shown in Fig 11.  For the AI/ML functionality the UE capability report involves grouping of AI/ML models into buckets corresponding to non-overlapping sets of supporting configurations/conditions (conditions associated with Model A, Model B, etc). These conditions are associated with the UE capability for functionality. For each of these generic model there are sub-models that can be associated with additional conditions for model identification. There are two types of additional conditions. The first type is the additional conditions that can be specified (and they can be signaled as assistance information to the UE to select the proper model). These NW side additional conditions should be aligned between training and inference for the UE sided model for performance improvement. According to this information, UE can select the most suitable model to improve performance and pass the test.

Fig. 11:  Capability report and addional conditions

RAN4 requirements should define tests for the AI/ML model such that the testing data align with the training data. That is, some of the conditions and additional conditions for functionalities and model identification, such as the beam pattern in Set B, the number of beams in setA/B and their associated mapping (wide SSB, versus CSI-RS narrow beams), time window configurations for BM case2, etc should align between training and RAN4 test. In order for the UE to select the appropriate model, some indication should also be part of the test definition. This assistance information will be part of the additional conditons that could be shared from the NW (NW side additional condition). Other more proprietary information (3dB beam width, antenna spacing, Tx beam angles) will be part of the additional conditions that cannot be shared (not part of UE capability) and could be considered to be  part of generalization tests

As mentioned, UE sided model would require some information on the configuration of set B codebook pattern for inference. This could be indicated as additional assistance information. Based on RAN1 discussion, there is no consensus to support the assistance information from NW on NW-side beam shape information. Simulation results confirm that the consistency / association of Set B beams and Set A beams across training and inference is “beneficial” from performance perspective from :

For BM-Case1 and BM-Case2 with a UE-side AI/ML model, consistency / association of Set B beams and Set A beams across training and inference is beneficial from performance perspective. Note: Whether specification impact is needed is a separate discussion.

Thus, concerning the testing implications, ensuring the consistency or association of Set B beams and Set A beams throughout training and inference should be guranteed in the RAN4 core requirements. This consistency serves as additional assistance information for inference at the UE from the network side.

Proposal 12:  In the RAN4 core requirement, it is mandated that the consistency or association between Set B beams and Set A beams during both training and inference must be guaranteed. This serves as additional assistance information from the network side for testing UE-sided models during inference and monitoring

From the RAN1 agreement captured above, it can be seen that a UE will report the supported configurations as part of capability signaling for a certain model/functionality. The other scenarios and/or configurations for generalization can be decided based on the supported configuration reported by UE.

For defining tests we can identify a set of reference conditions/configurations based on the capability signaling report from the UE. Example of reference radio conditions that UE could support can be an AWGN propagation condition or a particular CDL channel (link level). For example UE capability could indicate that the output type of the AI/ML model is label based (classifier), therefore the model should be tested under option 2 for BM KPI test metric.

For the other scenarions/configurations for defining generalization tests, we capture the following paragraph from the TR: “. In each of the test, TE configures the same specified UE configuration, and therefore the same specified UE configuration is tested under different conditions to verify its generalizability. (environment differs in each test but not changing dynamically during the test)

Specified UE configuration includes functionality and/or model ID if defined ”

For functionality based LCM, functionality is enabled by configurations supported by conditions indicated by UE capability. Given that configuration, and as part of generalization test we can vary some condition/additional conditions.  For example, tests can be defined where the patterns of beam set A and set B vary,  or their associated mapping changes (e.g. SSB to CSI-RS wide to narrow beam mapping), or different antenna configirations/layout like 4x16 antenna versous 8x8 antenna layout, different antenna spacings (e.g spacing = 0.8 lamda).   Another example of additional condition could be the  frequency/band of the sweeping beams, number of history measurements time measurements for BM case 2 (Set B pattern in temporal domain), system level propagation channels (Uma, Umi), differene UE speeds, different beam shapes, etc Some of these conditions can be signalled explicitly or implicitly to the UE during the test and the UE can select the appropriate model. This is shown in Fig 12 where UE2 switches models to improve generalization performance based on assistance signaling of additional conditions.

Fig. 12:  Generalization performance improvement with model switch based on assistance information or autonomous model switch

For generalization testing, there is a type of addtional conditions which are the sensitive proprietary information that NW vendors may not be willing to share. Examples of these conditions are:

- NW side beam shape information

  - 3dB bandwidth, physical antenna spacing, antenna array size, Tx beam angles, boressight directions

For defining generalization tests, the identified scenarios and configurations shall be associated with the UE capability of an AI/ML-enabled Feature FG. For example, we could define a generalization test associated with model B as it is shown in Fig 13. Model B is associated with some specific conditions/configurations, for functionality identification as indicated by UE capability. Under that configuration, UE could have other models associated with additional conditions (these conditions cannot be specified). These additional conditions will be the different scenations that will be part of the generalization tests.

Some of the additional conditions can be shared during the test in the form of assistance information to improve the generalization ability of the UE.

For the other additional conditions that the NW cannot share due to proprietary information, we can employ a virtual ID in the training dataset that indicates the additional conditions that the model was trained with. By signaling this ID, the UE can select the model that was trained with this dataset.

Observation 11: For BM use case the identified scenarios and configurations can be initially understood as those reported by UE through capability signaling as part of functionality identification.

Observation 12: The additional conditions for the AI/ML model training (which do not constitute part of UE capability) for the AI/ML-enabled feature/FG can serve as the different scenarios/configuration for defining generalization

Proposal 13: RAN4 should define identified scenarios/configurations associated with the UE capability report of an AI/ML-enabled Feature FG.  For defining generalization tests, the additional conditions can serve as the other identified scenarios/configurations for the BM use case

Proposal 14: RAN4 should investigate the feasibility of providing assistance information for the additional conditions to aid generalization and consistency across training and testing when defining requirements. Other additional conditions that are not part of UE capability can be used to define generalization tests

Proposal 15: For additional conditions that cannot be shared due to proprietary concerns, RAN4 can explore the feasibility of using a virtual ID to indicate the specific conditions under which a model was trained. This approach would assist in the proper selection of UE models to support generalization. Additionally, RAN4 should identify which additional conditions should be exclusively reserved for generalization tests.

## 2.7 Consistency between Training and Inference

In RAN4#110 bis  multiple companies brought up the need to discuss the consistency between set A and set B, otherwise it is not expected that beam prediction would work. In RAN4#111 the following options were provided:

Issue 2-7:	Consistency

- Proposals

  - Option 1: consistency between set A and set B is reflected by below aspects:

    - Same NW antenna/beam configurations for set A and set B, and that its configurations don’t change during training and inference.

    - Same channel model for set A and set B, and that its configurations don’t change during training and inference. (Question: if consistency is valid for a non-static(time-varying) channel for set A and set B?)

  - Option 2: Consistency should be defined in a different way

  - Option 3: no need for any consistency definition

- Recommended WF

  - To be discussed

Proposal 16: RAN4 to consider option 1 as a baseline for achieving consistency between training and inference for both set A and set B for beam spatial and temporal prediction

As it is shown in the figure below, in a non-static condition the set B configuration could change (let’s say B’). In this case the consistency is violated since set B’ is a different configuration from set B that the model was trained with. Consistency should be considered only for static conditions and not-static conditions should test the UE ability to adapt and employ diverse AI/ML models to match the conditions.

Table 1. Examples of Static versus Non-static Conditions

Proposal 17: Consistency should be considered in RAN4 only for static conditions and not-static conditions should test the UE ability to adapt and employ diverse AI/ML models to match the conditions (generalization)

The existence of generalization cases highlights the necessity of tackling performance issues that arise when the assumptions made during training, such as using a machine learning model trained with a specific set of network/user equipment (NW/UE)-side additional conditions, do not align with the assumptions made during inference. This misalignment can lead to certain performance degradations.

As agreed in RAN#102, ensuring consistency between training and inference regarding any identified additional conditions on the network side for inference at user equipment (UE) is a key objective for the beam prediction use case within the scope of Release 19 work item.

As highlighted in TR 38.843 (Clause 4.2.3), additional conditions refer to any aspects that are assumed for the training of the model but are not a part of UE capability for the AI/ML-enabled feature/FG. Moreover, additional conditions of a model can be divided into two categories: NW-side additional conditions and UE-side additional conditions

The following options can be taken as potential approaches to improve generalization and maintain performance by ensuring consistency between training and inference, in accordance with clause 4.2.3 of TR 38.843:

·  Model identification to achieve alignment on the NW-side additional condition between NW-side and UE-side.

·  Model training at NW and transfer to UE, where the model has been trained under the additional condition.

·  Information and/or indication on NW-side additional conditions is provided to UE.

·  Consistency assisted by monitoring (by UE and/or NW, the performance of UE-side candidate models/functionalities to select a model/functionality)

The concept of additional conditions was introduced during the latter stages of the study item when the necessity for these conditions became apparent due to the presence of AI/ML training variables that may lack standardization. For example, in AI/ML-based beam management use cases, elements of network topology such as cell layout or antenna configuration, or internal variables of user equipment (UE) such as UE speed, can influence the selection of the AI/ML model to be utilized during AI/ML operations.

Nonetheless, while these variables may be essential for training AI/ML models and thus required as input for AI/ML model operation, they may not be standardized due to proprietary information concerns. Therefore, categorizing these variables as additional conditions can facilitate the AI/ML operation.

Among the methods explored for ensuring consistency between the network and UE for these additional conditions, the "Consistency assisted by monitoring" methodology appears to be the least optimal solution. It is expected to cause significant delays in selecting the optimal AI/ML model for the UE.

Consider a scenario where multiple sets of beam management models are developed for different user equipment (UE) speeds, and the network intends to utilize the monitoring procedure to select the optimal model. In this case, the network must activate multiple models at the UE one by one, each associated with different UE speeds, until it identifies an AI/ML model that yields the best performance. During the period when the network is activating non-optimal model(s) at the UE, UE performance suffers, resulting in a delay in selecting the most optimal model.

Observation 13:  Achieving consistency between training and inference by model monitoring could result in delays and increased complexity in model management for BM use case

If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts.

Observation 14:  If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts for BM use case

In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings (additional conditions), thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance.  UE complexity increases if the UE needs to store and manage multiple AI/ML models.

Proposal 18:  In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings, thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance for BM use case

Proposal 19: For achieving consistency between training and inference, if the model drifts due to misalignment of network-side additional conditions, the alignment could be achieved through an ID assigned by the network during training data collection. This ID indicates the association of the training data with the additional conditions implied to generate those data

If the UE doesn’t have the ID but the NW has the model ID, then the NW can transfer the model to UE and UE update its list of models:

Proposal 20: For achieving consistency between training and inference, if the UE lacks the ID but the network possesses the model ID, then the network can transfer the model to the UE, and the UE can update its list of models

If the UE has a model that partially supports the NW-sided additional conditions then the NW can trigger data collection for fine tuning of the UE-sided model:

Proposal 21: For achieving consistency between training and inference, if the UE has a model that partially supports the network-sided additional conditions, then the network can trigger data collection for fine-tuning the UE-sided model. Subsequently UE updates its list of model IDs.

If there is no model at the network or UE, then the network configures the UE to fallback to legacy mode.

In case there is no ID available, then NW can configure a model monitoring procedure with UE-side measurements to check if the consistency of the additional conditions can be achieved. If performance criteria are met the consistency is achieved and the model has been verified. Otherwise, fallback to legacy mode is configured.

Proposal 22: The UE updates its model list with a new model derived from either fine-tuning, model transfer from the NW, or monitoring to ensure the consistency of additional conditions. Then, the UE assigns an ID to the new model that supports the NW's additional conditions and shares this information with the NW. If some of the new conditions/configurations are standardized, the UE updates its capability report accordingly.

Proposal 23: If there is no ID available to associate training data with additional conditions, and monitoring procedure fails to guarantee the consistency of the model with the additional conditions then UE should fallback to legacy mode.

For model identification each model is characterized by conditions and additional conditions. An example is shown in Fig 12. The complexity of LCM model management increases since the number of candidate models is large. Deactivations, latency and delays can occur when model monitoring takes place or when an appropriate model is not found (in this case UE fallback to legacy)

Fig. 13:  Model Identification and NW-side additional conditions

The current schemes proposed to ensure consistency between training and inference for sustaining system performance is through assistance information or model identification which is only used for model selection.

Observation 15: Current proposals on assistance information for additional conditions and Model Identification only serve the purpose of selecting the appropriate AI/ML model. However, this approach may not be scalable due to considerations of UE implementation complexities and granualtity of conditions/additional conditions. Complexity can increase substantially, especially if condition granularity is fine.

We propose defining some form of assistance information signal as input to the AI/ML model, which will be incorporated into the model’s inference engine. We propose the employment of an AI/ML supermodel that supplements its core inputs with auxiliary information signals, which will be integrated into its inference engine. The form of auxiliary information signals should optimize the tradeoff between disclosing proprietary information and enhancing the generalizability of the AI/ML model. As a result, the number of required AI/ML models needed to be stored, managed, and tested will be drastically reduced. An example is shown in Fig 13.

Fig. 14:  AI/ML Super Model with auxiliary information signals

Proposal 24:  Investigate the feasibility of enhancing the generalizability of the AI/ML model and reducing the number of AI/ML models and the testing burden for the beam management case by supplementing the core AI/ML input signals with both network (NW) and  UE auxiliary information signals integral to its inference engine

Proposal 25:  Investigate the feasibility of training the models with a mixed dataset associated with both network (NW) and UE auxiliary information signals to further enhance the generalizability of the AI/ML model for the beam management case and reduce the number of generalization tests.

# Conclusion

In this section we summarize our discussion by listing the observations and proposals discussed in this contribution:

Observation 1: The testing environment for Beam Management (BM) case 1 and BM case 2 should replicate conditions that sufficiently capture the correlations of the transmit (Tx) and receive (Rx) beam patterns across the entire spectrum of propagation conditions in both spatial (angles of arrival and departure - AoAs and AoDs) and temporal domains.

Observation 2:  The following conditions will introduce randomness and variations in propagation conditions across both time and spatial domains for the computation of L1-RSRP

- Different AoDs with respect to the Tx antenna array

- Different AoAs with respect to the Rx antenna array

- Different superpositions of {AoA,AoD} pairs

- Fading/Variation in time domain ( different {AoA,AoD} pairs per resolvable delay bin path)

- UE movement (including rotation)

Observation 3:  During real-world deployment, UE will encounter random radio propagations characterized by variations in both spatial and temporal domains (fading). Testing UE under similar conditions is important to reflect the realities of deployment accurately.

Observation 4: The following questions need to be answered to evaluate the feasibility of the FR2 OTA-based test procedure:

- How can we generate multiple beams from the Set-B and Set-A Tx beams given the limitation of two AoAs?

- What assumption is made regarding Rx beam sweeping? Does the UE utilize a fixed Rx beam, or does it sweep to find the optimal Rx beam?

- How can we simultaneously emulate different AoAs (Rx beam) and AoD (Tx beams)?

- How can we achieve dynamic variation in the AoD domain (Tx beam sweeping) for BM case 2 prediction?

Observation 5: From the network operator's perspective, a number of optimal beams need to be known in order to optimize load balancing and make trade-offs in performance and complexity

Observation 6: Ensuring that RSRP accuracy implies beam prediction accuracy is challenging and depends on absolute RSRP accuracy and the RSRP difference between the best beams

Observation 7: When using neural networks for regression, we rely on the Normalized Mean Squared Error (NMSE) criterion for training. It's crucial to ensure that the accuracy of weaker beams is not compromised by the accuracy of the strongest beams

- Observation 8: The source of training data for beam management will play a crucial role in AI/ML BM performance and in the generalization performance in real deployment

Observation 9: For training data based on real measurements, the quality of  training data depends on RF impairments, and other noise sources. There is tradeoff between training data quality and generalization performance. With training data collection from the field (across varying SNR conditions), both set-B and set-A beams will be affected ny impairments

Observation 10: To guarantee that the UE operates within acceptable margins, it's essential to subject it to various radio conditions and additional conditions for testing and generalization validation in RAN4

Observation 11: For BM use case the identified scenarios and configurations can be initially understood as those reported by UE through capability signaling as part of functionality identification.

Observation 12: The additional conditions for the AI/ML model training (which do not constitute part of UE capability) for the AI/ML-enabled feature/FG can serve as the different scenarios/configuration for defining generalization

Observation 13:  Achieving consistency between training and inference by model monitoring could result in delays and increased complexity in model management for BM use case

Observation 14:  If multiple models with varying generalization capabilities and requirements for network-side additional conditions are trained by different UE vendors, it would necessitate substantial standardization efforts for BM use case

Observation 15: Current proposals on assistance information for additional conditions and Model Identification only serve the purpose of selecting the appropriate AI/ML model. However, this approach may not be scalable due to considerations of UE implementation complexities and granualtity of conditions/additional conditions. Complexity can increase substantially, especially if condition granularity is fine.

Framework for Defining Requirements in Tests for AI/ML

Proposal 1: RAN4 should study the specification of reference AI/ML models, training procedure, and training data for defining performance requirements for BM use case while considering limitations on model complexity. We provide a reference table for initiating the discussion.

Beam Prediction Testability Discussion

Proposal 2: RAN4 should investigate the feasibility of the current FR2 OTA-based test procedure to capture random fading in both spatial and temporal domains, similar to CDL, Uma, etc. Additionally, considering the incorporation of UE rotation can help model randomness from the perspective of the Angle of Arrival (AoA)

Proposal 3:  If the current FR2 OTA-based test procedure proves to be not feasible, consider using CDL and Uma channels for testing in the BM AI/ML use case upon TE approval for feasibility

Proposal 4:  For testing the KPIs for BM-Case 1 spatial prediction we propose the test setup and framework described below upon TE vendor approval for the feasibility of this approach.

Proposal 5:  For testing the KPI for BM-Case 2 temporal prediction we propose the test set up and framework described below  upon TE vendor approval for the feasibility of this approach.

KPIs/Test Metrics for BM Use case

Proposal 6: We propose that RSRP accuracy be defined as the difference between the predicted RSRP and the measured (genie) RSRP associated with the same Tx beam. Accuracy requirements should apply to all predicted beams that satisfy the predefined side conditions, including SNR.

Proposal 7: When considering the necessity of additionally testing for beam prediction accuracy, we propose to study the additional information and significance that this test will provide, especially in light of our definition of RSRP accuracy.

Datasets for Training/Testing

Proposal 8: Regarding the source of training data, RAN4 should agree that the training dataset is for testing purposes only. In real-world scenarios, a different training dataset could be obtained to train a vendor-specific model, unless vendors are willing to share their codebooks. In such a case, we could define a mixed training dataset from multiple vendors. Further feasibility studies (FFS) are needed to determine if this approach is viable.

Proposal 9: For the source of the training data for BM, RAN4 should define the channel conditions and configurations to be used in the testing environment. Additionally, UE-generated training data should take into account each unique hardware implementation (Option 3, combination of Option 1 and 2).

Fig. 7:  Option 3 for generating the training dataset for beam management

Measurement Acuracy for BM

Proposal 10: To investigate performance degradation resulting from measurement accuracy, we propose specifying an impairment model along with its associated parameters. This ensures that companies provide comparable simulation results, which will guide the decision on changing RSRP accuracy requirements.

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Training Data for BM | Training datasets | System level channel model (CDL,Uma, etc) number of set -B/ set-A beams, specific TX and RX codebooks. Antena port layout Best Rx beam selection SSB Wide Tx beams vs CSI-RS Narrow beams Tx Codebook Tx antenn array dimensions Antenna Element Spacings |
|  | Impairment Model to be added to set B and set A measurmenets | Impairment types affecting set B measurements and set A measurements, quantization, SNR levels, RF impairmenets, other side conditions etc |

Fig. 8:  Proposed parameters to generate training data with impairment model for BM

Proposal 11: If the outcome of simulating the performance with the impairment model results in degraded performance, then we can consider the following options:

- Adjusting RSRP Requirements and Enhance Measurement Accuracy: Reevaluate and potentially modify the current RSRP accuracy requirements to better align with the observed performance under the impairment model.

- Adopting Compensation Mechanisms: Make the AI/ML adaptable to changing SNR conditions, for example to maintain performance the number of set B beams can increase, also the number of K in top-K could increase to compensate for degraded performance. (change model, address scalability of AI/ML model)

- Change the side conditions, (like SNR, etc) under which the AI/ML operation can be supported. Below a threshold SNR point, legacy procedures should be employed.

Generalization issues for BM

Proposal 12:  In the RAN4 core requirement, it is mandated that the consistency or association between Set B beams and Set A beams during both training and inference must be guaranteed. This serves as additional assistance information from the network side for testing UE-sided models during inference and monitoring

Proposal 13: RAN4 should define identified scenarios/configurations associated with the UE capability report of an AI/ML-enabled Feature FG.  For defining generalization tests, the additional conditions can serve as the other identified scenarios/configurations for the BM use case

Proposal 14: RAN4 should investigate the feasibility of providing assistance information for the additional conditions to aid generalization and consistency across training and testing when defining requirements. Other additional conditions that are not part of UE capability can be used to define generalization tests

Proposal 15: For additional conditions that cannot be shared due to proprietary concerns, RAN4 can explore the feasibility of using a virtual ID to indicate the specific conditions under which a model was trained. This approach would assist in the proper selection of UE models to support generalization. Additionally, RAN4 should identify which additional conditions should be exclusively reserved for generalization tests.

Consistency between Training and Inference

Proposal 16: RAN4 to consider option 1 as a baseline for achieving consistency between training and inference for both set A and set B for beam spatial and temporal prediction

Proposal 17: RAN4 to consider option 1 for achieving consistency between training and inference for only static conditions.

Proposal 18:  In order to ease the burden for testing models with different NW additional conditions, it would beneficial to train the UE-side model with mixed dataset from various gNB settings, thus reducing the number of AI/ML models (selected by NW-side additional conditions) required to guarantee generalization and maintain the system performance for BM use case

Proposal 19: For achieving consistency between training and inference, if the model drifts due to misalignment of network-side additional conditions, the alignment could be achieved through an ID assigned by the network during training data collection. This ID indicates the association of the training data with the additional conditions implied to generate those data

Proposal 20: For achieving consistency between training and inference, if the UE lacks the ID but the network possesses the model ID, then the network can transfer the model to the UE, and the UE can update its list of models

Proposal 21: For achieving consistency between training and inference, if the UE has a model that partially supports the network-sided additional conditions, then the network can trigger data collection for fine-tuning the UE-sided model. Subsequently UE updates its list of model IDs.

Proposal 22: The UE updates its model list with a new model derived from either fine-tuning, model transfer from the NW, or monitoring to ensure the consistency of additional conditions. Then, the UE assigns an ID to the new model that supports the NW's additional conditions and shares this information with the NW. If some of the new conditions/configurations are standardized, the UE updates its capability report accordingly.

Proposal 23: If there is no ID available to associate training data with additional conditions, and monitoring procedure fails to guarantee the consistency of the model with the additional conditions then UE should fallback to legacy mode.

Proposal 24:  Investigate the feasibility of enhancing the generalizability of the AI/ML model and reducing the number of AI/ML models and the testing burden for the beam management case by supplementing the core AI/ML input signals with both network (NW) and  UE auxiliary information signals integral to its inference engine

Proposal 25:  Investigate the feasibility of training the models with a mixed dataset associated with both network (NW) and UE auxiliary information signals to further enhance the generalizability of the AI/ML model for the beam management case and reduce the number of generalization tests.

- References

[1] RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#94e.

[2] RP-221348, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”,

[3] 3GPP TR 38.843, “Technical Specification Group Radio Access Network;Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface

[4] TR 38.810, v16.7.0, “Study on test methods”.