3GPP TSG-RAN WG4 Meeting #112                               			      				 R4-2411413

Maastricht, Netherlands, 19th – 23rd August 2024

Source: 	Apple

Title: 	in-GAP blocker impact on type 2 UE reconfiguration

Agenda Item:	5.7

Document for:	Discussion

# Introduction

Type 2 UE has been specified in Rel-18 for intra-band non-collocated CA and inter-band non-collocated EN-DC with overlapping DL frequency. This paper presents an issue that is relevant for type 2 UE RF reconfiguration.

# Discussion

In Rel-18, type 2 UE is specified assuming type 2 capability as default UE behaviour and RAN2 introduced BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for type 2 UE to reconfigure its RF to type 1 capability when collocated condition is met. Type 1 capability means Rx chain sharing between the aggregated non-contiguous CCs.

While during the discussion, there was no mentioning about the impact of the potential interference in the spectrum block between the aggregated CCs. Since CC1 and CC2 are already non-collocated. If there is an operator operating on the in-GAP spectrum between these 2 CCs, the signal from this operator will be a non-collocated signal either to CC1 or to CC2 or to both depending on the UE location and network deployment as illustrated in figure 2-1.

Figure 2-1: Deployment factors concerning type 2

The in-GAP signal is a interference for type 2 UE operating in CA between CC1 and CC2. The in-GAP interference level will largely impact whether the reconfiguration can be done or not as shown in figure 2-2.

(a) wrong reconfiguration                                                       (b) correct reconfiguration

Figure 2-1 type 2 UE reconfiguration in presence of in-GAP interference

Observation: The in-GAP signal from another operator will largely impact whether the reconfiguration can be done correctly.

Proposal: RAN4 discuss and confirm whether and how to factor in the in-GAP interference impact on type 2 UE reconfiguration.

# Summary

This contribution presented our view on additional factors that may impact type 2 UE reconfiguration. The following observation and proposal are concluded.

Observation: The in-GAP signal from another operator will largely impact whether the reconfiguration can be done correctly.

Proposal: RAN4 discuss and confirm whether and how to factor in the in-GAP interference impact on type 2 UE reconfiguration.

# References

[1] TS 38.101-1

[2] TS 38.101-3