3GPP TSG-RAN WG4 Meeting #112                               			      				 R4-2411414

Maastricht, Netherlands, 19th – 23rd August 2024

Source: 	Apple

Title: 	On remaining issue for type 4 UE RF requirement

Agenda Item:	8.5.2.1

Document for:	Discussion

# Introduction

RF requirement for Intra-band non-collocated NR CA/EN-DC has been discussed in the last RAN4 meeting and a WF was agreed [1]. The following issues are remaining open for further study.

< Issue 2-1-2:  How to capture the power imbalance and REFSENS requirements >

Agreement:

- Define the Type 4 receiver inmalande requirements in the same table as for Type 2.

  - FFS on the details of modification

< Issue 2-1-3:  Minimum DL frequency separation >

Way Forward:

- Continue further discussion on the following two options and conclude it in the next meeting.

  - Option 1. Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

  - Option 2. Max (5/2* BWanother, [50] MHz).

This contribution will give further analysis on these open issues and presents our views.

# Discussion

## How to capture the power imbalance requirements

In the last RAN4 meeting, it was agreed use 25dB power imbalance for 4Rx REFSENS + 1dB for Type 4 and define the type 4 UE receiver imbalance requirement in the same table as for type 2 UE. Though the detailed wording can be further reviewed during CR drafting phase, one thing yet to be clarified is how to capture this agreement in the table. In our view, there are 2 ways to capture these agreements in the power imbalance table.

Option 1: Adding NOTE 6 in Table 7.10A.2-1.

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: For UE indicating intraBandNR-CA-non-collocated-r18, REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. For UE indicating [intraBandNR-CA-non-collocatedType4-r19, REFSENS is the reference sensitivity level for four antenna port which is specified as the value in Table 7.3.2-1b modified by the amount ΔRIB,4R in Table 7.3.2-2. NOTE 5: Void. |  |  |  |  |

Option 2: Specified different Rx power for type 2 and type 4 UE.

- Note: For this option, it’s better to introduce definition for type 2 UE and type 4 UE in the specification.

| Test configurations | Carriers | Rx Power in transmission bandwidth configuration (dBm) | channel bandwidth | Center of BWanother Relative to edge of BWwanted |
| --- | --- | --- | --- | --- |
| 1 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | BWwanted ≤ BWanother | < max (5/2* BWanother, 50MHz |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| 2 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | BWwanted > BWanother |  |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 – 10*log10(BWwanted /BWanother) |  |  |
| 3 | Wanted carrier for type 2 UE | REFSENS NOTE 4 + 1 | NA | ≥ max (5/2* BWanother, 50MHz) |
|  | Wanted carrier for type 4 UE | REFSENS NOTE 4 + ΔRIB,4RNOTE 6 1 |  |  |
|  | Another wanted carrier | Power of wanted carrier + 25 |  |  |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,f,c at the minimum uplink configuration specified in Table 7.3.2-3 with PCMAX_L,f,c as defined in clause 6.2A.4. NOTE 2: BWwanted is the channel bandwidth of wanted carrier. BWanother is the channel bandwidth of another wanted carrier with 25 dB power imbalance. NOTE 3: It’s allowed to use one of test configurations to verify the RX power imbalance requirement for type 2 UE. NOTE 4: REFSENS is the reference sensitivity level for two antenna port in Table 7.3.2-1b. NOTE 5: Void. NOTE 6: ΔRIB,4R is the reference sensitivity allowance as specified in Table 7.3.2-2. |  |  |  |  |

Proposal 1: decide between Option 1 and Option 2 on how to capture power imbalance requirement for type 4 UE.

## Minimum downlink frequency separation

The following 2 options are open for further discussion.

  - Option 1. Center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

  - Option 2. Max (5/2* BWanother, [50] MHz).

In our view, these 2 options are not contradicted with each other but from different angle. Option 2 is the frequency boundary for different requirement applicability and channel bandwidth relation between wanted carrier and another carrier. While Option 1 is about the testing settings based on the band combos of interest for the time being. The latter one is to ensure that operators’ deployment demand (as shown in figure 2.2-1) as well as not to putting unnecessary implementation difficult and test effort for UE. This has been discussed in [2] for type 2 UE for non-collocated scenario. The minimum separation for non-contiguous case is 80MHz between B2 and B3. Since type 2 UE and type 4 UE both are for the same set of band combos, the same rules for minimum frequency separation should be applied for type 4 UE as well.

Proposal 2: it is proposed that center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

## Requirement applicability

The following has been agreed for type 4 UE

- Type 4 should support the following capabilities (Set 1) when operating in non-collocated scenarios.

  - Maximum 4 MIMO layers on each CC

  - Maximum power imbalance of 25dB

- Type 4 UE should support the following capabilities (Set 2=Type 1 capability) when operating in collocated scenarios.

  - Maximum 8 MIMO layers on each CC

    - It may be optionally supported

  - Maximum PSD imbalance of 6dB

.

With this agreement, it means there are 2 kind of type 4 UE. One kind is Capability set 1 capable UE. The other one is Capability set 1&2 capable UE, which is different from type 2 UE.

For capability set 1 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 4Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

For capability set 1&2 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 8Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

Proposal 3: Requirement applicability rules for type 4 UE,

- For capability set 1 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 4Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

- For capability set 1&2 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 8Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

# Summary

This contribution presented our consideration on the open issues for type 4 UE RF requirements. The following proposals are concluded.

Proposal 1: decide between Option 1 and Option 2 on how to capture power imbalance requirement for type 4 UE.

Proposal 2: it is proposed that center of BWanother relative to edge of BWwanted is assumed to be at least 80MHz+BWanother/2 away from the edge of the wanted CC.

Proposal 3: Requirement applicability rules for type 4 UE,

- For capability set 1 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 4Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

- For capability set 1&2 capable UE, it should be verified with power imbalance requirement for type 4 UE in 7.10A and 8Rx RF requirements for intra-band non-contiguous CA in 7.4A.2~7.8A.2.

# References

[1] R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI, RAN4#111

[2] R4-2311306, On frequency separation for intra-band non-collocated CA, Apple, RAN4#108