3GPP TSG-RAN WG4 Meeting #112                               			      				 R4-2411415

Maastricht, Netherlands, 19th – 23rd August 2024

Source: 	Apple

Title: 	On type 4 UE capability

Agenda Item:	8.5.2.2

Document for:	Discussion

# Introduction

Type 4 UE capability was discussed in the past RAN4 meetings the latest agreements were captured in the WF [1]. The following issues are remaining open for further study.

- Issue 2-2-1:  New UE Capabilities for Type 4a(EN-DC) and 4b(EN-DC/NR-CA)

- Issue 2-2-3:  New BS Signaling to switch between Type 4a/4b and Type 1(collocated)

- Issue 2-2-4:  New BS Signaling to switch between Type 4a/4b and Type 2

- Issue 2-2-5: When to inform RAN2 the demand on new UE capability(s) and new BS signaling(s)

- Issue 2-2-6:  UE behavior between Type 1, Type 2 and Type 4a/4b with new BS signaling

This contribution will give further analysis on these open issues and presents our views.

# Discussion

## New UE capabilities for Type 4 UE

In the last meeting, the following 2 options on how to define new UE capabilities were put on the table.

- Continue further discussion on the following two options and conclude it in the next meeting.

  - Option 1. Add new UE capabilities for Type 4a and Type 4b support indication.

  - Option 2. Add one new UE capability for Type 4 (no distinction between Type 4a and 4b) support indication.

Before deciding which option to go, we need to discuss how type 4 UE capabilities will be reflected by new signalling and existing signalling for CA and EN-DC. In our view, at first we need to introduce two new UE capabilities with 1bit to indicate the most critical character for EN-DC type 4 UE and CA type 4 UE respectively from RAN2 signalling structure perspective. Where, for both EN-DC and CA,

- When the bit is present, the UE is capable of handling up to 25dB power imbalance between carriers and up to 33us MRTD between carriers.

- When the bit is absent, the UE is capable of handling up to 6 dB power imbalance between CCs and up to 3us MRTD between carriers.

In addition, maxNumberMIMO-LayersPDSCH can be used to indicate the maximum number of MIMO layers supported on each CC since MIMO layer capability by this IE is reported per feature set per component carrier. And network should combine the new UE capability and maxNumberMIMO-LayersPDSCH to comprehend the overall capability of type 4 UE.

As we know, only 2Rx is mandatory and 4Rx is optional for Band 42 in LTE. It is the reason by which type 4a is motivated. So, for EN-DC type 4 UE, there are the following 2 options in terms of MIMO layer capabilities.

- max 2 MIMO layer in E-UTRA + max 4MIMO layer in NR (corresponding to type 4a UE)

- max 4 MIMO layer in E-UTRA + max 4MIMO layer in NR (corresponding to type 4b UE)

With the above-mentioned capability structure for type 4 UE, we think it is sufficient to differentiate these 2 kinds of EN-DC UE. E.g.

- Type 4a EN-DC UE is represented by new EN-DC UE capability reporting + indication of max 2 MIMO layer on E-UTRA carrier by maxNumberMIMO-LayersPDSCH.

- Type 4b EN-DC UE is represented by new EN-DC UE capability reporting + indication of max 4 MIMO layer on E-UTRA carrier by maxNumberMIMO-LayersPDSCH.

Proposal 1: It is proposed to introduce two new UE capabilities with 1bit to indicate the most critical character for EN-DC type 4 UE and CA type 4 UE respectively. Where, for both EN-DC and CA,

- When the bit is present, the UE is capable of handling up to 25dB power imbalance between carriers and up to 33us MRTD between carriers.

- When the bit is absent, the UE is capable of handling up to 6 dB power imbalance between CCs and up to 3us MRTD between carriers.

Proposal 2: The overall capability for type 4 UE is indicated by the new UE capability in proposal 1+inidication of maximum number of MIMO layers per CC by maxNumberMIMO-LayersPDSCH.

Proposal 3: No further distinction between Type 4a and 4b is needed for EN-DC type 4 UE from new UE capability perspective, i.e. Option 2.

## New BS signalling supporting type 4 UE switching

In Rel-18, RAN2 introduced BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC for type 2 UE for the case of maxMIMO-Layers with value less than or equal to 2. The description for nonCollocatedTypeNR-CA in TS 38.331 is reproduced as below. The description for nonCollocatedTypeMRDC is similar and omitted in this paper.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15] when indicating support of intraBandNR-CA-non-collocated-r18.

nonCollocatedTypeMRDC

This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

Though not mentioned in the above Rel-18 BS signalling nonCollocatedTypeNR-CA for CA and nonCollocatedTypeMRDC for EN-DC, it is by default 4Rx type 1 in collocated scenario when maxMIMO-layers equal to 4 for type 2 UE.

Observation 1: Rel-18 BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to switch between Type 1 and Type 2 is only specified for the case with maxMIMO-layers less than or equal to 2. It is by default 4Rx type 1 in collocated scenario when maxMIMO-layers equal to 4.

When it comes to type 4 UE, we think a similar approach could be considered for network indication supporting UE switching between 6/8Rx type 1 and type 4a/4b.

- When maxMIMO-layers equals 6 or 8, it is 6/8Rx type 1 in collocated scenario

- When maxMIMO-layers is less than or equal to 4, a clear network indication is needed to instruct UE to switch its RF chain between collocated scenario and non-collocated scenario.

  - Note: signaling details could be left for RAN2 decision. E.g. either change the existing signaling description to accommodate type 4 UE or introduce new signaling for type 4 UE.

Proposal 4: To support type 4 UE switching between type 1 capability and type 4 capability, consider the following proposals.

- When maxMIMO-layers equals 6 or 8, it is 6/8Rx type 1 in collocated scenario

- When maxMIMO-layers is less than or equal to 4, a clear network indication is needed to instruct UE to switch its RF chain between collocated scenario and non-collocated scenario.

  - Note: signaling details could be left for RAN2 decision. E.g. either change the existing signaling description to accommodate type 4 UE or introduce new signaling for type 4 UE.

## Type 2 capability for type 4 UE

At first, we would like to recall the following agreement for this issue in RAN4#111 meeting.

< Issue 2-2-2:  Whether to support Type 2 capability by UE having Type 4 capabilities >

Agreement:

- If UE reports the Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not.

  - If UE reports the Type 4 capability, it is not necessary for UE to report Type 2 capabilities.

  - If the issue is identified for these bullets, RAN4 will revisit them.

We think the previous agreement is reasonable and should be confirmed. Type 2 capability is a fall-back mode of type 4 capability. There is no need for a type 4 UE to report that it is capable of type 2 capability. Someone may argue that for legacy gNB who has no idea of Type-4 capability, UE still needs to report Type-2 UE capability if UE supports it. While our understanding is that, it should be recognized as a type 4 UE at first and then type 2 capability comes on top of the type 4 capability.  If type 4 UE cannot be recognized by the network at all, we just consider that this type of UE cannot access such network.

Proposal 5: Confirm that if UE reports Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not. It is not necessary for UE to report Type 2 capabilities.

The next issue is whether network indication is needed for UE switching between type 2 capability and type 4 capability. The answer is no since type 2 capability is just a fall-back mode of type 4 capability and can be done by network scheduling.

Proposal 6: no network indication is needed for type 4 UE to switch between type 2 capability and type 4 capability.

## UE behaviour between different capability set

For type 4 UE, there could be different working mode depending on different operating condition. The switch behaviour between non-collocated type 4 and other main modes is illustrated in figure 2.4-1.

Figure 2.4-1 illustration of UE behaviour and means to switch between different UE capabilities

It should be noted that,

- For non-collocated type 2, there could be further switching between collocated type 2 and collocate type 1 (4Rx). While these are already supported in Rel-18 and is omitted in the figure.

- For collocated type 1 with 4-layers/CC and 8-layers/CC, there will be further switching between them and other x-layers/CC. while it is just done by MIMO layer adaption as usual and is omitted in the figure.

Proposal 7: it is proposed that RAN4 confirm the UE behaviour for type 4 UE as illustrated in figure 2.4-1.

## Additional information for UE capability switching

In this section we will discuss how to maximize the potential of type 4 UE by additional assistant information.

In previous discussion, it is anticipated that type 4 UE will be work with type 4 capability in non-collocated condition and 6/8Rx type 1 capability in collocated condition. While how to determine the collocated/non-collocated condition is unclear. Without further assistant information, the network can only determine collocated condition and non-collocated condition based on network deployment (e.g. judge by TAE performance) which greatly limited type 4 UE potentials.

While even in a non-collocated deployment, there could be a lot of locations that a collocated condition is also met. As illustrated in figure 2.5-1, when type 4 UE is amounted in the area between the 2 TRPs, the propagation difference and power imbalance may not be that large and can also be scheduled with 6/8 Rx type 1 capability.

Figure 2.5-1 illustration of area satisfying collocated condition in a non-collocated scenario.

The simplest way is to introduce a RTD reporting to let network know that collocated condition is met and the UE can be scheduled as type 6/8 Rx capability even it is now in a non-collocated deployment scenario. Since type 4 UE is intended for FWA, such reporting only need to be done once.

Proposal 8: it is proposed to introduce RTD reporting to assist network scheduling type 4 UE with type 1 capability in non-collocated deployment.

Another issue is that in previous discussion the switching between type 1 and type 4 capability is considered as default only depending on whether collocation condition is met or not. While an important pre-requisition to use type 1 capability is there is no strong blocker in between the non-contiguous blocks to be aggregated. If there is strong blocker in-between, it is not ok to schedule type 4 UE with type 1 capability. It is proposed that RAN4 further investigate the condition to schedule type 4 UE with type 1 capability and further discuss whether to reflect such condition in the network indication signalling as a necessary condition.

(a) wrong reconfiguration                                                       (b) correct reconfiguration

Figure 2.5-2 illustration of RF reconfiguration in presence of in-GAP blocker

Proposal 9: RAN4 further investigate additional condition in terms of in-GAP blocker for type 2/4 UE reconfiguration as type 1 capability.

## LS to RAN2

As for when to inform RAN2 the demand on new UE capability(s) and new BS signalling(s), our proposal is that LS shall not be sent to RAN2 until all the above issues are clear in RAN4.

Proposal 10: LS to RAN2 shall not be sent until all issues are settled in RAN4.

# Summary

This contribution presented our consideration on the open issues for type 4 UE capability issue. The following observations and proposals are concluded.

Proposal 1: It is proposed to introduce two new UE capabilities with 1bit to indicate the most critical character for EN-DC type 4 UE and CA type 4 UE respectively. Where, for both EN-DC and CA,

- When the bit is present, the UE is capable of handling up to 25dB power imbalance between carriers and up to 33us MRTD between carriers.

- When the bit is absent, the UE is capable of handling up to 6 dB power imbalance between CCs and up to 3us MRTD between carriers.

Proposal 2: The overall capability for type 4 UE is indicated by the new UE capability in proposal 1+inidication of maximum number of MIMO layers per CC by maxNumberMIMO-LayersPDSCH.

Proposal 3: No further distinction between Type 4a and 4b is needed for EN-DC type 4 UE from new UE capability perspective, i.e. Option 2.

Observation 1: Rel-18 BS signalling nonCollocatedTypeNR-CA and nonCollocatedTypeMRDC to switch between Type 1 and Type 2 is only specified for the case with maxMIMO-layers less than or equal to 2. It is by default 4Rx type 1 in collocated scenario when maxMIMO-layers equal to 4.

Proposal 4: To support type 4 UE switching between type 1 capability and type 4 capability, consider the following proposals.

- When maxMIMO-layers equals 6 or 8, it is 6/8Rx type 1 in collocated scenario

- When maxMIMO-layers is less than or equal to 4, a clear network indication is needed to instruct UE to switch its RF chain between collocated scenario and non-collocated scenario.

  - Note: signaling details could be left for RAN2 decision. E.g. either change the existing signaling description to accommodate type 4 UE or introduce new signaling for type 4 UE.

Proposal 5: Confirm that if UE reports Type 4 capability, Type 2 capability shall be deemed as support by default regardless of whether UE indicates Type-2 capability or not. It is not necessary for UE to report Type 2 capabilities.

Proposal 6: no network indication is needed for type 4 UE to switch between type 2 capability and type 4 capability.

Proposal 7: it is proposed that RAN4 confirm the UE behaviour for type 4 UE as illustrated in figure 2.4-1.

Proposal 8: it is proposed to introduce RTD reporting to assist network scheduling type 4 UE with type 1 capability in non-collocated deployment.

Proposal 9: RAN4 further investigate additional condition in terms of in-GAP blocker for type 2/4 UE reconfiguration as type 1 capability.

Proposal 10: LS to RAN2 shall not be sent until all issues are settled in RAN4.

# References

[1] R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI, RAN4#111