3GPP TSG-RAN WG4 Meeting #112                               			      				 R4-2411416

Maastricht, Netherlands, 19th – 23rd August 2024

Source: 	Apple

Title: 	On maximum number of contiguous CCs

Agenda Item:	8.5.2.3

Document for:	Discussion

# Introduction

Number of contiguous CCs for LTE and NR has been discussed in the last RAN4 meeting and a WF was agreed in [1]. The following issues are remaining open for further study.

< Issue 2-3-1:  The number of B42 CCs for EN-DC >

Way Forward:

- Check the number with operators until next meeting again.

< Issue 2-3-2:  The number of NR CCs for EN-DC and NR-CA >

Way Forward:

- Check the number with operators until next meeting again.

This contribution will give further analysis on these open issues and presents our views.

# Discussion

As has been widely discussed, the motivation for Intra-band non-collocated NR CA/EN-DC is to address the Japanese operation deployment demand in 3.5GHz bands/band combinations. Below figure 2-1 shows the spectrum allocation for different operators.

Maximum number of LTE CCs

Block A1, B1, B2, C1, A2 are deployed with LTE technology in Japan. B1 and B2 are for the same operator, which means the maximum number of LTE CCs can be up to 4 (bandwidth class E) if considering 20MHz channel bandwidth. All other 3 blocks A1, C1 and A2 are either belong to different operators or non-contiguous. Hence, it is enough to accommodate up to 4 contiguous LTE CCs for inter-band EN-DC with overlapping DL frequency.

Proposal 1: It is proposed that the maximum number of contiguous LTE CCs is 4 (bandwidth class E) for inter-band EN-DC with overlapping DL frequency.

Maximum number of NR CCs

On the maximum number of NR CCs for EN-DC and NR CA, it is noted that the size of all spectrum blocks is below 100MHz width. from this perspective single NR CC should be sufficient for both NR CA and the NR mode of EN-DC. One point to be further confirmed is whether there is a demand to implement different co-existence requirement due to protection of incumbent service in part of a spectrum block, in which case multiple NR CCs might be needed.

Proposal 2: It is proposed to confirm whether there are cases where different co-existence requirements need to be implemented due to incumbent service in part of a spectrum block.

If the answer to proposal 2 is no, then single CC should be enough for NR CA and NR mode of EN-DC.

If the answer to proposal 2 is yes, we think it is sufficient to consider 2 contiguous CCs (e.g. bandwidth class C) within a spectrum block for NR mode. It is also noted that, to address such case for NR CA (if have) a combination of contiguous CA and non-contiguous CA is needed while it is still not specified for NR intra-band CA in Table 5.5A.2-1 of 38.101-1.

Proposal 3: It is proposed to consider bandwidth class C for NR CA or NR mode of EN-DC if the answer to proposal 2 is yes. Otherwise, one NR CC should be sufficient.

Observation:  For NR CA, the combination of contiguous CA and non-contiguous CA is still missing for Band n77 and n78.

# Summary

This contribution presented our consideration on the open issues for type 4 UE in terms of maximum number of CCs for NR and LTE. The following proposals are concluded.

Proposal 1: It is proposed that the maximum number of contiguous LTE CCs is 4 (bandwidth class E) for inter-band EN-DC with overlapping DL frequency.

Proposal 2: It is proposed to confirm whether there are cases where different co-existence requirements need to be implemented due to incumbent service in part of a spectrum block.

Proposal 3: It is proposed to consider bandwidth class C for NR CA or NR mode of EN-DC if the answer to proposal 2 is yes. Otherwise, one NR CC should be sufficient.

Observation:  For NR CA, the combination of contiguous CA and non-contiguous CA is still missing for Band n77 and n78.

# References

[1] R4-2410686, WF on Rel-19 non-collocated UE RF requirements, KDDI, RAN4#111

[2] R4-2311306, On frequency separation for intra-band non-collocated CA, Apple, RAN4#108