3GPP TSG-RAN WG4 Meeting #112                               			      				 R4-2411420

Maastricht, Netherlands, 19th – 23rd August 2024

Source: 	Apple

Title: 	Discussion on RRM requirements for type 4 UE

Agenda Item:	8.5.3

Document for:	Discussion

# Introduction

A Rel-19 WI on Support of intra-band non-collocated EN-DC/NR-CA deployment in [1]. The following scopes are included for study.

- Objectives for Rel-19 Support of intra-band non-collocated EN-DC/NR-CA deployment (RP-240838)

  - Specify the core requirements for type 4a/4b capable FWA UE that supports non-co-located scenarios for FR1 inter-band non-contiguous EN-DC with overlapping or partially overlapping bands and/or FR1 intra-band non-contiguous NR-CA for up to 4-layer DL MIMO per CC, where MRTD>CP is assumed.

    - RF power imbalance requirements of up to 25dB

    - RRM requirements including MTTD and MRTD requirements and other identified requirements such as scheduling restriction/availability, SCell operation related delay and interruption.

    - Work is limited to EN-DC/NR-CA for bands 42, n77/n78

  - Discuss and if needed, specify the UE behavior when the UE supports type 4a/b but operates in co-located scenarios depending on whether network signaling is provided. R18 handling of UE behavior can be used as a reference.

This contribution will present our consideration on RRM requirement impact for type 4 UE.

# Discussion

While the discussion on type 4 UE is still ongoing in RF session, it is anticipated that the type 4 UE behaves quite similar as type 2 UE except for the maximum number of MIMO layers supported on each CC in both collocated condition and non-collocated condition. And it is also like type 2 UE that, a type 4 UE is assumed to be able to switch its RF between type 1 capability (Rx chain sharing) and type 4 capability (separate RF chain sets for each CC). It is anticipated the RRM requirements to be satisfied will differ for the 2 kinds of RF configurations. The discussion of this paper assumes similar UE capability reporting and network indication will be introduced for type 4 UE and the default UE behaviour is type 4 capability.

The following requirements are expected to be impacted,

- MRTD/MTTD requirements

- Interruptions at SCell addition/release/activation/deactivation and interruptions during measurements on deactivated SCC.

- SCell activation delay requirements

- SCell BFD requirements

- Scheduling availability

## MTTD

For FR1 intra-band non-contiguous NR carrier aggregation, Type 4 UE shall be capable of handling at least a relative transmission timing difference as shown in Table 7.5.4-1, between slot timing of all FR1 pairs of TAGs provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r19] and [nonCollocatedTypeNR-CA-r19] is not provided, and in Table 7.5.4-2 (to be added for synchronous case) provided that it is not capable of [intraBandNRCA-NonCollocated-r19] or it is capable of [intraBandNRCA-NonCollocated-r19] and [nonCollocatedTypeNR-CA-r19] is provided.

Table 7.5.4-1: Maximum uplink transmission timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of TAGs | Maximum uplink transmission timing difference (µs) |
| --- | --- |
| FR1 | 34.6 |
| FR2-1 | 8.5 Note1 |
| Between FR1 and FR2-1 | 26.1 |
| Between FR1 and FR2-2 | 26.1 |
| Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping or partially overlapping DL bands, only synchronized operation is assumed. The UE shall be capable of handling a maximum uplink transmission timing difference (MTTD) between E-UTRA PCell and PSCell as shown in Table 7.5.2.1-1 provided that UE is capable of interBandMRDC-WithOverlapDL-Bands-r19 and not provided with nonCollocatedTypeMRDC-r19, and in Table 7.5.3-1 provided that UE does not indicate it is capable of interBandMRDC-WithOverlapDL-Bands-r19 or UE is capable of interBandMRDC-WithOverlapDL-Bands-r16 and provided with nonCollocatedTypeMRDC-r19.

Table 7.5.2.1-1 Maximum uplink transmission timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 35.21 |
| 15 | 30 | 35.21 |
| 15 | 60 | 35.21 |
| 15 | 120 | 35.21 |

Table 7.5.3-1: Maximum uplink transmission timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing in E-UTRA PCell (kHz) | UL Sub-carrier spacing for data in PSCell (kHz) | Maximum uplink transmission timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 5.21Note1,Note 2 |
| 15 | 30 | 5.21Note 2 |
| 15 | 60 | 5.21 Note 2 |
| NOTE 1: This is not applicable for a UE which indicates the capability of only supporting single UL timing (ul-TimingAlignmentEUTRA-NR is signalled). Single UL timing for E-UTRA and NR cell is assumed for this UE. NOTE 2: If the transmission timing difference exceeds the cyclic prefix length of the UL Sub-carrier spacing for data in PSCell, NR UE Tx EVM degradation is expected for the symbol that is overlapping the LTE subframe boundary NOTE 3: The requirement in this table is also applicable for intra-band synchronous NE-DC. |  |  |

Proposal 1: It is proposed to define MTTD requirement for type 4 UE as proposed in this section 2.1.

## MRTD

For FR1 intra-band non-contiguous NR carrier aggregation, Type 4 UE shall be capable of handling at least a relative receive timing difference as shown in 7.6.4-2, between slot timing of different FR1 carriers to be aggregated at the UE receiver provided that UE indicates that it is capable of [intraBandNRCA-NonCollocated-r19] and [nonCollocatedTypeNR-CA-r19] is not provided, and in Table 7.6.4-1 provided that it is not capable of [intraBandNRCA-NonCollocated-r19] or it is capable of [intraBandNRCA-NonCollocated-r19] and [nonCollocatedTypeNR-CA-r19] is provided.

Table 7.6.4-1: Maximum receive timing difference requirement for intra-band non-contiguous NR carrier aggregation

| Frequency Range | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 31 |
| FR2-1 | 0.26 |
| Note 1: In the case of different SCS on different CCs, if the receive time difference exceeds the cyclic prefix length of that SCS, demodulation performance degradation is expected for the first symbol of the slot. |  |

Table 7.6.4-2: Maximum receive timing difference requirement for inter-band NR carrier aggregation

| Frequency Range of the pair of carriers | Maximum receive timing difference (µs) |
| --- | --- |
| FR1 | 33 |
| FR2-1 | 8 note1 |
| Between FR1 and FR2-1 | 25 |
| Between FR1 and FR2-2 | 25 |
| Note1: This requirement applies to the UE capable of independent beam management for FR2-1 inter-band CA. |  |

For E-UTRA TDD-NR TDD inter-band EN-DC with overlapping or partially overlapping DL bands, only synchronized operation is assumed. The UE shall be capable of handling at least a relative receive timing difference between subframe timing of signal from a E-UTRA cell belonging to the MCG and slot timing of signal from a cell belonging to the SCG at the UE receiver as shown in Table 7.6.2.1-1 provided that UE is capable of interBandMRDC-WithOverlapDL-Bands-r19 and not provided with nonCollocatedTypeMRDC-r19, and in Table 7.6.3-1 provided that UE is not capable of interBandMRDC-WithOverlapDL-Bands-r19 or UE is capable of interBandMRDC-WithOverlapDL-Bands-r19 and provided with nonCollocatedTypeMRDC-r19.

Table 7.6.2.1-1: Maximum receive timing difference requirement for inter-band synchronous EN-DC

| Sub-carrier spacing of E-UTRA cell in MCG (kHz) | DL Sub-carrier spacing of cell in SCG (kHz) (Note1) | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 33 |
| 15 | 30 |  |
| 15 | 60 |  |
| 15 | 120 |  |
| Note 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. |  |  |

Table 7.6.3-1 Maximum receive timing difference requirement for intra-band synchronous EN-DC

| Sub-carrier spacing of E-UTRA cell in MCG (kHz) | DL Sub-carrier spacing of cell in SCG (kHz) Note1 | Maximum receive timing difference (µs) |
| --- | --- | --- |
| 15 | 15 | 3 |
| 15 | 30 | 3 |
| 15 | 60 | 3 |
| NOTE 1: DL Sub-carrier spacing is min{SCSSS, SCSDATA}. NOTE 2: The requirement in this table is also applicable for intra-band synchronous NE-DC. |  |  |

Proposal 2: It is proposed to define MRTD requirement for type 4 UE as proposed in this section 2.2.

## Interruptions

For type 4 UE, since separate set of RF chains is used to handle the Rx signal for each CC, it behaves like an inter-band CA UE when receiving signals from 2CC in the same band. The legacy FR1 inter-band CA interruption requirements apply for interruption at SCell addition/release/activation/deactivation and interruption during measurements on deactivated SCC.

Some clarifications are needed with regard to the UE capability reporting signalling and network indication. Can be settled when the signalling name is confirmed.

Proposal 3: The legacy FR1 inter-band CA interruption requirements apply for type 4 UE in the aspects of interruption at SCell addition/release/activation/deactivation and interruption during measurements on deactivated SCC.

## SCell activation delay requirements

The existing SCell activation delay requirements in clause 8.3.2 apply for type 4 UE if the values of TSMTC_MAX and TFirstSSB_MAX for FR1 inter-band CA SCell activation delay requirements is reused for FR1 intra-band non-contiguous CA.

Some clarifications are needed with regard to the UE capability reporting signalling and network indication. Can be settled when the signalling name is confirmed.

Proposal 4: The existing SCell activation delay requirements in clause 8.3.2 apply for type 4 UE if the values of TSMTC_MAX and TFirstSSB_MAX for FR1 inter-band CA SCell activation delay requirements is reused for FR1 intra-band non-contiguous CA.

## SCell BFD requirements

Similar as for type 2, it is suggested that that Type 4 UE can be configured to perform beam failure detection on no more than 2 serving cells per band if these 2 serving cells are in non-contiguous carriers.

Proposal 5: It is suggested that that Type 4 UE can be configured to perform beam failure detection on no more than 2 serving cells per band.

## Scheduling availability

Scheduling availability requirement should be introduced for type 4 UE performing beam failure detection with a different subcarrier spacing as PDSCH/PDCCH on FR1, L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1, measurement in TDD band, or measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1.

Proposal 6: Scheduling availability requirement should be introduced for type 4 UE performing beam failure detection with a different subcarrier spacing as PDSCH/PDCCH on FR1, L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1, measurement in TDD band, or measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1.

# Summary

This contribution presented our view on additional factors that may impact type 2 UE reconfiguration. The following observation and proposal are concluded.

Proposal 1: It is proposed to define MTTD requirement for type 4 UE as proposed in this section 2.1.

Proposal 2: It is proposed to define MRTD requirement for type 4 UE as proposed in this section 2.2.

Proposal 3: The legacy FR1 inter-band CA interruption requirements apply for type 4 UE in the aspects of interruption at SCell addition/release/activation/deactivation and interruption during measurements on deactivated SCC.

Proposal 4: The existing SCell activation delay requirements in clause 8.3.2 apply for type 4 UE if the values of TSMTC_MAX and TFirstSSB_MAX for FR1 inter-band CA SCell activation delay requirements is reused for FR1 intra-band non-contiguous CA.

Proposal 5: It is suggested that that Type 4 UE can be configured to perform beam failure detection on no more than 2 serving cells per band.

Proposal 6: Scheduling availability requirement should be introduced for type 4 UE performing beam failure detection with a different subcarrier spacing as PDSCH/PDCCH on FR1, L1-RSRP measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1, measurement in TDD band, or measurement with a same subcarrier spacing as PDSCH/PDCCH on FR1.

# References

[1] TS 38.101-1

[2] TS 38.101-3