3GPP TSG-RAN WG4 Meeting #112                               			      				 R4-2411422

Maastricht, the Netherland, 19th – 23rd August 2024

Source: 	Apple

Title: 	On CSI-RS based L1-RSRP measurement

Agenda Item:	8.23.2

Document for:	Discussion

# Introduction

A new work item on NR mobility enhancements Phase 4 was approved in [1]. One of the most important objectives is to specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM. This contribution will give tentative impact analysis on RRM requirement for this objective.

# Discussion

## Preliminary analysis for CSI-RS based measurement

### Pre-requisite for CSI-RS based L1-RSRP

In our view, it is only meaningful to perform CSI-RS based L1-RSRP measurement for neighbour cells that have good enough quality. The cells for L1-RSRP measurement could be selected from those ranked based on L3 measurement. So, the next question is which type of L3 measurement should be used to determine the ranking for target cells for CSI-RS based L1-RSRP measurement. In the current RRM spec, there are SSB based L3 measurement and CSI-RS based L3 measurement. From technical perspective, both can be used as the pre-requisite condition for cell quality ranking. While we all know that CSI-RS based L3 measurement is only specified assuming the existence of associated SSB and it is not a standalone measurement. In this regard, using CSI-RS based L3 measurement as condition will unnecessarily complicate the measurement design. Our view is that it is sufficient to only use SSB based L3 measurement as the prerequisite condition to determine the target cells for CSI-RS based L1-RSRP measurement. In this case it is by default to consider the scenario for CSI-RS based L1 measurement with associated SSB, which is also aligned with L3 CSI-RS measurement principle in terms of timing acquisition.

Proposal 0: It is proposed to consider SSB based L3 measurement as the pre-requisite condition to determine the target cells for CSI-RS based L1-RSRP measurement.

### Definition of intra-frequency and inter-frequency

For SSB based L1-RSRP measurement for LTM, it was decided to follow the definition of L3 measurement.

Following the same logic, for CSI-RS based L1 RSRP measurement, we think it’s reasonable to follow the intra-frequency/inter-frequency definition for CSI-RS based L3 measurement. i.e. a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement

Proposal 1: it is proposed to follow intra-frequency/inter-frequency definition for CSI-RS based L3 measurement, e.g. a measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

### Scenarios to be considered

#### Frequency range

It seems there is no strong motivation to consider FR2-2 in CSI-RS based LTM and L1-RSRP measurement, and SSB based LTM and L1-RSRP measurement does not consider FR2-2 as well. Hence, we suggest only focusing FR1 and FR2-1.

Proposal 2: it is proposed only consider FR1 and FR2-1 for CSI-RS based LTM and L1-RSRP measurement.

#### Measurement type

L3 based CSI-RS only considered intra-frequency without GAP and inter-frequency with GAP. As for CSI-RS based L1 RSRP measurement, it is suggested to follow the same logic as CSI-RS based L3 measurement to prioritize intra—frequency measurement without GAP and inter-frequency measurement with GAP as starting point. Inter-frequency measurement without GAP seems benefit from measurement delay perspective. While CSI-RS resource configuration is quite flexible and complicated. Our preference would be to deprioritize inter-frequency measurement without GAP before resource configuration issue is clear.

Proposal 3: It is proposed to prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP for CSI-RS based L1 measurement.

#### RTD assumption

It is possible that RTD between serving cell and neighbor cell on the same carrier is larger than CP from deployment perspective. While it will put a lot complexity for UE implementaiton complexity in terms of inreased number of FFTs. We think RTD<CP should be taken as the baseline. It is noted that R18 SSB based L1 RSRP measurement on neighbor cell already addressed the case when RTD is larger than CP by UE capability. We can accept to introduce similar capability for CSI-RS based L1 RSRP measurement.

Proposal 4: RTC<CP is taken as baseline. For the case of RTD>CP between serving cell and neighbour cell on the same carrier, UE capability should be introduced.

#### CSI-RS resource configuration

According to legacy physiscal layer design, CSI-RS can be allocated with different PRB width, e.g. PRB= RB = {24，48，96，192，256} with different RE density within each PRB, D = {0.5，1，3}. RAN2 further limited D = {1，3} configuration for RRM measurement. It is unclear whether RAN1 would have some new restriction for L1 measurement on neighbour cell. It means CSI-RS resource configuration can be very flexible. In Rel-16 CSI-RS based L3 measurement discussion, several confiurations has been discussed. In the end, only {D=3, PRB≧48} was chosen for requirement development based on trade off between measruement accuracy and UE power consumption. We propose to reuse the same resource configuration of {D=3, PRB≧48} as a starting point.

Proposal 5: the same CSI-RS resource configuration {D=3, PRB≧48} as for CSI-RS based L3 measurement is considered as a starting point. FFS on resource location limitation.

#### Know/unknown cell

From LTM delay and measurement delay perspective, it is suggested to consider know cell only since the measurement delay will be quite long for unknown cell. In Rel-18 SSB based L1-RSRP discussion, the cell is considered as known if the following conditions are met:

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements in clause 9.2.

When it comes to CSI-RS based L1-RSRP measurement, the question is whether we choose CSI-RS based L3 measurement during the last 5s or SSB based L3 measurement during the last 5s as the decisive condition for known cell. In Rel-16, CSI-RS based L3 measurement was introduced by only assuming associated SSB timing as pre-requisite condition and CSI-RS cannot be used for neighbour cell measurement as a standalone reference signal. Hence, CSI-RS based L3 measurement is not the good choice to determine the known cell condition. We propose that the pre-requisite condition for CSI-RS based L1-RSRP measurement is also based on whether UE has performed SSB based L3 measurement in the last 5s.  For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met,

-	The UE has performed L3 measurement on the target cell during the last 5 seconds, and

-	The CSI-RS from the target cell configured for L1 measurement remains detectable.

Proposal 6: It is proposed to consider know cell case only.

Proposal 7: The target cell is considered as known provided that the UE has performed L3 measurement on the target cell -during the last 5 seconds, and the CSI-RS configured for L1 measurement remains detectable.

#### Accuracy

In legacy CSI-RS L1 measurement accuracy is defined as ±5dB at SINR=-3dB (48 PRBs and the density is 3, one sample). It is suggested to reuse the existing side condition for CSI-RS based L1-RSRP measurement for serving cell as the starting point for CSI-RS based L1-RSRP measurement for neighbour cell.

Proposal 8: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point.

# Summary

This contribution presented our consideration on CSI-RS based L1-RSRP measurement. The following proposals are concluded.

Proposal 0: It is proposed to consider SSB based L3 measurement as the pre-requisite condition to determine the target cells for CSI-RS based L1-RSRP measurement.

Proposal 1: it is proposed to follow intra-frequency/inter-frequency definition for CSI-RS based L3 measurement, e.g. a measurement is defined as a CSI-RS based intra-frequency measurement provided that:

-	the SCS of the CSI-RS resource of the neighbour cell configured for measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for measurement, and

-	the CP type of the CSI-RS resource of neighbour cell configured for measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for measurement, and

-	It is applied for SCS = 60KHz

-	the centre frequency of the CSI-RS resource of the neighbour cell configured for measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for measurement

Proposal 2: it is proposed only consider FR1 and FR2-1 for CSI-RS based LTM and L1-RSRP measurement.

Proposal 3: It is proposed to prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP for CSI-RS based L3 measurement.

Proposal 4: RTC<CP is taken as baseline. For the case of RTD>CP between serving cell and neighbour cell on the same carrier, UE capability should be introduced.

Proposal 5: the same CSI-RS resource configuration {D=3, PRB≧48} as for CSI-RS based L3 measurement is considered. FFS on resource location limitation.

Proposal 6: It is proposed to consider know cell case only.

Proposal 7: The target cell is considered as known provided that the UE has performed L3 measurement on the target cell during the last 5 seconds, and the CSI-RS configured for L1 measurement remains detectable.

Proposal 8: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point.

# References

[1] RP-234036, New WID: NR mobility enhancements Phase 4, Apple, RAN#102