3GPP TSG-RAN WG4 Meeting #112                                  R4-2411423

Electronic Meeting, August 15 – August 26, 2022

Source: 	Apple

Title: 	RRM requirement impact for ATG

Agenda Item:	8.10.4

Document for:	Discussion

# Introduction

An Rel-19 WI on ATG was approved in [1]. RRM requirements need to be developed for CA for ATG UE.

The core part of the work item includes:

- Specify the RF and RRM core requirements for intra-band co-located and inter-band co-located DL CA [RAN4]:

  - FR1 intra-band contiguous CA

    - Example band combination: n79

  - FR1+FR1 inter-band CA

    - Example band combination: n3+n39

- Specify the RF requirements for support of UL MIMO with 2TX for single CC for UE. [RAN4]

      - Specify signaling and UE capabilities if necessary [RAN2].cell sizes and other relevant aspects

    - Specify new UE/BS type(s) for ATG network if necessary

This contribution will present an overview on the requirement structure and give initial analysis on the RRM requirements impact dur to CA for ATG.

# Discussion

The analysis is focused on Idle Mode CA/DC, MRTD/MTTD, RRC_INACTIVE state CA/DC Measurement, interruptions for various features.

| Sub-clause | Requirement | Comments |
| --- | --- | --- |
| 3 | Definitions, symbols and abbreviations |  |
| 3.5 | Frequency band grouping |  |
| 3.6 | Applicability of requirements |  |
| 4 | RRC_IDLE state mobility |  |
| 4.1 | Cell Selection |  |
| 4.2 | Cell Res-election |  |
| 4.3 | Minimization of Drive tests (MDT) |  |
| 4.4 | Idle Mode CA/DC measurement | This feature is intended for fast CA/DC establishment after RRC connection setup. Regarding ATG, it is intended for in-flight eMBB service. There is little chance for ATG UE to go to RRC_IDLE mode since we can assume that there is always data connection demand from some passengers. It’s hard to see that all passengers go to seep even in red-eye flight. We think this feature has little meaning for ATG UE. But if there is interest to keep this for ATG, it’s also fine since there is no additional effort and requirement can be reused directly. |
| 5 | RRC_INACTIVE state mobility |  |
| 5.1 | Cell Re-selection |  |
| 5.3 | Minimizaiton of Drive Test (MDT) |  |
| 5.4 | Idle Mode CA/DC Measurement | Same analysis as for Idle Mode CA/DC measurement in section 4.4. this feature has meaningless for ATG UE. But if there is interest to keep this for ATG, it’s also fine since there is no additional effort and requirement can be reused directly. |
| 5.5 | Configured Grant based small date transitions |  |
| 5.6 | NR measurements for positioning |  |
| 6 | RRC_CONNECTED state mobility |  |
| 6.1 | Handover |  |
| 6.2 | RRC Connection Mobility Control |  |
| 7 | Timing |  |
| 7.1 | UE transmit timing |  |
| 7.2 | UE timer accuracy |  |
| 7.3 | Timing advanced |  |
| 7.4 | Cell phase synchronization |  |
| 7.5 | Maximum Transmission Timing Difference | In section 7.5.4, MTTD requirement is only specified for inter-band CA and intra-band non-contiguous CA for non-collocated scenario. For ATG UE, only collocated scenario is considered for both intra-band contiguous CA_n79C and inter-band CA_n3-n39. It is suggested that RAN4 discuss whether MTTD requirement for collocated scenario is needed for ATG supporting intra-band contiguous CA and inter-band CA. |
| 7.6 | Maximum Receiving Time Difference | In section 7.6.4, MRTD requirement is specified for intra-band non-contiguous CA for collocated scenario only and for inter-band CA for non-collocated scenario. It is suggested that RAN4 discuss whether MRTD requirement for collocated scenario need to be specified for intra-band contiguous CA and inter-band CA. |
| 7.7 | deriveSSB-IndexFromCell tolerance |  |
| 7.9 | deriveSSB-IndexFromCell tolerance |  |
| 8 | Signaling characteristics |  |
| 8.1 | Radio link monitoring |  |
| 8.2 | Interruption |  |
| 8.2.2 | Introduction | Some adaption on the requirement applicability is needed depending on the supported feature by ATG UE. |
| 8.2.2.2.1 | Interruption at Scell addition/release | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. FR2 and FR1 non-collocated part should be deleted. |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. FR2 and FR1 non-collocated part should be deleted. |
| 8.2.2.2.3 | Interruptions during measurements on deactivated SCC | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. FR2 and FR1 non-collocated part should be deleted. |
| 8.2.2.2.4 | Interruptions at UL carrier RRC reconfiguration | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. SUL part should be deleted. |
| 8.2.2.2.5 | Interruptions due to Active BWP switching Requirement | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. |
| 8.2.2.2.6 | Interruptions at inter-frequency SFTD measurement | The requirement can be reused by focusing on the applicable part for ATG by considering the same corresponding scenario. |
| 8.2.2.2.7 | Interruptions at SCell activation/deactivation with multiple downlink SCells | Not applicable to ATG UE due to single Scell in Rel-19. |
| 8.2.2.2.8 | Interruptions due to UE-specific CBW change | Can be completely reused for ATG UE. |
| 8.2.2.2.9 | Interruptions at NR SRS carrier based switching | Can be reused for ATG UE. |
| 8.2.2.2.10 | DL Interruptions at UE switching between two uplink carriers | RAN4 need to discuss whether ATG UE would aim to support the feature of UE switching between 2 uplink carriers. We think current inter-band CA_n3-n39 combination seem has no such demand. |
| 8.2.2.2.10A | DL Interruptions at UE switching between two uplink carriers with two transmit antenna connectors | Same comment as for 8.2.2.10. |
| 8.2.2.2.10B | DL Interruptions at UE switching between one uplink band with one transmit antenna connector and one uplink band with two transmit antenna connectors | Same comment as for 8.2.2.10. |
| 8.2.2.2.10C | DL Interruptions at UE switching between two uplink bands with two transmit antenna connectors | Same comment as for 8.2.2.10. |
| 8.2.2.2.10D | DL Interruptions at UE switching across three or four uplink bands | Same comment as for 8.2.2.10. |
| 8.2.2.2.11 | Interruptions at direct SCell activation | Can be reused. |
| 8.2.2.2.12 | Interruptions due to SCell dormancy | Can be reused. |
| 8.2.2.2.13 | Interruptions at transitions between active and non-active during DRX | Not applicable |
| 8.2.2.2.14 | Interruptions when identifying CGI of an NR cell with autonomous gaps | Seems not useful for ATG UE. |
| 8.2.2.2.15 | Interruptions when identifying CGI of an E-UTRA cell with autonomous gaps | Not applicable |
| 8.2.2.2.16 | Interruptions at NR SRS antenna port switching | Can be reused. |
| 8.2.2.2.17 | Interruptions at fast SCell activation | Can be reused. |
| 8.2.2.2.18 | Interruptions due to PUCCH SCell activation/deactivation | Can be reused. |
| 8.2.2.2.19 | Interruptions due to measurements without gap carried out by UE supporting [NeedForInterruptionInfoNR-R18] | Can be reused for ATG. |
| 8.2.2.2.20 | Interruptions due to PDCCH ordered RACH on target LTM cell | ATG cell switching is not so frequent. And LTM feature is still in the process of optimizing and being enhanced. We think it’s not necessary to complicate the ATG mobility at this stage. If there is real demand for this feature, can be reconsidered in future release when LTM feature is stable for TN. |

# Summary

This contribution presented some analysis on ATG UE RRM requirements in the aspects of Idle Mode CA/DC, MRTD/MTTD, RRC_INACTIVE state CA/DC Measurement, interruptions for various features.

# References

[1] RP-240839, New WID on Enhancements for Air-to-ground network for NR, CMCC