3GPP TSG-RAN WG4 Meeting #112 					             					         R4-2411424

Maastricht, Netherlands, August 19 – 23, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4664 | rev |  | Current version: | 16.20.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on RSRP Measurement Report Mapping - R16 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_CSIRS_L3meas-Core |  |  |  |  |  | Date: |  |  | 2024-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current RRM requirements of RSRP Measurement Report Mapping, differential SS-RSRP and CSI-RSRP applies to both L1 and L3 reporting. However, differential L3 reporting is not supported. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Remove L3 reporting related content in differential RSRP reporting requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Specification would still be incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< START OF CHANGES >>

### 10.1.6 RSRP Measurement Report Mapping

The reporting range of SS-RSRP and CSI-RSRP for L3 reporting is defined from -156 dBm to -31 dBm with 1 dB resolution. The reporting range of SS-RSRP and CSI-RSRP for L1 reporting is defined from -140 to -44 dBm with 1 dB resolution.

The mapping of measured quantity is defined in Table 10.1.6.1-1. The range in the signalling may be larger than the guaranteed accuracy range.

The reporting range of differential SS-RSRP and CSI-RSRP for L1 reporting is defined from 0 dB to -30 dB with 2 dB resolution.

The mapping of measured quantity is defined in Table 10.1.6.1-2. The range in the signalling may be larger than the guaranteed accuracy range.

Table 10.1.6.1-1: SS-RSRP and CSI-RSRP measurement report mapping

| Reported value | Measured quantity value (L3 SS-RSRP and CSI-RSRP) | Measured quantity value (L1 SS-RSRP and CSI-RSRP) | Unit |
| --- | --- | --- | --- |
| RSRP_0 | RSRP<-156 | Not valid | dBm |
| RSRP_1 | -156≤RSRP<-155 | Not valid | dBm |
| RSRP_2 | -155≤RSRP<-154 | Not valid | dBm |
| RSRP_3 | -154≤RSRP<-153 | Not valid | dBm |
| RSRP_4 | -153≤RSRP<-152 | Not valid | dBm |
| RSRP_5 | -152≤RSRP<-151 | Not valid | dBm |
| RSRP_6 | -151≤RSRP<-150 | Not valid | dBm |
| RSRP_7 | -150≤RSRP<-149 | Not valid | dBm |
| RSRP_8 | -149≤RSRP<-148 | Not valid | dBm |
| RSRP_9 | -148≤RSRP<-147 | Not valid | dBm |
| RSRP_10 | -147≤RSRP<-146 | Not valid | dBm |
| RSRP_11 | -146≤RSRP<-145 | Not valid | dBm |
| RSRP_12 | -145≤RSRP<-144 | Not valid | dBm |
| RSRP_13 | -144≤RSRP<-143 | Not valid | dBm |
| RSRP_14 | -143≤RSRP<-142 | Not valid | dBm |
| RSRP_15 | -142≤RSRP<-141 | Not valid | dBm |
| RSRP_16 | -141≤RSRP<-140 | RSRP<-140 | dBm |
| RSRP_17 | -140≤RSRP<-139 | -140≤RSRP<-139 | dBm |
| RSRP_18 | -139≤RSRP<-138 | -139≤ RSRP<-138 | dBm |
| … | … |  | … |
| RSRP_111 | -46≤RSRP<-45 | -46≤ RSRP<-45 | dBm |
| RSRP_112 | -45≤RSRP<-44 | -45≤ RSRP<-44 | dBm |
| RSRP_113 | -44≤RSRP<-43 | -44≤ RSRP | dBm |
| RSRP_114 | -43≤RSRP<-42 | Not valid | dBm |
| RSRP_115 | -42≤RSRP<-41 | Not valid | dBm |
| RSRP_116 | -41≤RSRP<-40 | Not valid | dBm |
| RSRP_117 | -40≤RSRP<-39 | Not valid | dBm |
| RSRP_118 | -39≤RSRP<-38 | Not valid | dBm |
| RSRP_119 | -38≤RSRP<-37 | Not valid | dBm |
| RSRP_120 | -37≤RSRP<-36 | Not valid | dBm |
| RSRP_121 | -36≤RSRP<-35 | Not valid | dBm |
| RSRP_122 | -35≤RSRP<-34 | Not valid | dBm |
| RSRP_123 | -34≤ RSRP<-33 | Not valid | dBm |
| RSRP_124 | -33≤RSRP<-32 | Not valid | dBm |
| RSRP_125 | -32≤RSRP<-31 | Not valid | dBm |
| RSRP_126 | -31≤RSRP | Not valid | dBm |
| RSRP_127 (Note) | Infinity | Infinity | dBm |
| Note: The value of RSRP_127 is applicable for RSRP threshold configured by the network as defined in TS 38.331 [2], but not for the purpose of measurement reporting. |  |  |  |

Table 10.1.6.1-2: Differential SS-RSRP and CSI-RSRP measurement (for L1 reporting) report mapping

| Reported value | Measured quantity value (difference in measured RSRP from strongest RSRP) | Unit |
| --- | --- | --- |
| DIFFRSRP_0 | 0≥ΔRSRP>-2 | dB |
| DIFFRSRP_1 | -2≥ΔRSRP>-4 | dB |
| DIFFRSRP_2 | -4≥ΔRSRP>-6 | dB |
| DIFFRSRP_3 | -6≥ΔRSRP>-8 | dB |
| DIFFRSRP_4 | -8≥ΔRSRP>-10 | dB |
| DIFFRSRP_5 | -10≥ΔRSRP>-12 | dB |
| DIFFRSRP_6 | -12≥ΔRSRP>-14 | dB |
| DIFFRSRP_7 | -14≥ΔRSRP>-16 | dB |
| DIFFRSRP_8 | -16≥ΔRSRP>-18 | dB |
| DIFFRSRP_9 | -18≥ΔRSRP>-20 | dB |
| DIFFRSRP_10 | -20≥ΔRSRP>-22 | dB |
| DIFFRSRP_11 | -22≥ΔRSRP>-24 | dB |
| DIFFRSRP_12 | -24≥ΔRSRP>-26 | dB |
| DIFFRSRP_13 | -26≥ΔRSRP>-28 | dB |
| DIFFRSRP_14 | -28≥ΔRSRP>-30 | dB |
| DIFFRSRP_15 | -30≥ΔRSRP | dB |

<< END OF CHANGES >>