3GPP TSG-RAN WG4 Meeting #112		R4-2411429

Maastricht, Netherlands, 19 – 23, 2024

Agenda Item:	5.15.1

Source: 	Apple

Title:	Discussion of R18 gap core requirements maintenance

Document for:	Approval

# Introduction

Core part of joint configuration of pre-configured MGs, multiple concurrent MGs and NCSG has been finalized. However, there are still some remaining issues based on the maintenance discussion in previous RAN4 meeting. The last agreements can be found in [1].

In this contribution, we continue discussing the open issues.

# Discussion

The only remaining issue is about the UE behavior for deactivated SCell measurements with NCSG:

### Sub-topic 3-2: Rel-18 UE behavior for deactivated SCell measurements with NCSG

Issue 3-2-1: [Case 2] When the UE is configured with Concurrent gaps with NCSG, what is the potential changes to UE behaviour for NCSG upon SCell activation (in Rel-18)

Agreement from online session:

- For UE configured with one NCSG and one Type 1/2 MG: All deactivated SCells are measured within NCSG, regardless of the reported UE capabilities [and gap association].

  - Further details on the processing delay between NCSG and Type 1/2 MG can be further discussed.

- For UE configured with 2 NCSG, deactivated SCells are measured with NCSG

  - If the association is provided, deactivated SCells are measured with NCSG according to gap association.

  - If the association is not provided, UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells, and the existing measurement delay requirement does not apply to this case.

According to the above agreement, measurement type of MO on the SCC may change upon SCell activation/deactivation. Following are the possible transitions in R18:

- Intra-frequency measurement without gap -> intra-frequency measurement with gap

- Intra-frequency measurement with gap -> intra-frequency measurement without gap

- Intra-frequency measurement without gap -> intra-frequency measurement with NCSG

- Intra-frequency measurement with NCSG -> intra-frequency measurement without gap

- Intra-frequency measurement with Type 1/2 MG -> intra-frequency measurement with NCSG

- Intra-frequency measurement with NCSG-> intra-frequency measurement with Type 1/2 MG

Scenario 1) and 2) are legacy scenarios, e.g. when SCell is active but the active BWP cannot cover target SSB, the MO needs to be measured within MG. while when the SCell becomes deactivated, it can be measured without gap with potential interruption. RAN4 already has requirement to cover the transition between measurement without gap and measurement with gap:

### 9.1.6 Minimum requirement at transitions When the measurement on one intra-frequency measurement object transitions from measurements performed outside gaps to measurements performed within gaps or vice versa during one measurement period, the cell identification and measurement period requirements with the longer delay apply. The carrier-specific scaling factor specified in clause 9.1.5 that applies to the other impacted measurement objects will also apply based on the longer measurement or cell identification delay before or after the transition. When the UE transitions between DRX and non-DRX or when DRX cycle periodicity changes, the cell identification and measurement period requirements apply based on the longer delay before or after the transition. Subsequent to this measurement period, the cell identification and measurement period requirements on each measurement object are corresponding to the second mode after transition.

Scenario 3) and 4) actually are legacy R17 scenarios. Note that this type of transition is not covered by current spec yet. Thus we’ve proposed a CR to introduce the missing requirements in R17.

Scenario 5) and 6) are supported in R18. There is no transition requirement to cover that either. Therefore, we propose to add the missing part in this meeting.

Observation 1: minimum requirements at transition between intra-frequency measurement with NCSG and intra-frequency measurement with gaps are still missing.

Proposal 1: introduce the missing requirements at transition between intra-frequency measurement with NCSG and intra-frequency measurement with gaps.

Regarding the processing delay upon transition, UE needs to refresh the measurement scheme for all the configured measurement objects. Because the CSSF for MO groups associated to MG and NCSGs are changed. Which carrier to measure within each NCSG and MG occasion needs to be updated by the UE. This is very similar to Pre-MG activation/deactivation, wherein 5ms processing delay is assumed. Here we propose to also consider 5ms processing delay upon SCell deactivation.

Proposal 2: similar to Pre-MG deactivation/activation, 5ms processing delay can be considered for measurement type transition between intra-frequency measurement with NCSG and intra-frequency measurement with Type 1/2 MG upon SCell deactivation.

We are open to discuss how to capture this 5ms.

As for the case that UE is configured with two NCSG without any association to the measurement on SCC, some company mentioned that this might also be a valid case, e.g. active BWP of the SCell can cover target SSB to measure thus no need to provide associated NCSG. Although this could be a valid case, we still encourage NW to provide association. Following agreement in previous meeting, RAN4 measurement delay requirement would not apply if the MO has to be measured within NCSG while there is no association. The agreement also said that UE is not expected to cause interruption outside the VIL due to measurement on any of the deactivated SCells. We believe this is enough. From UE point of view, UE has the flexibility to decide which NCSG to use for the measurement, e.g. use the one with smaller CSSF, or the one not being used for SpCell measurement, or use the one in the same CG and etc. This is up to UE, as long as UE won’t cause unexpected interruption.

# Conclusion

In this contribution, we provide discussion on core maintenance for pre-configured MGs, multiple concurrent MGs and NCSG. After discussion, the following conclusions are provided:

Observation 1: minimum requirements at transition between intra-frequency measurement with NCSG and intra-frequency measurement with gaps are still missing.

Proposal 1: introduce the missing requirements at transition between intra-frequency measurement with NCSG and intra-frequency measurement with gaps.

Proposal 2: similar to Pre-MG deactivation/activation, 5ms processing delay can be considered for measurement type transition between intra-frequency measurement with NCSG and intra-frequency measurement with Type 1/2 MG upon SCell deactivation.

# Reference

- R4-2410376, WF on NR_MG_enh2_part1, MediaTek inc.

- R4-2406439, WF on NR_MG_enh2_part1, MediaTek

- R4-2403542, WF on NR_MG_enh2_part1, MediaTek inc.

- R4-2402880, Big CR to TS 38.133 on Further enhancements on NR and MR-DC measurement gaps and measurements without gaps, MediaTek inc., Intel Corporation

- R4-2321640, WF for NR_MGE-2_part1, MediaTek inc.

- R4-2317305, WF on NR_MG_enh2_part1, MediaTek inc.

- R4-2314323, WF on NR MG enhancements (Part 1), MediaTek inc.

- R4-2310175, WF on NR MG enhancements, MediaTek inc.

- R4-2306330, WF on Rel-18 NR MG enhancements, MediaTek inc.

- R4-2220359, WF on NR_MG_enh2 Part 1, MediaTek inc.

- R4-2217251, WF on further enhancements on measurement gaps and measurements without gaps, MediaTek inc.

- R4-2202604, Further reply LS on R17 NR MG enhancements – Concurrent MG, MediaTek inc.

- RP-230755, New WID: Further Enhancements on NR and MR-DC Measurement Gaps and Measurements without Gaps, MediaTek Inc, Intel Corporation