3GPP TSG-RAN WG4 Meeting #112		R4-2411431

Maastricht, Netherlands, 19 – 23, 2024

Agenda Item:	5.16

Source: 	Apple

Title:	Discussion of R18 BWP wor maintenance

Document for:	Discussion

# Introduction

R18 BWP without restriction was completed in previous meetings. However, we found that some core requirements are still missing. In this contribution, we elaborate the issue and propose to introduce the missing requirements.

# Discussion

In option C, NCD-SSB based RLM/BFD/BM and RRM measurement is supported if the target NCD-SSB is within UE active BWP. Besides, transition between NCD-SSB and CD-SSB is also supported. correspondingly, RAN4 introduced minimum requirements at transition between NCD-SSB and CD-SSB for RLM and BFD:

### 8.1.4 Minimum requirement at transitions When the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each RLM-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each RLM-RS resource. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. When the UE transitions from a first configuration of RLM resources to a second configuration of RLM resources that is different from the first configuration, for each RLM resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each RLM resource present in the second configuration. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. When the UE transitions from a first configuration of active TCI state of the CORESET to a second configuration of active TCI state of the CORESET, for each CSI-RS for RLM present in the second configuration, the UE shall use an evaluation period corresponding to the second configuration from the time of transition. This requirement shall be applied to both out-of-sync evaluation and in-sync evaluation of the monitored cell. For UE supporting ncd-SSB-BWP-Wor-r18, when the UE transitions between RLM CD-SSB resource and RLM NCD-SSB resource due to BWP switching or due to SCG activation or deactivation during one evaluation period, the UE shall use an evaluation period that is the maximum of the evaluation periods corresponding to the first SSB type and the second SSB type after the BWP switching or the SCG activation or deactivation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each RLM-RS resource. … ### 8.5.10 Minimum requirement at transitions for beam failure detection When the UE transitions between DRX and no DRX or when DRX cycle periodicity changes, for each BFD-RS resource, for a duration of time equal to the evaluation period corresponding to the second mode after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation period corresponding to the first mode and the second mode. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second mode for each BFD-RS resource. When the UE transitions from a first configuration of BFD resources to a second configuration of BFD resources that is different from the first configuration, for each BFD resource present in the second configuration, for a duration of time equal to the evaluation period corresponding to the second configuration after the transition occurs, the UE shall use an evaluation period that is no less than the minimum of evaluation periods corresponding to the first configuration and the second configuration. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second configuration for each BFD resource present in the second configuration. When the UE transitions from a first configuration of active TCI state of the CORESET to a second configuration of active TCI state of the CORESET, for each CSI-RS for BFD present in the second configuration, the UE shall use an evaluation period corresponding to the second configuration from the time of transition. For UE supporting ncd-SSB-BWP-Wor-r18, when the UE transitions between BFD CD-SSB resource and BFD NCD-SSB resource due to BWP switching or due to SCG activation or deactivation during one evaluation period, the UE shall use an evaluation period that is the maximum of the evaluation period corresponding to the first SSB type and the second SSB type after the BWP switching or the SCG activation or deactivation. Subsequent to this duration, the UE shall use an evaluation period corresponding to the second SSB type for each BFD-RS resource.

However, corresponding requirements are still missing in L1 and L3 measurement requirements. therefore, we propose to introduce corresponding requirements.

Observation 1: transition between NCD-SSB and CD-SSB for RLM/BFD/L1-RSRP/L1-SINR and RRM measurement is supported.

Observation 2: minimum requirements at transition between NCD-SSB and CD-SSB have been defined for RLM and BFD, but not for L1-RSRP, L1-SINR and RRM measurement.

Proposal 1: introduce minimum requirements at transition between NCD-SSB and CD-SSB for L1-RSRP, L1-SINR and RRM measurement.

We also prepare a companion CR [1] to introduce the corresponding requirements.

# Conclusion

In this contribution, we provide discussion on core maintenance for BWP without restriction. After discussion, the following conclusions are provided:

Observation 1: transition between NCD-SSB and CD-SSB for RLM/BFD/L1-RSRP/L1-SINR and RRM measurement is supported.

Observation 2: minimum requirements at transition between NCD-SSB and CD-SSB have been defined for RLM and BFD, but not for L1-RSRP, L1-SINR and RRM measurement.

Proposal 1: introduce minimum requirements at transition between NCD-SSB and CD-SSB for L1-RSRP, L1-SINR and RRM measurement.

# Reference

- R4-2411432, CR for BWP wor maintenance, Apple.