3GPP TSG-RAN WG4 Meeting #112		R4-2411433

Maastricht, Netherlands, 19 – 23, 2024

Agenda Item:	5.24.1

Source: 	Apple

Title:	Discussion of R18 LTM core requirements maintenance

Document for:	Discussion

# Introduction

Core requirements maintenance for LTM have been discussed widely in the previous RAN4 meeting. The latest agreement can be found in the approved WF [1]. There are still some remaining issues that need to be addressed.

In this contribution, we provide further discussion on the open issues.

# Discussion

The first issue we would like to discuss is about the leftover due to decoupling L1-RSRP measurement from LTM cell switch. In the previous RAN4 meeting, it was agreed to decouple these two procedures in FR1 in general. However, some details are still open.

Issue 3-3-1: known TCI state conditions

Online Agreement

< Agreement >

- For FR1, the cell switch delay requirement is applicable for the case without L1 measurement and report, under certain conditions.

  - Further discuss the how the requirement applies together with the condition.

< Agreement>:

- In FR2: the RAN4 Rel-18 related LTM requirements are not applicable if L1 measurement of LTM candidate cell is not supported or configured.

<Way Forward >: Further discuss the following options in FR1:

  - Option 1: change known TCI state conditions to make cell switch delay requirements applicable to the case without L1 measurement and report

  - Option 2: Allow cell switch delay requirements applicable to unknown TCI state case with conditions.

  - Regarding the conditions, further discuss the following options when SNR of the TCI state≥ -3dB

      - Condition 1: TCI state has been activated and

        - Condition 1A: the TCI state activation command was received not more than 1280 ms before the cell switch command

        - Condition 1B: Beam-level L3 measurement results have been reported in 1280 ms

        - Condition 1C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command

      - Condition 2: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command

<Way Forward >: Further discuss the following options in FR2:

    - Condition 3: TCI state has been activated and

      - Condition 3A: the TCI state activation command was received not more than 1280 ms before the cell switch command

      - Condition 3B: the TCI state activation command was received not more than 160 ms before the cell switch command

      - Condition 3C: The RS associated to the target TCI state is available at least every 1280 ms after TCI state activation command

      - Condition 3D:

        - Beam-level L3 measurement results have been reported in 1280ms

        - L1-RSRP measurement period is no larger than 1280ms in FR2

    - Condition 4: UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in [1280]ms before the cell switch command

For the following two options for FR1:

  - Option 1: change known TCI state conditions to make cell switch delay requirements applicable to the case without L1 measurement and report

  - Option 2: Allow cell switch delay requirements applicable to unknown TCI state case with conditions.

Option 1 is NOT supported, since known TCI state has been widely used in RRM spec. Although exact wordings are slightly different in different sections, they are all based on L1-RSRP rather than L3-RSRP. If we change the conditions here, it will cause unnecessary misalignments.

Option 2 is acceptable to us. Regarding the conditions, condition 1 can apply to UE with early TCI activation configured/supported. Condition 2 can apply to UE without early TCI activation configured/supported.

Proposal 1: to decouple L1-RSRP with LTM for target cell in FR1, allow cell switch delay requirements applicable to unknown TCI state case with the following conditions:

- TCI state has been activated and the TCI state activation was completed not more than 1280 ms before the cell switch command, or

- UE has reported L3-RSRP measurements for the SSB associated to the target TCI state in 1280 ms before the cell switch command.

Issue 1-3-2: The definition of Tfirst-SSB in early candidate cell’s TCI state activation delay for inter-frequency with gap for known TCI state case

Ad hoc agreement

< Agreement>:

  - In early candidate cell’s TCI state activation delay for known TCI state case: Tfirst-SSB is the time to the first SSB occasion overlapped with MGL after slot n + THARQ +3Nslotsubframe,µ + [2ms] if the SSB needs to be measured with MG.

    - FFS: this is only applicable to UE supporting inter-frequency L1 measurement with MG.

  - Same agreement applies to the definition of Tfirst-SSB in PDCCH order RACH delay requirement.

To decouple L1-RSRP with LTM procedure, it is also beneficial to remove the FFS part. Then for UE without L1 RSRP configured/supported, as long as MG is configured, TCI activation delay and PDCCH ordered RACH delay requirements can apply.

Proposal 2: remove the FFS regarding support of inter-frequency L1 measurement with MG in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements.

Issue 3-1-2-1: Extra time for PL-RS measurement

Ad hoc agreement

< Agreement>:

  - No additional PL-RS measurement time is needed, provided

    - Option 1: L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported

    - Option 2: Target PL-RS is maintained.

Another issue we would like to discuss is regarding extra time for PL-RS measurement. Option 1 in the WF is acceptable to us. If the SSB associated with PL-RS has been measured/reported recently, UE can try to rely on the previous measurement result.

Proposal 3: No additional PL-RS measurement time is needed, provided L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported.

One last thing we would like to raise is about the SFN assumption. The following applicability rule is defined in current spec:

The requirements for inter-frequency cell switch to an FR2 target cell in this clause are only applicable, when - network has configured UE to perform L3 measurement with SSB index or L1 measurement for the target cell before the cell switch command, or - the SFN of the serving cell from which cell switch command is received and the SFN of the target cell are the same.

As can be seen, only FR2 is covered by this applicability. Regarding FR1, there is some guidance in RAN1 spec of RACH resource:

For handover purposes to a target cell in paired or unpaired spectrum where the target cell uses Lmax=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if the association pattern period in clause 8.1 of [5, TS 38.213] is not equal to 10 ms.

For inter frequency handover purposes where the source cell is either in paired or unpaired spectrum and the target cell is in unpaired spectrum and uses Lmax=8, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 76800Ts.

Note that a very soft tone is used in RAN1 spec “UE may assume” and it is only specified in RACH resource section (TS38.211 clause 6.3.3). We are not sure if NW would strictly follow this especially for RACH-less scenario. Therefore, it is proposed to include FR1 in the applicability as well. Regarding how to get the exact SFN, we can rely on network signaling (e.g. deriveSSB-IndexFromCellInter). If no indication from NW, although we can force UE to try the first transmission assuming same SFN, UE may fail e.g. to receive RAR since it transmitted preamble in a wrong frame. Existing RRM requirement (UE shall transmit RACH no later than the delay specified in our spec) does not apply. UE may ramp up the RACH power or take additional time to read SFN to transmit RACH again in the right slot. All of these are purely up to UE. A simple way is to include FR1 as well in the applicability of inter-frequency cell switch delay requirements.

Proposal 4: in RRM requirements applicability of LTM cell switch delay and PDCCH order RACH delay, clarify that SFN between serving and target cell should be the same even for inter-frequency target cell in FR1. Otherwise, additional time for UE to perform SFN acquisition is expected as for minimum requirements.

# Conclusion

In this contribution, we provide further discussion on core part maintenance of LTM. After discussion, the following conclusions are provided:

Proposal 1: to decouple L1-RSRP with LTM for target cell in FR1, allow cell switch delay requirements applicable to unknown TCI state case with the following conditions:

Proposal 2: remove the FFS regarding support of inter-frequency L1 measurement with MG in definition of TSSB in PDCCH ordered RACH and early TCI activation requirements.

Proposal 3: No additional PL-RS measurement time is needed, provided L3-RSRP or L1-RSRP on the SSB associated with PL-RS has been measured/reported.

Proposal 4: in RRM requirements applicability of LTM cell switch delay and PDCCH order RACH delay, clarify that SFN between serving and target cell should be the same even for inter-frequency target cell in FR1. Otherwise, additional time for UE to perform SFN acquisition is expected as for minimum requirements.

# Reference

- R4-2410302, WF on NR mobility enhancements (part 1), MediaTek Inc.