3GPP TSG-RAN WG4 Meeting #112		R4-2411438

Maastricht, Netherlands, 19 – 23, 2024

Agenda Item:	8.22.2

Source: 	Apple

Title:	Discussion on impact of inter-CU on LTM RRM requirements

Document for:	Discussion

# Introduction

In RAN#102 meeting, a new WI for R19 NR mobility enhancement phase 4 was agreed, which was revised to [1] in RAN#104 with the following objectives:

- Specify support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM) [RAN2, RAN3] - Prioritize the case when CU is acting as MN when DC is not configured - When DC is configured, inter-CU LTM can be configured either in MN or in SN but not both at the same time. For such cases: - As secondary priority, support the case where CU is acting as SN and MN is unchanged - As secondary priority, support the case where CU is acting as MN and SN is unchanged or SN is released - Specify support for subsequent LTM mobility procedures aiming to avoid RRC configuration between cell switches as per Rel-18 LTM - Coordination with SA3 needed with respect to security key handling - Note: Rel. 18 intra-CU LTM procedure is considered as baseline for adding inter-CU support - Measurements related enhancements for purpose of supporting LTM: [RAN2, RAN1] - Measurement related enhancements are applicable to Intra-CU MCG/SCG LTM and Inter-CU MCG/SCG LTM - Specify necessary components to support event triggered L1 measurement reporting [RAN2, RAN1] - RAN1 and RAN2 to progress independently on the event triggered measurements objectives of their respective MIMO and Mobility enhancement WIs. Review progress at RAN#105 to see if any modification of objectives is required to avoid/manage any overlap in the work - Specify support for CSI-RS measurements for LTM procedures and enable CSI-RS based beam management, and/or other necessary physical layer operations on candidate cells before LTM [RAN1] - Specify support of conditional LTM [RAN2, RAN3, RAN1] - Specify UE evaluated conditions for triggering LTM - Aim to support conditional LTM including subsequent LTM - Prioritise intra-CU LTM - Checkpoint to review objective at RAN#105. RAN WG work to not start before this checkpoint - Specify RRM requirements related to the above objectives as necessary [RAN4]

In this contribution, we analyse the RRM scope of the first objective, i.e. support for inter-CU Layer1/Layer 2 Triggered Mobility (LTM).

# Discussion

In R18 LTM objective, the following RRM requirements were introduced:

- PDCCH ordered Random Access for LTM

  - Section 6.2.2C for delay

  - Section 7.1 for timing

- LTM cell switch delay

  - Section 6.3 for PCell

  - Section 8.20 for PSCell

- Early TCI activation

  - Section 8.25

- L1 RSRP measurement

  - Section 9 for measurement period

    - 9.14 for intra-frequency and 9.15 for inter-frequency

  - Section 10 for accuracy

    - 10.1.19D for intra-frequency in FR1

    - 10.1.19E for inter-frequency in FR1

    - 10.1.20A for intra-frequency in FR2

    - 10.1.20B for inter-frequency in FR2

- Test cases

  - Section A.6 and A.7

Note that all above RRM requirements are defined irrspectively of whether the configruations are from intra-CU or inter-CU. Similar to legacy handover, RRM requirements are the same regardless of whether it is an intra-CU or inter-CU handover. Therefore, it is rational to assume that existing RRM requirements defined in R18 can also cover the inter-CU LTM as well.

Observation 1: R18 LTM related RRM requirements were defined irrspectively of whether the configruations are from intra-CU or inter-CU.

Proposal 1: existing R18 LTM related RRM requirements also apply to same procedures in inter-CU scenario.

Thus, there is no need to introduce any new requirements specifically to support inter-CU. However, if RAN1/2 eventually introduce new procedure in this objective, RAN4 can of course further discuss whether and how to define corresponding RRM requirements. Informatively, RAN2 progress of this WI was duplicated below:

RAN2#125bis 1. Inter-CU LTM Agreements on scenarios: 1. RAN2 first focus on inter-CU LTM in NR standalone scenario and use it as baseline for supporting inter-CU LTM in NR-DC scenarios. 2. Rel-19 inter-CU LTM also supports mixture of subsequent inter-CU LTM and subsequent intra-CU LTM after an inter-CU or intra-CU LTM switch. 3. UE can be configured with a mixture of intra-CU and inter-CU candidate LTM cells and irrespective of how the UE is configured with this mixture, UE measurement and reporting procedures will be the same for both intra-CU and inter-CU candidate LTM cells. Agreements on latency analysis: 1. Mobility latency analysis of rel-18 intra-CU LTM is reused for Rel-19 inter-CU LTM. Agreements on early sync phase: 1. Early DL and UL sync is also supported for inter-CU LTM. Inform RAN3 of this. Early DL sync using CSI-RS should be considered, pending RAN1 approval. 2. PDCCH ordered early RACH is supported for inter-CU LTM. 3. For early TA acquisition, Rel-18 option is baseline. FFS for RAR based option. Agreements on LTM cell switch execution phase: 1. Upon inter-CU LTM execution, UE performs - MAC reset - RLC re-establishment - PDCP re-establishment - Security key update FFS if there is an inter-CU LTM w/o security key change. RAN2#126 1.Inter-CU LTM Clarification of DC 1. An LTM configuration with inter-CU LTM candidate cells can be configured either by the MCG or SCG (but not for both simultaneously) and it is up to the network to handle this (further details up to RAN3, if any). No restriction for intra-CU LTM candidate cells. Xn-based and N2-based inter-CU LTM 2. Xn-based inter-CU LTM is prioritized in Rel-19. Stage-2 signaling flows and procedures 3. The preparation of inter-CU LTM configuration is initiated by the source gNB-CU. 4. For each candidate cell, the preparation of lower layer configuration is initiated by the candidate gNB-CU, based on the LTM request from the source gNB-CU. RAN2 assumes the interaction between the candidate gNB-CU and candidate gNB-DU follows the same signaling procedure for intra-CU LTM. 5. The source gNB-CU is responsible to collect the configurations and information of candidate cells from multiple candidate gNB-CUs and generates the common CSI resource configuration for L1 measurement on candidate cells. 6. In order to support subsequent LTM, the source gNB-CU needs to inform the candidate gNB-CU(s) about the common CSI resource configuration and the collected information of candidate cells from multiple candidate gNB-CUs. The candidate gNB-CU(s) responds with the candidate configuration to the source gNB-CU accordingly (if needed). Preparation: RRC Configuration/structure 7. The RRC signalling structure and modelling for Rel-18 LTM is taken as the baseline for inter-CU LTM. LTM Candidate ID 8. For inter-CU LTM, LTM candidate ID is unique across all the participating gNB-CUs. Max number of LTM candidate IDs 9. The maximum number of LTM candidate cell configuration is 8, regardless of whether these are intra-CU or inter-CU LTM candidate configurations. Early sync: RAR based option 10. RAR-based TA acquisition is not supported for inter-CU LTM for non-conditional LTM. FFS on conditional LTM. Execution: LTM Cell Switch Command 11. R18 LTM CSC MAC CE is baseline to trigger LTM cell switch for inter-CU LTM. LTM Cell switch completion 12. Support CG-based RACH-less and DG-based RACH-less procedures for inter-CU LTM. 13. The LTM completion defined for intra-CU LTM is followed for R19 LTM.

As can be seen, no any new procedure which may have RAN4 impact was agreed yet.

Proposal 2: no RRM impact is foreseen to support inter-CU LTM. RAN4 can revisit this if any new procedure is introduced by RAN1/2.

# Conclusion

In this contribution, initial discussion on scope of inter-CU LTM is provided. After discussion, the following conclusions are made:

Observation 1: R18 LTM related RRM requirements were defined irrspectively of whether the configruations are from intra-CU or inter-CU.

Proposal 1: existing R18 LTM related RRM requirements also apply to same procedures in inter-CU scenario.

Proposal 2: no RRM impact is foreseen to support inter-CU LTM. RAN4 can revisit this if any new procedure is introduced by RAN1/2.

# Reference

- RP-241515, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom