3GPP TSG-RAN WG4 Meeting #112		R4-2411440

Maastricht, Netherlands, 19 – 23, 2024

Agenda Item:	8.24

Source: 	Apple

Title:	Discussion of RRM impact of XR for NR Phase 3

Document for:	Discussion

# Introduction

XR for NR Phase 3 is supported in R19. The latest WID can be found in [1] with the following objectives:

The Rel-19 XR Phase 3 objectives are as follows: - Study and if justified, specify aspects related to multi-modality (intra-UE) (with coordination with SA2/SA4 as needed by LS request). Aim to facilitate efficient and effective support for XR application with Multiple QoS flows with multi-modal inter-dependencies, meeting multi-modal QoS requirements, e.g. synchronization and/or coordination. Efficiency enhancements are expected to be visible in terms of capacity or power consumption. [RAN2]. NOTE: Check in RAN#105 (check also other WG involvement if needed). - Specify enhancements to enable transmission/reception in gaps/restrictions that are caused by RRM measurements (from inter-frequency RRM measurement gaps, or intra-frequency measurements, or other scheduling restrictions etc). [RAN1, RAN2, RAN4] - Specify the corresponding measurement gap and scheduling restriction to enable the identified enhancements with RRM performance impact taken into consideration, work being triggered by LS. [RAN4] - Specify Enhancements for Scheduling, as follows: - For the UL, Study and if justified, Specify enhancements using delay/deadline information, for support of UL scheduling to enable high XR capacity while meeting delay requirements/avoiding too late PDUs. [RAN2]. NOTE: LCP implementation complexity need to be taken into account when evaluating solutions. NOTE: Check in RAN#105 - Specify the following user plane enhancements [RAN2] - RLC re-transmission related enhancements for operation of RLC Acknowledged Mode (AM) with small packet delay budget. - Specify Core requirements related to the above objectives as necessary [RAN4] - Extend Release 18 standalone mechanism to support NR-NR dual connectivity as follows [RAN3] - PDU set based handling - ECN marking - Burst Arrival Time reporting, if needed - PSI Discard coordination, if needed - Note: No RAN2 impact from above items NOTE: Whether / to what extent network exposure / RAN awareness / e.g. RAN involved rate control, possibly additional info for DL scheduling, parallel with SA2 work, shall be covered in this WI is TBD.

RAN4 related scope is highlighted in yellow above. In this contribution, initial discussion on RRM for XR for NR phase 3 is provided.

# Discussion

In May meeting, RAN1 approved an LS to RAN4 regarding UE assistance information [2]:

# 1 Overall description RAN1 discussed UE assistance information related to measurements occasions and related to channel conditions, traffic, UE mobility. The following agreement and conclusions were agreed: Agreement RAN1 continues to discuss and decide whether or not to introduce new UE assistance information for solution(s) to enable Tx/Rx in gaps/restrictions that are caused by RRM measurements. At least the following UE assistance information is considered for further study: - FFS: UE assistance information related to measurement occasions: - FFS: The number of needed measurement gaps/SMTC with restrictions within a time period; - FFS: The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period; - FFS: The number of required SSBs within a time period; - FFS: The number of consecutive RRM measurements that can be skipped; - FFS: The maximum interval between two consecutively reserved gap/restriction occasions for RRM measurements; - FFS: The patterns of gap(s)/restriction(s) where skipping is feasible or acceptable; - FFS: UE assistance information related to channel conditions: - FFS: RSRP is below/above search threshold (s-MeasureConfig); - FFS: UE assistance information related to traffic: - FFS: PSI (PDU set importance); - FFS: UE assistance information related to UE mobility: - FFS: L3 parameters related to mobility, e.g., static or not Companies are encouraged to provide additional details (e.g. how often the UE assistance info is provided, timing, applicable scenarios, performance gains, etc) on their preferred scheme. Note: From specification point of view, there is no mandated gNB behavior in response to any of the UE assistance information. RAN1 to make decision, from RAN1 perspective, in RAN1#117 on the support of UE assistance information. Conclusion RAN1 does not further discuss new UE assistance information related to channel conditions, traffic, UE mobility. Conclusion There is no consensus in RAN1 to support UE assistance information related to measurements occasions. The reason for this situation is lack of consensus on the need/feasibility for UAI and lack of technical understanding on issues outside of RAN1 expertise (e.g. impact of RRM measurement performance). It is up to other working groups to trigger further work in RAN1 on UE assistance information.. Particularly, it was discussed whether impact on RRM performance from skipping measurement occasions may be reduced if UE sends additional information: - Information about the maximum number of MGs/SMTC with restrictions that can be skipped within a time period. - Information about the patterns of gap(s)/restriction(s) where skipping is feasible or acceptable. # 2 Actions To RAN4 ACTION: RAN1 kindly asks RAN4 to consider the above information into account and decide whether or not to introduce any UE assistance information related to measurement occasions.

This is an action to RAN4: decide whether or not to introduce any UE assistance information related to measurement occasions. Some of the above UE assistance information seem a bit complicated from UE design point of view (except for channel conditions, e.g. RSRP is below/above search threshold (s-MeasureConfig). However we believe this is already supported in legacy event triggered reporting). Regarding number of occasions that can be skipped in a given time window, on one hand number of samples needed to get accurate results highly depends on channel condition. Especially for L3 measurement, different neighbor cells may have different channel conditions due to different distacne to the UE. On the other hand, since SMTC pattern on different carriers could be different, it is challenging for UE to simply say how many occasions can be skipped (number of carriers to measure within each SMTC/MG could be different). In other words, even if UE tells NW that 20% of SMTC/MG can be skipped, it doesn’t mean NW can skip any 20% of the SMTC/MG during the given time window. Some carriers can only be measured within certain gap occasions. Measurement opportunities sharing between L1 and L3 also needs to be considred. UE and NW may need to align exactly on which MG/SMTC occasions there will be data scheduled.

Observation 1: it is challenging for UE to provide the following assistance information in a static manner.  Besides, they are not enough for NW to determine which measurement occasion(s) can be skipped for data Rx/Tx.

- The number of needed measurement gaps/SMTC with restrictions within a time period;

- The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period;

- The number of required SSBs within a time period;

However, we are open to further discussion if any good solution is identified. Anyway, RAN1 already confirmed that they will not continue discussion on this issue. This can be done in RAN4.

Proposal 1: Reply RAN1 that RAN4 hasn’t identified an efficient way to let UE provide assistance information in a static manner.

In legacy, data is always depriorized duruing gap and scheduling restriction. One of the main goals is to find out solution to prioritize data over measurement during gap and scheduling restriction occasions. Solutions are being discussed in RAN1, e.g. there are two alternatives on the table:

- Alt. 1: Dynamic indication to enable Tx/Rx in particular gap(s)/restriction(s) that are caused by RRM measurements.

  - FFS: Alt 1-1: Explicit indication by DCI to skip a particular gap(s)/restriction(s);

    - Indication is included as part of scheduling DCI:

      - FFS: Bit-field size is one bit;

      - FFS: Bit-field size is >1 bit;

    - Note: Minimum time offset(s) between the end of [the first] received dynamic indication and start of corresponding gap(s)/restriction(s) occasion that is going to be skipped shall be introduced.

  - FFS: Alt 1-2: Explicit indication by DCI to indicate a time window where to skip a particular gap(s)/restriction(s);

    - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion in time window that is going to be skipped shall be introduced.

  - FFS: Alt 1-3: Implicit indication by DCI scheduling a transmission/reception overlapping with a gap(s)/restriction(s) to skip the gap(s)/restriction(s);

    - Note: Minimum time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped shall be introduced.

  - FFS: DCI format, DCI content, DCI bit-field size;

  - FFS: Whether indication is for one or more occasions;

  - FFS: How to consider time offset between the end of received dynamic indication and start of gap(s)/restriction(s) occasion that is going to be skipped.

- Alt. 3: Semi-static solution to enable TX/RX in gaps/restrictions that are caused by RRM measurements.

  - FFS: Alt 3-1: Configure a pattern(s) via RRC to indicate occasions where to skip gaps/restrictions;

    - FFS: Details of pattern:

      - FFS: Pattern is based on periodicity, offset and duration;

      - FFS: Pattern is based on a bitmap;

    - FFS: whether a pattern is applied to all or subset of configured MG configurations/scheduling restrictions.

  - FFS: Alt 3-3: Gaps/restrictions that are caused by RRM measurements are skipped if collided with particular semi-statically pre-configured Tx/Rx occasions.

  - FFS: Alt. 3-4: Gaps/restrictions that are caused by RRM measurements are skipped based on semi-statically configured priority information for particular semi-statically pre-configured Tx/Rx and/or particular gaps/restrictions.

It can be observed that solutions being discussed in RAN1 only focus on RRM measurement. However, not only RRM measurement could cause scheduling restriction, but also L1 operation, including RLM, BFD, CBD and L1 measurement.

Observation 2: solutions being discussed in RAN1 to enable Tx/Rx in particular gap(s)/restriction(s) only focus on RRM measurement. However, according to RAN4 design not only RRM measurement could cause scheduling restriction, but also L1 operation, including RLM, BFD, CBD and L1 measurement.

If some measurement occasions (with gap or scheduling restriction) are dropped due to XR traffic, UE may not be able to meet legacy accuracy requirement within the same measurement period from minimum requirement point of view. Therefore, some discussion is expected to address this issue. For instance, whether and how to define conditions under which there is no impact on measurement accuracy, if there is impact then how to handle it from RAN4 spec point of view (e.g. extend measurement delay).

Observation 3: enabling Tx/Rx in gap/restriction may have negative impact on measurement performance. Some study is expected in RAN4 to address this issue.

Besides, some design in above candidate options is highly related to RAN4. For instance, in Alt.3-1 a pattern to skip gap/restriction is mentioned. From RRM point of view, this pattern can be considered as counter measurement pattern, which may have significant impact on measurement behaviour and performance, depending on detailed design and configuration (e.g. how many percentage of gap will be dropped). It seems RAN4 is also a right place to discuss this pattern. We had simlar discussion in legacy gap related work items on how to handle the different types of collisions, which can be extended to cover this work item without significant extra effort. For instance, how to handle fully overlapping, partially overlaping, fully partial overlapping and partially partial overlapping. Whether and how to define proximity rule among gap patterns, SMTC pattern, and the new counter measurement pattern. How to guarantee the minimum measurement performance and so on.

Observation 4: some of the solutions being discussed in RAN1 is also highly related to RAN4.

Proposal 2: RAN4 shall proactively participate in solution design to enable Tx/Rx in gap/restriction. At least the following aspects could be led by RAN4:

- Tx/Rx in occasions of L1 operation including RLM, BFD, CBD and L1 measurement.

- Impact on measurement performance due to measurement cancellation and corresponding solution to address the impact.

- Possible UE assistance information.

- Possible pattern for measurement cancellation.

# Conclusion

In this contribution, initial discussion on RRM for XR for NR phase 3 is provided. After discussion, the following conclusions are made:

Observation 1: it is challenging for UE to provide the following assistance information in a static manner.  Besides, they are not enough for NW to determine which measurement occasion(s) can be skipped for data Rx/Tx.

- The number of needed measurement gaps/SMTC with restrictions within a time period;

- The maximum number or ratio of MGs/SMTC with restrictions that can be skipped within a time period;

- The number of required SSBs within a time period;

Proposal 1: Reply RAN1 that RAN4 hasn’t identified an efficient way to let UE provide assistance information in a static manner.

Observation 2: solutions being discussed in RAN1 to enable Tx/Rx in particular gap(s)/restriction(s) only focus on RRM measurement. However, according to RAN4 design not only RRM measurement could cause scheduling restriction, but also L1 operation, including RLM, BFD, CBD and L1 measurement.

Observation 3: enabling Tx/Rx in gap/restriction may have negative impact on measurement performance. Some study is expected in RAN4 to address this issue.

Observation 4: some of the solutions being discussed in RAN1 is also highly related to RAN4.

Proposal 2: RAN4 shall proactively participate in solution design to enable Tx/Rx in gap/restriction. At least the following aspects could be led by RAN4:

- Tx/Rx in occasions of L1 operation including RLM, BFD, CBD and L1 measurement.

- Impact on measurement performance due to measurement cancellation and corresponding solution to address the impact.

- Possible UE assistance information.

- Possible pattern for measurement cancellation.

# Reference

- RP-240791, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia

- R1-2405736, LS on UE assistance information, Nokia