3GPP TSG-RAN WG4 Meeting #112					          	R4-2411441

Maastricht, Netherlands, 19 – 23, 2024

Title:	Reply LS to RAN1 on UE assistance information

Response to:	R1-2405736

Release:	Release - 18

Work Item:	NR_XR_Ph3-Core

Source:	RAN4

To:	RAN1

Cc:	RAN2

Contact Person:

#### Name:	Qiming Li

E-mail Address:	< li_qiming@apple.com>

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	none

1. Overall Description:

RAN4 thanks RAN1 for the LS on UE assistance information. In RAN4#112, UE assistance information was extensively discussed. So far RAN4 hasn’t identified an efficient way to let UE provide assistance information in a static manner. Because 1) number of samples needed to get accurate results highly depends on channel condition, while channel conditions for different neighbor cells could be different and may change from time to time. 2) some neighbor cell can only be measured within a certain occasions due to flexible configuration of gap patterns and SMTC on different carriers. Therefore, it is not enough to only let NW know how many number (or ratio) of measurement occasions can be skipped within a time period.

Besides, RAN4 would like to point out that not only RRM measurement could cause scheduling restriction, but also RLM, BFD, CBD and L1 measurement. Therefore, solutions to enable Tx/Rx in gap/restriction shall also cover RLM, BFD, CBD and L1 measurement.

2. Actions:

To RAN1 group.

ACTION: 	RAN4 kindly asks RAN1 to take above information into consideration in future design.

3. Date of Next TSG-RAN WG4 Meetings:

TSG RAN WG4 Meeting #112bis		              14 - 18 October 2024 			Heifei, China

TSG RAN WG4 Meeting #113		              18 - 22 November 2024 			Orlando, US