# 3GPP TSG-RAN WG4 Meeting #112	R4-2411442

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.14.1

Source:	Apple

Title:	On RRM core requirements maintenance for SCell activation enhancement

Document for:	Discussion

# Introduction

In last RAN4#111 meeting, the WF for SCell activation has been agreed in [1]. In this contribution, we discuss the newly identified issue for the FR1 SCell activation enhancement with L3 report and provide some potential solutions.

# Discussion

In Rel-18 RRM enhancement WI, the L3 reporting based SCell activation enhancement has been introduced, and the corresponding UE requirement is defined in TS38.133 section 8.3.17 as following, and the key enhancement is UE can report L3 measurement result after receiving the SCell activation command, and it can help to reduce the unknown SCell activation delay significantly.

However, even though the L3 report after SCell activation command can transition an unknown SCell activation procedure to a known SCell activation, the FR1 known SCell activation has the following requirements in TS38.133,

In FR1 SCell activation, although the SCell being activation is known, the T/F tracking or AGC refinement is still needed, depending on the measurement period of target SCell, i.e.,

- Needs T/F tracking of TfisrtSSB for target SCell when measurement period is <=2400ms

- Needs AGC refinement of TfirstSSB_MAX and T/F tracking of Trs for target SCell when measurement period is >2400ms

In our understanding, the Rel-18 SCell activation enhancement turns an unknown SCell to be like a known SCell for shortening the SCell activation delay, however, the T/F tracking or AGC refinement may still be needed depending on the measurement period of this FR1 target being-activated SCell.

Proposal 1: RAN4 to discuss if measurement period shall be considered as a condition to differentiate the requirements for FR1 SCell activation enhancement with L3 report.

The same condition of the measurement period can be used to decide if UE needs T/F tracking time or both T/F tracking time and AGC refinement for FR1 SCell activation with L3 report.

Proposal 2: Like the legacy FR1 known SCell activation, SCell activation delay requirement with L3 report shall be differentiated according to measurement period below or above 2400ms, and decide if AGC refinement or T/F tracking is needed.

After RAN4 has consensus on the above proposals, we can bring in CRs and detailed equation proposals in next meeting.

# Conclusion

In this contribution, we discuss the new issue for the FR1 SCell activation enhancement with L3 report and provide some potential solutions.

Proposal 1: RAN4 to discuss if measurement period shall be considered as a condition to differentiate the requirements for FR1 SCell activation enhancement with L3 report.

Proposal 2: Like the legacy FR1 known SCell activation, SCell activation delay requirement with L3 report shall be differentiated according to measurement period below or above 2400ms, and decide if AGC refinement or T/F tracking is needed.

# References

[1] R4-2410259, WF for [111][205] NR_RRM_enh3_part1, Apple, RAN4 #111