# 3GPP TSG-RAN WG4 Meeting #112	R4-2411443

# Maastricht, Netherlands, 19th – 23rd August, 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 4674 | rev |  | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_FR1_lessthan_5MHz_BW-Perf) test case of FR1 intra-frequency handover for less than 5MHz |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Apple |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_FR1_lessthan_5MHz_BW-Perf |  |  |  |  |  | Date: |  |  | 2024-07-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The PDSCH and CORESET configurations are missing in test case of FR1 intra-frequency handover to unknown target cell for less than 5MHz |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add PDSCH and CORESET configurations in test case of FR1 intra-frequency handover to unknown target cell for less than 5MHz |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PDSCH and CORESET configurations are missing in test case of FR1 intra-frequency handover to unknown target cell for less than 5MHz |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.3.1.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Start of Change 1

#### A.6.3.1.15	Intra-frequency handover from FR1 to FR1; unknown target cell operating with 12 PRB SSB bandwidth

##### A.6.3.1.15.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency handover requirements for unknown target cell operating with 12 PRB SSB bandwidth specified in clause 6.1.1.2.

##### A.6.3.1.15.2	Test Parameters

Supported test configurations are shown in table A.6.3.1.15.2-1. General test parameters as specified in A.6.3.1.2.2-2 with config 1 apply except those specified in A.6.3.1.15.2-2. Cell specific test parameters as specified in A.6.3.1.2.2-3 with config 1 apply except those specified in A.6.3.1.15.2-3.

The test procedure specified in A.6.3.1.2.2 applies to this test. The cell 2 is the unknown target cell operating with 12 PRB SSB bandwidth.

Table A.6.3.1.15.2-1: Intra-frequency handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 3 MHz bandwidth with 15PRB CBW, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 3 MHz bandwidth with 12PRB CBW, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 3 MHz bandwidth with 12PRB CBW, FDD duplex mode |
| Note1: The UE is only required to be tested in one of the supported test configurations Note2: Configuration 2 is only applied for the testing on band n100 |  |

Table A.6.3.1.15.2-2: General test parameters Intra-frequency handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 | unknown target cell operating with 12 PRB SSB bandwidth |
| Final condition | Active cell |  | Cell 2 | unknown target cell operating with 12 PRB SSB bandwidth |

Table A.6.3.1.15.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| BWchannel |  | Config 1 | MHz | 3: NRB,c = 15 |  |  |  |
|  |  | Config 2 |  | 3: NRB,c = 12 |  |  |  |
| BWP BW |  | Config 1 | MHz | 3: NRB,c = 15 |  |  |  |
|  |  | Config 2 |  | 3: NRB,c = 12 |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.2 FDD |  |  |  |
|  |  | Config 2 |  | SR.1.2 FDD |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.3 FDD |  |  |  |
|  |  | Config 2 |  | CR.1.2 FDD |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.13 FR1 |  |  |  |
| IoNote1 | Config 1 |  | dBm/ 2.7MHz | -66.81 | -64.11 | -66.81 | -64.11 |
|  | Config 2 |  | dBm/ 2.16MHz | -67.78 | -65.07 | -67.78 | -65.07 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| Note 1: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |

##### A.6.3.1.15.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than 132 ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90%.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = 122 ms in the test. Tinterrupt is defined in clause 6.1.1.2.2.

This gives a total of 132 ms.

End of Change 1