# 3GPP TSG-RAN WG4 Meeting #112	R4-2411444

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.29.1

Source:	Apple

Title:	On RRM core requirements maintenance for NES

Document for:	Discussion

# Introduction

In last RAN4#111 meeting, the WF for NES has been agreed in [1] and moderator has captured all open issues in the topic summary [2]. There are still some opens issues for further discussion in the maintenance stage, and in this contribution, we continue the discussion on remaining RRM core requirements for NES.

# Discussion

Issue 1-1-1: Power difference conditions

The options and remaining issues from last meeting are as followings:

Issue 1-1-1: Power difference conditions Agreement: Clarify that EPRE difference is based on the EPRE normalized by SCSs of SSB of reference Cell and A-TRS/P-TRS of SSB-less Cell. Only change [9] dB to [12] dB in this meeting. Further discuss whether and how to support larger EPRE difference in next meeting: - Option 1: Clarify that EPRE difference is smaller or equal to [12] dB + ¦20*log (f1/f2)¦ - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS. - Option 2: Further increase [12] dB and the value is FFS - Option 3: Update the requirements with one more P-TRS sample for larger EPRE difference - Other options are no precluded.

In existing SSB-less SCell activation for FR1 intra-band contiguous case, it was specified that,

If the SCell being activated belongs to FR1 and if there is at least one active serving cell contiguous to the SCell on that FR1 band, if the UE is not provided with SSB configuration (absoluteFrequencySSB) nor SMTC configuration for the target SCell, Tactivation_time is 3 ms for UE supporting scellWithoutSSB, provided - The RTD between the target SCell and the contiguous active serving cell is within within ±260ns, and - The difference of the reception power with the contiguous active serving cell is <= 6dB, and - The RS(s) of SCell being activated is (are) QCL-TypeA with TRS(s) of the SCell being activated, and the TRS(s) of the SCell being activated is (are) further QCL-TypeC with SSB(s) of any active serving cell that is contiguous to the SCell being activated on that FR1 band.

In the inter-band scenario, UE may support high Rx power difference, but such high Rx power difference is only supported when UE have sufficient SSB or TRS/A-TRS samples for AGC settling. However, in SSB-less inter-band CA scenario, SCell only has TRS/A-TRS for AGC fine tuning, and therefore RAN4 needs to carefully check the Rx power difference side condition for the requirement.

In our view, in SSB-less inter-band CA with co-located cells, the Rx power difference is mainly up to the following aspects:

- (a). Pathloss difference,

  - a-1: carrier frequency difference;

  - a-2: propagation path difference;

- (a). BW size difference;

- (a). Typical PSD difference at gNB Tx

After some offline checking with some infra vendors, we assume the same PSD can be assumed for this inter-band FR1 co-located CA case (btw. Reference cell and target SSB-less SCell). The BW size difference and carrier frequency difference between reference cell and target SSB-less SCell is also known to the UE, but we need to understand the typical value of these two aspects to decide the power difference side condition.

If we cannot decide on such typical values for the above aspects, we may only consider the UE implementation for AGC settling without any pre-information. Here, the AGC settling without any pre-information means the coarse AGC value from reference cell can have big difference from target SSB-less SCell and therefore such reference cannot help any more for UE to decide the AGC on the target SCell, and therefore what UE will do is exactly like an initial or blind AGC estimation. Thus, we think the compensation for AGC is kind of UE implementation on compensation for both pathloss difference and BW size difference, however, as we discussed in last meeting, it’s hard to clarify the details for such compensation due to different UE implementations. However, it would be also fine to further clarify the AGC-compensation for pathloss between inter-band CCs is based on a normalized equation (e.g., free space pathloss model) with implementation margins from different vendors.

We are fine to keep “EPRE after pre-compensation” in the spec text with some clarification on a normalized pathloss equation with implementation margins, e.g., the option 1, and we can also compromise to remove it without changing power difference value. Since last meeting the power difference has been increased from initial 9dB to 12dB already, if RAN4 cannot have consensus on the definition for “EPRE after pre-compensation”, we support to keep 12dB as power difference side condition and leave the “EPRE after pre-compensation” to UE implementation.

Proposal 1: For power difference conditions in SSB-less SCell activation requirement, we support either one of the following alternatives:

- Alt 1: RAN4 to keep “EPRE after pre-compensation” and “12dB EPRE difference” in the spec text, and no any further change is expected.

- Alt 2: Clarify that EPRE difference is smaller or equal to [12] dB + |20*log (f1/f2)| - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS.

Issue 1-1-3: Multiple SSB-less SCells activation

Agreements: - Also define requirements for following scenario: - When all to-be-activated SCells are SSBless on the same band and SCells are contiguous. - All to-be-activated SCells have same QCL source cell. - P-TRS is used. - Option 1: The multiple SCell activation delay requirements are based on TRS with the shortest periodicity. The detailed requirements can be discussed in the CR. - Option 2: UE activate each to-be-activated SCell based on the TRS on the SCell, and the requirements to be defined accordingly (i.e. single CC delay) - FFS A-TRS based requirements.

The previous discussion only focused on single being-activated SCell, that is, there is only one SSB-less SCell to be activated. It’s unclear what the UE behavior is, when all to-be-activated SCells are intra-band contiguous SSB-less SCells and all to-be-activated SCells have same QCL source cell.

So far, the activation procedure is independent when multiple SSB-less SCells are being activated even though the target being-activated SCells are intra-band contiguous CCs. Like shown in the example of following figure, the SSB-less SCell 1 and SSB-less SCell 2 activation are performed independently: SCell 1 needs two samples of TRS1 for AGC settling and T/F tracking, while SCell 2 needs another two samples of TRS2 for AGC settling and T/F tracking.

Figure 1. example for multiple SSB-less SCell activation with TRS

We know that the current multiple SCell activation requirement has one side condition that “any to-be-activated unknown SCell has active serving cell(s) or known to-be-activated SCell(s) on the same band”, however, such condition with respect to the searcher limitation may not be needed for SSB-less SCell activation case (UE does not need coarse timing acquisition or cell searching). In our view, if the being-activated SSB-less SCell#1 and the being-activated SSB-less SCell#2 are intra-band contiguous CCs, it would make sense to prioritize the SCell with the smallest TRS periodicity for activation, and then treat the other SCell(s) activation by reusing all the AGC and T/F information (no additional time is needed for activation), and the total delay for multiple SCell activation would still be: Tfirst_TRS_min + TTRS_min +[5]ms, i.e., the multiple SCell activation delay requirements are based on TRS with the shortest periodicity.

Proposal 2: for multiple SSB-less SCell activation with TRS, when all to-be-activated SCells are intra-band contiguous SSB-less SCells and all to-be-activated SCells have same QCL source cell, the multiple SCell activation delay requirements are based on TRS with the shortest periodicity, i.e., the total delay for multiple SCell activation would still be: Tfirst_TRS_min + TTRS_min +[5]ms. Tfirst_TRS_min, TTRS_min is the corresponding time component for the TRS with the shortest periodicity.

In last meeting companies mentioned the A-TRS case when considering the multiple SSB-less SCell activation. In current single SSB-less SCell activation, if UE supporting aperiodicCSI-RS-FastScellActivation-r17 and A-TRS is configured, SSB-less SCell activation needs 2 A-TRS occasions and each occasion consists of four CSI-RS resources in two consecutive slots. As shown in the figure 2, we think UE can use the first arrived A-TRS to complete the multiple intra-band contiguous SSB-less SCell activation, and it’s not necessary to wait the A-TRS for each individual to-be-activated SCell. Furthermore, if RAN4 defines to use the earlier arrived A-TRS, we don’t think network would need to configure A-TRS for all target to-be-activated SCells.

Figure 2. example for multiple SSB-less SCell activation with A-TRS

Proposal 3: for multiple SSB-less SCell activation with A-TRS, when all to-be-activated SCells are intra-band contiguous SSB-less SCells and all to-be-activated SCells have same QCL source cell, the multiple SCell activation delay requirements are based on A-TRS on the SCell which has the earliest arrived A-TRS after MAC CE decoding.

One more possible scenario is: A-TRS and TRS are configured for different to-be-activated intra-band contiguous SSB-less SCells, as shown in figure 3. In our view, A-TRS can be dynamically controlled by network and it shall achieve the fastest SCell activation speed. Thus, for multiple SSB-less SCell activation, when “all to-be-activated SCells are intra-band contiguous SSB-less SCells” and “all to-be-activated SCells have same QCL source cell” and “A-TRS and TRS are configured for different to-be-activated intra-band contiguous SSB-less SCells”, the multiple SCell activation delay requirements are based on A-TRS on the SCell which has the earliest arrived A-TRS after MAC CE decoding.

Figure 3. example for multiple SSB-less SCell activation with A-TRS and TRS (TRS on target SCell1 and A-TRS on SCell2)

Proposal 4: for multiple SSB-less SCell activation, when “all to-be-activated SCells are intra-band contiguous SSB-less SCells” and “all to-be-activated SCells have same QCL source cell” and “A-TRS and TRS are configured for different to-be-activated intra-band contiguous SSB-less SCells”, the multiple SCell activation delay requirements are based on A-TRS on the SCell which has the earliest arrived A-TRS after MAC CE decoding.

Issue 1-1-4: Neighbour cells on carrier of SSB-less SCell

The remaining issues from last meeting are as followings:

- Proposals - Option 1: if neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell (Apple, Huawei) - Option 2: For the case of the neighbour cell(s) is on the carrier of SSB-less SCell and the SSB from neighbour cell(s) can be contained in the active BWP of SSB-less SCell, the measurement for such neighbour cell(s) can be treated as intra-frequency measurement (CMCC) - Option 3: If the SSB of neighbour cell is fully contained by the active BWP of the SSB-less SCell, the SSB based neighbour cell measurment is defined as intra-frequency measurement, and no gap is needed. Otherwise, the SSB based neighbour cell measurement is defined as inter-frequency measurement and gap is needed. (ZTE) - Option 3: Do not discuss the neighbor cell measurement on the carrier of SSB-less SCell. (Nokia, QC, Huawei, Ericsson) - Recommended WF: - Companies are encouraged to discuss the issue considering legacy SSB-less and R18 inter-band SSB-less jointly in unified manner. And no further discussion in R18 NES.

When we assume SSB-less SCell operation, we never touch the neighbor cell measurement on the carrier of SSB-less SCell. The SSB-less SCell is because network would like to save the energy of this cell when the traffic load of this cell is low, but it does not necessarily mean all the other cells on this carrier are in the NES mode (without SSB transmission). If some of the neighbor cells on this carrier have SSB transmission and network configure such MO to UE, it’s unclear to us if it shall be treated as intra-frequency or inter-frequency. In R16 we have a feature for inter-frequency measurement without MG, and the condition to support that feature is also “the SSB is completely contained in the active BWP of the UE”, as following,

Figure 4. example of neighbor cell SSBs located in the active BWP of SSB-less SCell

Similarly, in NES case, if there are multiple SSBs from neighbor cells located in the active BWP of the SSB-less SCell as figure 4, these neighbor cell measurements cannot be treated as intra-frequency anyway. Thus, we think if neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell.

Since we have already completed the core part of this WI, if companies have concern to open new discussion in the maintenance feature, we are also fine to limit the scenario in R18 to that all neighbor cells on the CC of SSB-less SCell shall be SSB-less cells as well.

Proposal 5: if neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell.

Issue 1-1-5: Relation to R15 intra-band SSB-less

The remaining issues from last meeting are as followings:

Agreement: From RAN4 understanding, for UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less, - It is not expected that the to-be-activated SCell is configured with QCL source to both intra-band contiguous and inter-band Cells [when the Rel-18 reference cell indication is not configured]. - When R18 reference cell indication is configured, network configure with QCL source to both intra-band contiguous and inter-band Cells, there is no RAN4 agreement on whether Rel-15 or Rel-18 requirement is applied. - The above RAN4 understanding does not intend to trigger any further RAN2 discussion.

The remaining issue is: when R18 reference cell indication is configured, network configures with QCL source to both intra-band contiguous and inter-band serving cells, which requirement shall UE follow for SSB-less SCell activation. For a UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less, if R18 reference cell indication is configured and network configures with QCL source to both intra-band contiguous and inter-band serving cells, UE may either use Rel-15 implementation for the faster SCell activation, or UE may follow the network reference cell indication to do the inter-band SCell activation. The spec cannot limit the network implementation for such configuration, but at least no requirement shall be applied for this scenario, and we shall leave it to UE implementation.

Proposal 6: for UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less SCell operation, if the to-be-activated SSB-less SCell is configured with QCL source to both intra-band contiguous and inter-band Cells, no SCell activation requirement shall be applied, regardless of whether the Rel-18 reference cell indication is configured.

Issue 1-1-6: Intra-band non-contiguous CA

RAN4 has agreed side conditions for SSB-less SCell activation in inter-band FR1 CA, e.g., RTD and power imbalance, however, these side conditions might not be applicable to intra-band NCCA case.

For inter-band SSB-less SCell activation, RTD between the target SCell and the colocated reference serving cell is within CP where CP is corresponding to the SCS of target SSB-less SCell, and the main rational is that UE will typically use separated FFTs between reference cell and target SSB-less SCell for inter-band CA. However, for intra-band NCCA case, if typically, UE is using single FFT or single FFT timing, then UE will only support SCell activation/operation when RTD shall be <= CP of the max SCS between reference cell and target SSB-less intra-band NCCA SCell.

For inter-band SSB-less SCell activation, power imbalance condition for SSB-less SCell activation is specified as: The [EPRE] difference at UE side is smaller than or equal to [12] dB, where [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC]. If UE is using single RF chain between reference-cell and target SSB-less intra-band NCCA SCell, UE can support SCell activation/operation when power imbalance(EPRE difference) shall be <= 6dB between reference cell and target SSB-less intra-band NCCA SCell.

Since RAN4 is defining the minimum requirement based on the worst case, the side condition above shall consider the single FFT and single RF chain case for the minimum requirement design for intra-band NCCA case. Thus, we propose followings:

Proposal 7: for intra-band FR1 NCCA case, the side condition of RTD for SSB-less SCell activation shall be defined as:

- The RTD between the target SSB-less intra-band NCCA SCell and the collocated reference serving cell is within CP where CP is corresponding to the max SCS between reference cell and target SCell.

Proposal 8: for intra-band FR1 NCCA case, the side condition of power imbalance for SSB-less SCell activation shall be defined as:

- The [EPRE] difference at the UE is smaller than or equal to [6] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC].

And we are also fine that RAN4 will not consider SSB-less SCell operation for intra-band NCCA in R18.

# Conclusion

In this contribution, we continue the discussion on remaining RRM core requirements for NES.

Proposal 1: For power difference conditions in SSB-less SCell activation requirement, we support either one of the following alternatives:

- Alt 1: RAN4 to keep “EPRE after pre-compensation” and “12dB EPRE difference” in the spec text, and no any further change is expected.

- Alt 2: Clarify that EPRE difference is smaller or equal to [12] dB + |20*log (f1/f2)| - Margin, where f1 and f2 is the center frequency of reference Cell and SSB-less Cell and the value of Margin is FFS.

Proposal 2: for multiple SSB-less SCell activation with TRS, when all to-be-activated SCells are intra-band contiguous SSB-less SCells and all to-be-activated SCells have same QCL source cell, the multiple SCell activation delay requirements are based on TRS with the shortest periodicity, i.e., the total delay for multiple SCell activation would still be: Tfirst_TRS_min + TTRS_min +[5]ms. Tfirst_TRS_min, TTRS_min is the corresponding time component for the TRS with the shortest periodicity.

Proposal 3: for multiple SSB-less SCell activation with A-TRS, when all to-be-activated SCells are intra-band contiguous SSB-less SCells and all to-be-activated SCells have same QCL source cell, the multiple SCell activation delay requirements are based on A-TRS on the SCell which has the earliest arrived A-TRS after MAC CE decoding.

Proposal 4: for multiple SSB-less SCell activation, when “all to-be-activated SCells are intra-band contiguous SSB-less SCells” and “all to-be-activated SCells have same QCL source cell” and “A-TRS and TRS are configured for different to-be-activated intra-band contiguous SSB-less SCells”, the multiple SCell activation delay requirements are based on A-TRS on the SCell which has the earliest arrived A-TRS after MAC CE decoding.

Proposal 5: if neighbor cells on carrier of SSB-less SCell have SSB transmission, the measurement for those neighbor cells shall be treated as inter-frequency measurement without MG as long as the SSBs from those neighbor cells can be contained in the active BWP of SSB-less SCell.

Proposal 6: for UE supports both R18 inter-band SSB-less and R15 intra-band contiguous SSB-less SCell operation, if the to-be-activated SSB-less SCell is configured with QCL source to both intra-band contiguous and inter-band Cells, no SCell activation requirement shall be applied, regardless of whether the Rel-18 reference cell indication is configured.

Proposal 7: for intra-band FR1 NCCA case, the side condition of RTD for SSB-less SCell activation shall be defined as:

- The RTD between the target SSB-less intra-band NCCA SCell and the collocated reference serving cell is within CP where CP is corresponding to the max SCS between reference cell and target SCell.

Proposal 8: for intra-band FR1 NCCA case, the side condition of power imbalance for SSB-less SCell activation shall be defined as:

- The [EPRE] difference at the UE is smaller than or equal to [6] dB, where, [EPRE] difference is the power difference between TRS/A-TRS symbol on the SSB-less SCell and SSB symbol on the reference serving cell [after the compensation for AGC].

# References

[1] R4-2410269, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN4 #111

[2] R4-2408023, Topic summary for [111][226] Netw_Energy_NR, Moderator (Huawei), RAN4#111