# 3GPP TSG-RAN WG4 Meeting #112	R4-2411445

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.29.2

Source:	Apple

Title:	On RRM test cases for NES

Document for:	Discussion

# Introduction

In last RAN4#111 meeting, the WF for NES has been agreed in [1] and moderator has captured all open issues in the topic summary [2]. There are still some opens issues for further discussion in the testing stage, and in this contribution, we continue the discussion on testing setup for NES.

# Discussion

Issue 2-1-1: Test configurations for SSB-less

The options and remaining issues from last meeting are as followings:

Issue 2-1-1: Test configurations for SSB-less - EPRE - Which Cell has higher EPRE: - Option 1: Pcell with higher EPRE than the SSB-less Scell (MTK) - Option 2: configure higher transmit power for TRS of the SSBless Scell than SSB transmit power of the reference cell (QC, vivo) - Configuration of EPRE: - Option 1: RAN4 to set TE transmission power difference as [9 dB] + [ΔP] in the test cases for reference cell and SSB-less SCell, and ΔP is decided only the CC BW difference between reference cell and target SCell. ΔP = 6dB with the following setups (Apple) - reference cell is set as 10MHz CC BW with 15kHz SCS - SSB-less SCell is set as 40MHz CC BW with 15kHz SCS - Option 2: Set EPRE difference = 9 dB + ΔEPRE, where ΔEPRE is equal to 20*log(f1/f2), and f1 and f2 are the frequency radio channel 1 and radio channel 2 (CMCC, CTC, ZTE, Ericsson) - Option 3: Do not introduce pathloss margin [ΔEPRE]. (QC, vivo) - Recommended WF: - Discuss this issue with core maintenance issue 1-1-1 in this meeting.

Since the pathloss compensation for AGC is up to UE implementation, it’s hard to set up an exact value in the test for different carrier frequencies; and also this value would be a variable based on the different band combinations. We prefer to not consider the pathloss difference inside ΔPL design. However, the BW size difference between CCs can be deterministic for compensation. In the existing test cases, we found that both 10MHz and 40MHz are used for different configurations, and these BW configurations can also be used for inter-band FR1 SSB-less SCell activation testing.

The EPRE difference between 10MHz and 40MHz shall also consider the SCS difference. Even though the existing test case for legacy SCell activation considered 15kHz SCS for 10MHz and 30kHz for 40MHz, but we don’t think the practical scenario for SSB-less SCell will have difference SCS between reference cell and target SCell. As in the endorsed CR last meeting (R4-2410276), in current section A.4.5.3.11.1, the configuration table is as followings. The same BW and SCS has been configured same values between reference cell and target SCell.

No matter which proposal is adopted in the core requirement, the AGC compensation is anyway dependent on the UE implementation, and therefore we really don’t have strong need to introduce a certain margin to test this implementation part. We support the option 3 in last meeting as:

- Option 3: Do not introduce pathloss margin [ΔEPRE] for inter-band SSB-less SCell activation test cases.

Whether PCell EPRE is higher or lower than SCell is not an issue to verify the UE behavior in the test as long as the lower one still can meet the side condition. We have no strong view on it.

Proposal 1: Do not introduce pathloss margin [ΔEPRE] for inter-band SSB-less SCell activation test cases.

# Conclusion

In this contribution, we continue the discussion on testing setup for NES.

Proposal 1: Do not introduce pathloss margin [ΔEPRE] for inter-band SSB-less SCell activation test cases.

# References

[1] R4-2410269, WF on RRM requirements for NR network energy saving, Huawei, HiSilicon, RAN4 #111

[2] R4-2408023, Topic summary for [111][226] Netw_Energy_NR, Moderator (Huawei), RAN4#111