# 3GPP TSG-RAN WG4 Meeting #112	R4-2411446

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.23.6

Source:	Apple

Title:	On R18 NTN RRM core requirements

Document for:	Discussion

# Introduction

In last RAN4#111 meeting, the WF for eNTN has been agreed in [1] and moderator has captured all open issues in the topic summary [2]. There are still some opens issues for further discussion, and in this contribution, we continue the discussion on remaining RRM core requirements for eNTN.

# Idle/Inactive mode mobility enhancements

### Issue 4-1: TN to NTN cell reselection

Agreement: (online) - It is a common understanding that location-based measurement triggering parts are not applicable for cell reselection from TN to NTN. Whether/How to implement this in RAN4 spec is left to CR. - If both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode, - For NTN capable UE, the cell reselection requirements (i.e., the TN to NTN reselection requirement agreed in RAN4 #110) are applied to both TN and NTN target cells/carriers. - FFS: For NTN incapable UE, the existing TN-to-TN cell reselection requirements are applied. - Further check does UE know whether it is a TN carrier or NTN carrier

In TN-to-NTN cell reselection, both the NTN and TN carrier are counted in the TN-to-NTN reselection delay equation, however, the NTN carrier has not been reflected in the TN-to-TN reselection delay equation. In our view, it shall be symmetric between these two kinds of reselection scenarios, i.e., if SIB3/4/19 are broadcasted to UE for both TN and NTN neighbor cells measurements, UE needs to measure both TN and NTN carriers and in the worst case both TN and NTN carrier shall be counted in the delay equation, and it has been agreed in last meeting that “o	For NTN capable UE, the cell reselection requirements (i.e., the TN to NTN reselection requirement agreed in RAN4 #110) are applied to both TN and NTN target cells/carriers”.

However, it was unclear in last meeting that how NTN incapable UE can know whether the target carrier in SIB4 (inter-frequency cell re-selection information) is NTN carrier or TN carrier. In current SIB4, the inter-frequency carrier information is provided as following:

If the R17 inter-frequency carrier list is present, it shall contain the same number of entries as the baseline inter-frequency carrier list. The NTN UE can read the SIB19 to understand which carrier in this inter-frequency list has the target NTN cell to measure, however, such information in SIB19 will not be understandable to an NTN incapable UE. Furthermore, there are some spectrums overlapping between NTN and TN, e.g., between n255 for NTN and n25 for TN, and between n254 for NTN and n53 for TN. The NTN incapable UE cannot use the band or ARFCN to differentiate the TN or NTN band.

| NTN satellite operating band | Uplink (UL) operating band Satellite Access Node receive / UE transmit FUL,low – FUL,high | Downlink (DL) operating band Satellite Access Node transmit / UE receive FDL,low – FDL,high | Duplex mode |
| --- | --- | --- | --- |
| n256 | 1980 MHz – 2010 MHz | 2170 MHz – 2200 MHz | FDD |
| n255 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n254 | 1610 – 1626.5 MHz | 2483.5 – 2500 MHz | FDD |
| NOTE: NTN satellite bands are numbered in descending order from n256. |  |  |  |

| NR operating band | Uplink (UL) operating band BS receive / UE transmit FUL_low – FUL_high | Downlink (DL) operating band BS transmit / UE receive FDL_low – FDL_high | Duplex Mode |
| --- | --- | --- | --- |
| n2416 | 1626.5 MHz – 1660.5 MHz | 1525 MHz – 1559 MHz | FDD |
| n53 | 2483.5 MHz – 2495 MHz | 2483.5 MHz – 2495 MHz | TDD |
| NOTE 16: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. |  |  |  |

Observation 1: For NTN incapable UE in R17 and later release, it cannot know whether it is a TN carrier or NTN carrier in the inter-frequency carrier list in SIB4 if the target NTN carriers include the ones on band n255/n254 .

The NTN incapable UE will try to search the NTN carrier if it’s in the inter-frequency carrier list of SIB4, and it will eventually fail the cell searching and cause huge delay to the other TN carrier searching. Thus, we think if both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode and the target NTN carriers include the ones on band n255/n254, for NTN incapable UE, the existing TN-to-TN cell reselection requirements are not applied. In the future release, if more overlapped bands between TN and NTN are introduced besides n254/255, this applicability requirement shall be revisited.

Proposal 1: for NTN incapable UE, if both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode and the target NTN carriers include the ones on band n255 or n254, the existing TN-to-TN cell reselection requirements are not applied.

- In the future release, if more overlapped bands between TN and NTN are introduced besides n254/255, this applicability requirement shall be revisited.

As discussed in previous meetings, when UE is camping on a TN serving cell but may perform cell re-selection to a target NTN cell, the difficult part is when UE shall turn on its GNSS modem for its GNSS location acquisition. Since UE is in TN coverage and serving cell is good enough, it will waste power if UE keeps the GNSS modem ON. However, if UE turns on the GNSS when mobility/re-selection is triggered, then it will need much long time for UE to acquire the GNSS location (first time to fix for GNSS localization is time consuming).

Observation 2: Since the first fix of GNSS will take very long time during the cell reselection procedure, it’s not necessary to always consider UE turns on the GNSS only when the neighbor NTN measurement is triggered.

In our view, there might be some scenarios can help UE to determine GNSS is ON or OFF. For instance, if UE is configured by network to have at least one high priority carrier which contains NTN cells, UE shall keep GNSS ON during such high priority frequency layer measurement even the UE is in TN coverage. As shown in the following spec text of NTN requirement, even though UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection after determining that reselection has not occurred, the high priority carrier searching is always there after configured in SIB4.

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, and the distance between UE and serving cell reference location is smaller than distanceThresh if distanceThresh is configured and UE has location information, then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2C.2.9. -------Unrelated section omitted------- When higher priority cells are found by the higher priority search, they shall be measured at least every Tmeasure,NR_Inter. If, after detecting a cell in a higher priority search, it is determined that reselection has not occurred then the UE is not required to continuously measure the detected cell to evaluate the ongoing possibility of reselection. However, the minimum measurement filtering requirements specified later in this clause shall still be met by the UE before it makes any determination that it may stop measuring the cell. If the UE detects on a NR carrier a cell whose physical identity is indicated as not allowed for that carrier in the measurement control system information of the serving cell, the UE is not required to perform measurements on that cell.

Another way is to consider a new threshold or threshold offset of RSRP/RSRQ for UE to trigger GNSS, and this offset can be applied on top of the serving cell RSRP/RSRQ threshold (legacy threshold for triggering neighbor cell measurement). For example, in the following figure, X1 is the legacy threshold to trigger neighbor cell measurement, and even higher than X1, there is a threshold X2 to trigger UE turning on the GNSS (X2>X1). That can makes UE turn on the GNSS before it reaches X1 for neighbor NTN cell measurement.

Figure 1. extra threshold to help turning on the GNSS at UE

However, regarding the limited time left for this WI, we think at least we can assume the GNSS is always ON when higher priority search is configured.

Observation 3: if UE is configured by network to have at least one high priority carrier which contains NTN cells, UE will keep GNSS ON during such high priority frequency layer measurement even the UE is in TN coverage.

Thus, we propose:

Proposal 2: For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied.

# HO for satellite switching without PCI change

The open issues in last meeting are summarized in [2] as followings:

Moderator’s WF - UE capability of parallelMeasurementWithoutRestriction-r17 is mandatorily supported for a UE supporting soft satellite switching. - Do not define known case for soft satellite switch. - Do not consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change. - During and after soft satellite switch, the total timing to detect neighbour cells has to be extended. - UE is allowed to skip measurement gaps not associated to the satellite switching from the duration of t-serviceStart and t-service - Measurement gaps or SMTCs colliding with RLM-RS are not accounted for during soft satellite switching operation

How to optimize the scheduling restriction is not fully concluded yet as highlighted in yellow above. In previous meeting, some companies proposed to use PDD measurement/report to enhance the scheduling restriction during the soft satellite switch. However, RAN2 had following agreements that,

3. We don’t introduce the support for configuring and reporting measurements on the target cell (from target satellite) in the soft switch case 4. RAN2 understands that the NW is not expected to configure PDD reporting between serving and target satellites involved in the satellite switch

In addition, when to start target satellite synchronization is up to UE implementation, even though network can somehow know the time location of received target SAT’s SSB occasion at UE, network may still need to apply scheduling restriction on all possible SSB occasions, therefore PDD reporting benefit is very limited. So RAN4 shall not consider PDD reporting between serving and target satellites involved in the satellite switch. Regarding the scheduling restriction itself, if UE is not capable of parallelMeasurementWithoutRestriction-r17, network can still configure hard satellite switching for simplicity, and it can always be alternative for these special cases. From UE capability perspective, our understanding is: If UE indicates to support soft satellite switching without PCI change, it would be typical for UE also to indicate to support simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17 as well. And in this case, then no scheduling restriction will be applied during the time period of [t-serviceStart, t-Service] during the soft satellite switching.

Proposal 3: not consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change.

Proposal 4: to enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports both simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17.

# Other issues

SFN information is essential in mobility procedure. For instance, in RACH transmission RO may not appear in every frame according to RAN1 design (TS38.211 clause 6.3.3). In TN scenario, UE can assume same SFN between target cell and source cell in PRACH transmission to target cell, e.g. during handover procedure:

For handover purposes to a target cell in paired or unpaired spectrum where the target cell uses Lmax=4, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 153600Ts if the association pattern period in clause 8.1 of [5, TS 38.213] is not equal to 10 ms. For inter frequency handover purposes where the source cell is either in paired or unpaired spectrum and the target cell is in unpaired spectrum and uses Lmax=8, the UE may assume the absolute value of the time difference between radio frame i in the current cell and radio frame i in the target cell is less than 76800Ts.

However, above assumption doesn’t cover FR2 NTN FDD bands (n510, n511, n512). More importantly, due to long propagation delay difference, above assumption actually cannot cover FR1 bands (neither TDD nor FDD). Even if different satellites can align DL frame, UE may still observe different SFN at its DL receiver (e.g. T1-T2>half frame).

To address this, we can either allow some additional time for UE to obtain SFN info or consider some assistant information to UE. The assistant information could be from source cell (if somehow source cell can get the delta SFN in certain scenario) or from some other UE nearby. SL propagation distance is quite limited compared with distance between UE and satellite. Therefore, same SFN information can be assumed for UEs close to each other. Once some UE has already got the SFN info of target cell, it can indicate this info via SL to other UEs. There could some other solutions. We are open for further discussion on whether and how to address this issue in future release.

# Conclusion

In this contribution, we continue the discussion on remaining RRM core requirements for eNTN.

Proposal 1: for NTN incapable UE, if both TN and NTN carriers are broadcasted for neighbour cells measurement in IDLE/Inactive mode and the target NTN carriers include the ones on band n255 or n254, the existing TN-to-TN cell reselection requirements are not applied.

- In the future release, if more overlapped bands between TN and NTN are introduced besides n254/255, this applicability requirement shall be revisited.

Proposal 2: For TN to NTN cell re-selection requirement when Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, if UE is configured by network to have at least one high priority carrier which contains NTN cells, the requirements for GNSS ON shall be applied.

Proposal 3: not consider PDD reporting between serving and target satellites involved in the satellite switching without PCI change.

Proposal 4: to enhance scheduling restriction in soft satellite switching, consider modifying the capability rule such that: if UE indicates to support soft satellite switching without PCI change, it also means UE supports both simultaneousRxDataSSB-DiffNumerology and parallelMeasurementWithoutRestriction-r17.

# References

[1] R4-2410380, WF on RRM requirements for NR_NTN_enh, Qualcomm Incorporated, RAN4 #111

[2] R4-2408016, Topic summary for [111][219] NR_NTN_enh, Qualcomm Incorporated, RAN4#111