# 3GPP TSG-RAN WG4 Meeting #112	R4-2411447

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	5.23.7

Source:	Apple

Title:	On R18 NTN RRM core requirements

Document for:	Discussion

# Introduction

In last RAN4#111 meeting, the WF for eNTN has been agreed in [1] and moderator has captured all open issues in the topic summary [2]. There are still some opens issues for further discussion, and in this contribution, we continue the discussion on remaining RRM performance requirements for eNTN.

# Discussion

The previous agreement and open issue for UE accuracy requirement over 10GHz is:

Issue 6-2-2: (FR2-NTN) Rx beam gain

Agreement: (ad-hoc + online) - The existing absolute measurement accuracy requirement and relative measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz with [1]dB relaxation. - Note: Companies are encouraged to further evaluate the performance loss due to single polarization assumption on FR2 Ka band VSAT UE. It’s not precluded to further update the tentative relaxation value in future RAN4 meeting. - Remove an RF margin for different RX beams in the relative accuracy. - Define the minimum SSB_RP condition for accuracy requirement of five UE types specified in table 9.2.1.0-1 of TS38.101-5. Minimum SSB_RP_NTN_FR2 for Rx Beam Peak angle of arrival = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n, where, - Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5. - PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32; - SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB; - SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise; - ∆MBP,n is 0. - For the minimum SSB_RP condition, - The gain difference between fine and rough beams is YdB: - For mechanical steering antenna, Y=0 - For electronic steering antenna, FFS Y - Do not define separate spherical coverage unless spherical coverage is introduced in RF session. - FFS: Gmin and Gmax - Note: If anything above inconsistent with RF requirement is identified, RAN4 to make updates to those aspects accordingly.

In last RAN4 meeting, we have identified that RAN4 RF had one assumption in TS38.101-5 that, the minimum requirements on the receiver characteristics apply under one polarization.

In FR2 TN, the dual polarization is always assumed, i.e., DL signals measurement in orthogonal polarizations. RAN4 never discussed if single polarization can achieve the same accuracy as dual-polarization case, but we can use RedCap 1Rx as one example to discuss the requirement. When 1Rx is used for FR1 RedCap UE from 2Rx, the 1dB relaxation on accuracy was allowed due to the degradation of Rx diversity. Thus, the same relaxation in FR1 1Rx RedCap can be reused here to reflect such degradation by losing the diversity reception, e.g., 1dB relaxation is assumed.

Proposal 1: remove the bracket for the following 1dB relaxation:

- The existing absolute measurement accuracy requirement and relative measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz with 1dB relaxation.

The TN measurement accuracy is defined as following example:

The parameter of SSB_RP has been specified in annex B of TS38.133, as following example:

The UE power class for legacy FR2 TN UE may not be applied for NTN UE above 10GHz. In UE RF spec session, one big CR has been agreed in last meeting in [3], and the UE class were defined quite different from legacy power class, as duplicated below,

Thus, instead of using UE power class, RAN4 shall define the minimum SSB_RP condition for accuracy requirement for 5 UE types specified in table 9.2.1.0-1 of TS38.101-5.

In the legacy TN FR2 accuracy requirement, the minimum SSB power is determined based on the following criteria:

In NTN above 10GHz, the same principle can be applied to derive the mini SSB_RP, which is based on the equation of Minimum SSB_RP = Reference sensitivity UE type, n512, 50MHz +Y -10Log10(PRBRefsens x 12) – SNRRefsens + SSB Ês/Iot + ∆MBP,n, where,

Reference sensitivity UE type, n512, 50MHz is the reference sensitivity value in dBm specified for a specific UE type in Band n512 for 50 MHz Channel bandwidth in Table 10.3.2-1 and section 10.3.3-1 of TS 38.101-5 as following:

Y is the gain difference between fine and rough beams. Since RF has no spherical coverage requirement for NTN above 10GHz, we assume there is no such difference between fine and rough beam. Moreover, RF session has the same rule for all 5 NTN VSAT types, i.e., no different between fine and rough beam for all 5 NTN VSAT types. Thus, we propose Y=0 for both mechanical steering antenna and electronic steering antenna.

PRBRefsens is NRB associated with subcarrier spacing 120 kHz for 50MHz in TS 38.101-5 Table 5.3.2-2, and is 32;

12 is the number of subcarriers in a PRB;

SNRRefsens is the SNR used for simulation of Refsens and EIS spherical coverage, and is -1 dB;

SSB Ês/Iot is the minimum value required by the UE to perform measurements, and is -6 dB for intra-frequency measurements and -4 dB for inter-frequency measurements. The only contribution to Iot is the UE internal noise;

∆MBP,n is 0 for NTN above 10GHz since CA/DC is not considered.

Proposal 2: For the minimum SSB_RP condition, only single type of beam is assumed, i.e., the gain difference between fine and rough beams is YdB:

- Y=0, for both mechanical steering antenna and electronic steering antenna

To derive the Rx beam gain in the proposal 2, we may consider the following formulas,

- REFSENS_50MHz = -174 dBm/Hz + 10log10(50MHz) + NF + SNR (QPSK) + IM - Ant_Gain

Therefore Ant_Gain can be derived as,

- Ant_Gain = -174 dBm/Hz + 10log10(50MHz) + NF + SNR (QPSK) + IM - REFSENS_50MHz

For FR2-NTN, two REFSENS_50MHz values have been specified,

- -126.8 dBm for Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO

- -115.6 dBm for fixed VSAT communication with LEO only

With the following parameter assumptions,

- NF = 10 dB (we used a typical value here from previous FR2 discussion in RF, but can be adjusted if companies have concern)

- SNR = -1 dB

- IM = 2 dB (here, it’s just our understanding for example, may need FFS in RAN4 to decide the value among companies)

We can derive the following two Ant_Gain values,

- For Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO:

  - Ant_Gain = -174dBm + 77dB + 10dB + (-1dB) + 2dB - (-126.8dBm) = 40.8dB

- For fixed VSAT communication with LEO only,

  - Ant_Gain = -174dBm + 77dB + 10dB + (-1dB) + 2dB - (-115.6dBm) = 29.6dB

Thus, based on our calculation the lower bound of Rx beam gain can be as following:

- For Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO, the lower bound of Rx beam gain is 41dB.

- For fixed VSAT communication with LEO only, the lower bound of Rx beam gain is 30dB.

However, the calculation can only be used to decide the lower bound of beam gain, and we still need VSAT vendor to claim the upper bound of the Rx beam gain.

Proposal 3:

For Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO, the lower bound of Rx beam gain is 41dB.

Proposal 4:

For fixed VSAT communication with LEO only, the lower bound of Rx beam gain is 30dB.

Proposal 5:

VSAT vendor to claim the upper bound of the Rx beam gain.

# Conclusion

In this contribution, we discuss the NR-NTN measurement accuracy requirements in above 10 GHz bands.

Proposal 1: remove the bracket for the following 1dB relaxation:

- The existing absolute measurement accuracy requirement and relative measurement accuracy requirement of TN FR2 (including intra-frequency and inter-frequency) can be applied for NTN UE above 10GHz with 1dB relaxation.

Proposal 2: For the minimum SSB_RP condition, only single type of beam is assumed, i.e., the gain difference between fine and rough beams is YdB:

- Y=0, for both mechanical steering antenna and electronic steering antenna

Proposal 3:

For Mobile VSAT communication with GSO, and for fixed VSAT communication with GSO and LEO, the lower bound of Rx beam gain is 41dB.

Proposal 4:

For fixed VSAT communication with LEO only, the lower bound of Rx beam gain is 30dB.

Proposal 5:

VSAT vendor to claim the upper bound of the Rx beam gain.

# References

[1] R4-2410380, WF on RRM requirements for NR_NTN_enh, Qualcomm Incorporated, RAN4 #111

[2] R4-2408016, Topic summary for [111][219] NR_NTN_enh, Qualcomm Incorporated, RAN4#111