# 3GPP TSG-RAN WG4 Meeting #112	R4-2411448

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.4.3

Source:	Apple

Title:	On RRM core for less than 5MHz Phase 2

Document for:	Discussion

# Introduction

From this meeting, RAN4 will start the work on R19 WI of NR channel BW less than 5MHz for FR1 Phase 2, and the latest WID[1] approved in last RANP#104 meeting includes following RRM topics:

- Define common co-located and non-co-located inter-band NR CA/DC UE RF requirements with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band - Example band combination: CA/DC of 3MHz in band n100 and 5MHz or 10MHz in band n101 - Define RRM requirements for inter-band CA and DC for combinations introduced in RF part Note: other band combinations than example band combinations can be specified in basket WIs after the above generic requirements are specified.

In this contribution, we discuss the RRM requirement scope for co-located and non-co-located inter-band NR CA/DC UE with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band.

# RRM for CA/DC in less than 5MHz

In R18 less than 5MHz RRM, RAN4 didn’t define requirements for some features and scenarios, e.g., CSI-RS based L1/L3/RLM/BFD/CBD measurement, and therefore we think the features/scenarios not considered in R18 less than 5MHz RRM shall not be discussed in R19 CA/DC with less than 5MHz band.

Proposal 1: the features/scenarios not considered in R18 less than 5MHz RRM shall not be discussed in R19 CA/DC with less than 5MHz band, e.g., CSI-RS based L1/L3/RLM/BFD/CBD measurement.

In CA/DC with less than 5MHz, if the PCell is using 3MHz in band n100, the legacy R18 less than 5MHz RRM requirement in RRC connected mode can be applied without any change, e.g., cell identification/measurement/SSB-index reading, and SSB based RLM/BFD/CBD, handover and etc.

Proposal 2: In CA/DC with less than 5MHz, if PCell is using 3MHz, the legacy R18 less than 5MHz RRM requirement in RRC connected mode can still be applied on this PCell.

As described in WID, we didn’t see any scenario required CA/DC have two or more serving cells with less than 5MHz, that is, RAN4 only needs to consider following scenarios:

- CA with 3MHz PCell + 5MHz or 10MHz SCell(s)

- CA with 5MHz or 10MHz PCell + one 3MHz SCell

- DC with 3MHz PCell + 5MHz or 10MHz PSCell

- DC with 5MHz or 10MHz PCell + 3MHz PSCell

Since the detailed band combination of CA/DC needs to wait RF conclusions, it’s better to decide the corresponding RRM requirement after RF session has concrete conclusions, e.g., whether or not we need multiple SCell activation, or whether more than 2 serving cells shall be considered in this inter-band CA with less than 5MHz.

Proposal 3: RAN4 to decide the corresponding RRM requirement after RF session has concrete conclusions, e.g., whether or not we need multiple SCell activation, or whether or not more than 2 serving cells shall be considered in this inter-band CA with less than 5MHz.

In R18 less than 5MHz, we had some agreements in RAN4#109 (R4-2321629) on whether network needs to provide the bandwidth information to UE during MO configuration or HO, and the main reason is because CD-SSB PCell has to be on sync raster and R18 only considered single carrier case for less than 5MHz.

## Sub-topic 1-2 Assistance information for PBCH is 12 or 20 PRBs Agreement: There is no need not include information regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

However, if now SCell can also use 12 PRB SSB bandwidth, as CD-SSB of SCell can be off sync raster, then it would be necessary to provide assistance information regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command, otherwise, UE may assume wrong PBCH DMRS bandwidth and fail the SSB index reading.

Proposal 4: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

Following table is the summary for CA/DC requirement and potential impacts due to new bandwidth.

Table 1. CA/DC requirements impacts due to less than 5MHz band

| Section ID | CA/DC requirements | Impacts due to less than 5MHz band |
| --- | --- | --- |
| 6.1.5 | HO with PSCell | Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. |
| 6.1.6 | NR Conditional Handover including target MCG and target SCG | Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. |
| 7.1 | UE transmit timing | No, the reference cell for timing requirement can reuse the legacy assumption. |
| 7.5, 7.6 | MRTD/MTTD requirement | No, reuse the legacy CA/DC requirement. |
| 8.2.2, 8.2.4 | Interruption requirement related with SCell/PSCell operation | No, reuse the legacy interruption requirement. |
| 8.3 | SCell Activation and Deactivation Delay | Need FFS, if target to-be-activated SCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the SCell activation requirement. |
| 8.4 | UL carrier RRC reconfiguration delay | No, reuse the legacy requirement. |
| 8.5.9 | Requirements for Beam Failure Recovery in SCell | No, reuse the legacy BFR requirement. |
| 8.9, 8.9A, 8.9C | NR-DC: PSCell Addition and Release Delay; Conditional PSCell Addition Delay; Subsequent Conditional PSCell Addition Delay | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell addition requirement. |
| 8.11, 8.11B, 8.11E | PSCell Change; Conditional PSCell Change; Subsequent Conditional PSCell Change | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell change requirement. |
| 8.17 | SCG Activation and Deactivation Delay | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell activation requirement. |
| 9.1.2 | Measurement gap | No, reuse the legacy MG requirement. |
| 9.1.3 | UE Measurement capability | No, reuse the legacy requirement. |
| 9.1.4 | Capabilities for Support of Event Triggering and Reporting Criteria | No, reuse the legacy requirement. |
| 9.1.5 | CSSF | No, reuse the legacy CSSF requirement. |
| 9.1.7~13 | MG enh and MUSIM MG | NA for this WI (MG enh is not considered) |
| 9.2.3 | Number of cells and number of SSB (FR1) | No, reuse the legacy requirement. |
| 9.2.5 | Intra-frequency measurements without measurement gaps | Yes, time period for time index detection for deactivated SCell or deactivated PSCell can be extended of this SCell or PSCell is using 3MHz. |
| 9.2.6 | Intra-frequency measurements with measurement gaps | No, reuse the legacy requirement. SCell and PSCell measurement is not in this section. |
| 9.2.7 | Intra-frequency measurements with NCSG | NA for this WI (MG enh is not considered) |
| 9.3 | NR inter-frequency measurements | No, reuse the legacy requirement. SCell and PSCell measurement is not in this section. |

Proposal 5: agree the above summary table.

# Conclusion

In this contribution, we discuss the RRM requirement scope for co-located and non-co-located inter-band NR CA/DC UE with 3MHz CBW in the one band and 5MHz or 10MHz CBW in the other band.

Proposal 1: the features/scenarios not considered in R18 less than 5MHz RRM shall not be discussed in R19 CA/DC with less than 5MHz band, e.g., CSI-RS based L1/L3/RLM/BFD/CBD measurement.

Proposal 2: In CA/DC with less than 5MHz, if PCell is using 3MHz, the legacy R18 less than 5MHz RRM requirement in RRC connected mode can still be applied on this PCell.

Proposal 3: RAN4 to decide the corresponding RRM requirement after RF session has concrete conclusions, e.g., whether or not we need multiple SCell activation, or whether or not more than 2 serving cells shall be considered in this inter-band CA with less than 5MHz.

Proposal 4: If SCell can use 12PRB SSB bandwidth in R19, it’s necessary to provide assistance information to UE regarding whether the PBCH is 12 or 20 PRBs in either MO or HO command.

Proposal 5: agree the following summary table.

| Section ID | CA/DC requirements | Impacts due to less than 5MHz band |
| --- | --- | --- |
| 6.1.5 | HO with PSCell | Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. |
| 6.1.6 | NR Conditional Handover including target MCG and target SCG | Yes. SSB index acquisition of less than 5MHz cell can be extended in legacy requirement. |
| 7.1 | UE transmit timing | No, the reference cell for timing requirement can reuse the legacy assumption. |
| 7.5, 7.6 | MRTD/MTTD requirement | No, reuse the legacy CA/DC requirement. |
| 8.2.2, 8.2.4 | Interruption requirement related with SCell/PSCell operation | No, reuse the legacy interruption requirement. |
| 8.3 | SCell Activation and Deactivation Delay | Need FFS, if target to-be-activated SCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the SCell activation requirement. |
| 8.4 | UL carrier RRC reconfiguration delay | No, reuse the legacy requirement. |
| 8.5.9 | Requirements for Beam Failure Recovery in SCell | No, reuse the legacy BFR requirement. |
| 8.9, 8.9A, 8.9C | NR-DC: PSCell Addition and Release Delay; Conditional PSCell Addition Delay; Subsequent Conditional PSCell Addition Delay | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell addition requirement. |
| 8.11, 8.11B, 8.11E | PSCell Change; Conditional PSCell Change; Subsequent Conditional PSCell Change | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell change requirement. |
| 8.17 | SCG Activation and Deactivation Delay | Need FFS, if target PSCell is using 3MHz, the SSB index acquisition of less than 5MHz cell may extend the PSCell activation requirement. |
| 9.1.2 | Measurement gap | No, reuse the legacy MG requirement. |
| 9.1.3 | UE Measurement capability | No, reuse the legacy requirement. |
| 9.1.4 | Capabilities for Support of Event Triggering and Reporting Criteria | No, reuse the legacy requirement. |
| 9.1.5 | CSSF | No, reuse the legacy CSSF requirement. |
| 9.1.7~13 | MG enh and MUSIM MG | NA for this WI (MG enh is not considered) |
| 9.2.3 | Number of cells and number of SSB (FR1) | No, reuse the legacy requirement. |
| 9.2.5 | Intra-frequency measurements without measurement gaps | Yes, time period for time index detection for deactivated SCell or deactivated PSCell can be extended of this SCell or PSCell is using 3MHz. |
| 9.2.6 | Intra-frequency measurements with measurement gaps | No, reuse the legacy requirement. SCell and PSCell measurement is not in this section. |
| 9.2.7 | Intra-frequency measurements with NCSG | NA for this WI (MG enh is not considered) |
| 9.3 | NR inter-frequency measurements | No, reuse the legacy requirement. SCell and PSCell measurement is not in this section. |

# References

[1] RP-241622, NR channel BW less than 5MHz for FR1 Phase 2, Intel, RANP#104