# 3GPP TSG-RAN WG4 Meeting #112	R4-2411449

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.22.4.1

Source:	Apple

Title:	On simulation assumption for LP-WUR based measurement

Document for:	Approval

# Introduction

In last RAN4#111 meeting, the WF for RRM requirements for LP-WUSWUR was agreed in [1], and open issues have been summarized in [2]. In this contribution, we continue the discussion on the simulation assumption.

# Simulation assumptions

In [1], the draft simulation assumption has been attached in the appendix. In our view, RAN4’s simulation assumption shall be close to the one for RAN1’s evaluation, and of course RAN4 can add something necessary to RAN4 requirement design work. The justification is to make sure the RAN4 assumption for requirement design would be generally in line with RAN1’s performance expectation. And also, in last RAN4 meeting, the LLS simulation assumption for RF evaluation has been agreedin WF R4-2410569 [3].

Proposal 1: For LP-WUR based RRM, RAN1’s LLS simulation assumption and RAN4 RF session’s LLS simulation assumption can be used as baseline for RAN4 RRM simulation work.

Followings are our understanding on the simulation assumption, and the comments are added in the most right column for each row.

Table 1: General parameters

| Simulation parameters | Comments/values | Apple’s comments |
| --- | --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | FR1: 2.6 GHz/700MHz | Pick one carrier frequency which is used in RAN1 R18 SI. |
| System bandwidth | 20/100 MHz; | RAN1 agreed following: From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X = 11PRBs - FFS the number of PRBs for 15kHz - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2 Thus, only need to pick one system BW >=5MHz and assume SCS=30kHz for RAN4 simulation. |
| Prior knowledge of Cell 1 / Cell 2 by the UE | No / Yes | Cell 2 is used as an interference cell and no need to have such assumption here. |
| DRX | No | The DRX shall be always used for IDLE/inactive mode, but it’s not necessary to mention it in simulation assumption. |
| BS transmit antennas for LP-SS blocks | 1 tx or single layer transmissions |  |
| UE receive antennas | 1 rx |  |
| Data and control channel subcarrier spacing | 30kHz for data/control/SSB/LP-SS | There is no need to consider the mixed numerology case in simulation. Same SCS can be considered to simplify the simulation work. |
| Measurement period (in number of measurement samples) | OOK based: [4, other number of samples may also be studied upon a need] OFDM based: [4] | In IDLE mode current intra-frequency and serving cell measurement is based 4 samples for the lowest DRX case. Propose to use 4 samples instead of 5, and other values can be FFS. |
| Subcarrier spacing | 30KHz 2.6GHz: 15 kHz and 30 kHz; 700MHz: 15kHz | Same comment as for system BW. |
| Number of LP-SS blocks per SS burst set, K | [1], FFS on other values | It could be revised in case RAN1 has any assumption. |
| LP-SS/SSB burst periodicity | LP-SS based [320 ms] OFDM based: [320 ms] |  |
| Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |  |
| LP-SS block BW | [132 subcarriers for SCS=30kHz;] [288 subcarriers for SCS=15kHz ] Note: May need be updated based on RAN1 conclusion | RAN1 agreed that From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X = 11PRBs |
| Actual LP-SS block transmissions | always transmitted |  |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal | Choose one candidate from RF simulation assumption agreed in last RAN4 meeting, - For ASCS: 0 or 1RB on each side of LP-WUS bandwidth - For ACS: 1/2/3/4 RB We chose 1RB here. |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 | Apple’s comments |
| --- | --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |  |
| PSS, SSS of SSB(OFDM based LP-WUR) | - | Same as in R4-1708698 |  | To pick the sequence from legacy PSS/SSS assumption in R4-1708698. |
| Overlaid OFDM sequence(s) for LP-SS |  | FFS | FFS | It’s still open in RAN1, and current agreements are: Agreement Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS - Down select among gold sequence, m sequence and ZC sequence - FFS the overlaid OFDM sequence is time or frequency domain sequence. - FFS how to reuse the existing sequences - Note: Strive to minimize the impact on OOK detection performance - If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS |
| LP-SS assumption |  | OOK-1, OOK-4 with M=2,4, FFS:1,8,16 Note: select one M for OOK-4 based simulation. | Same assumption as for cell 1. | Working Assumption in RAN1: Support the following options for LP-SS - Option 1: OOK-1 - Option 2: OOK-4 with M=2,4, FFS:1,8,16 - FFS whether value of M depends on SCS - The SCS of a CP-OFDM symbol used for LP-SS generation is the same as that used for LP-WUS generation FFS how OOK-1 and OOK-4 are specified To simplify RAN4 simulation, it’s not that necessary to try all candidates of M, we may only need to evaluate OOK-1 and OOK-4 with one selected M. |
| Binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS |  | Wait for RAN1 conclusion | Same assumption as for cell 1. | Agreements from RAN1: For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following: - Gold sequence - M sequence - Computer searched sequence - FFS: the length of LP-SS sequence |
| number of binary LP-SS sequences for the ‘ON-OFF’ pattern |  | Wait for RAN1 conclusion | Same assumption as for cell 1. | Agreements from RAN1: Further down-select the number of binary LP-SS sequences for the ‘ON-OFF’ pattern: - 3 - 4 - 8 - 16 |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |  |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |  |
| RB Utilization | % | 100 | 100 |  |
| Data Modulation |  | Uncorrelated pseudo random QPSK modulated data | Uncorrelated pseudo random QPSK modulated data | Create random data with QPSK like in OCNG |
| Slot length | - | 14 symbols | 14 symbols |  |
| CP Length | - | Normal | Normal |  |
| Residual Frequency error |  | [0/10/20]ppm Note: can also be FFS | [0/10/20]ppm Note: can also be FFS | RAN1 Agreement: The LP-WUS and LP-SS design assumes the residual frequency error after frequency error correction without considering impact of drift, is up to X ppm for OOK-based LP-WUR. - FFS X which is no larger than 20ppm - Initial frequency error assumption: up to company report For the overlaid OFDM sequence design of LP-WUS, it is assumed that the residual frequency error for OFDM-based LP-WUR after frequency error correction without considering impact of drift is not larger than Y. - FFS Y which is no larger than 20ppm and lower than X - Initial frequency error assumption: up to company report In last RAN4 meeting, RF session agreed that: Issue 2-1-6: Frequency error assumption for LLS simulation Agreement - Frequency error assumption for LLS simulation - Up to 20ppm |
| Timing error |  | 0 |  | In last RAN4 meeting, RF session agreed that time error=0 for LLS. |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | [CP/2] |  |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | [3 ms] |  |
| SNR | dB | FFS | FFS | To chieve the target SINR. |
| Es/IoT (calculated from SNR) | dB | [-3dB] Note: -3dB can be used as starting point but need FFS based on RAN1 discussion. Other candidate values are not precluded. | NA | -3dB can be used as starting point but need FFS based on RAN1 discussion. Other candidate values are not precluded. |
| Propagation conditions | - | FR1: AWGN TDL-A 30ns TDL-B 100ns TDL-C 300ns |  |  |
| UE speed |  | FR1: 3 km/h |  |  |

Table 3: UE-specific parameters

| Parameters | Assumptions | Apple’s comments |
| --- | --- | --- |
| [Receiver Filter] | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case | To be in line with RF LLS assumptions in [3]. But it can be left to UE implementation. |
| [Receiver ADC bit width] | 4/8 bits ADC for ASCS/ACS | To be in line with RF LLS assumptions in [3]. But it can be left to UE implementation. |
| [Receiver Sampling Rate for LP-SS only] | 7.68MHz | To be in line with RF LLS assumptions in [3]. But it can be left to UE implementation. |

Proposal 2: to agree the above LLS simulation assumption for LP-WUR based RRM measurement.

# Conclusion

In this contribution, we continue the discussion on the simulation assumption.

Proposal 1: For LP-WUR based RRM, RAN1’s LLS simulation assumption and RAN4 RF session’s LLS simulation assumption can be used as baseline for RAN4 RRM simulation work.

Proposal 2: to agree the following LLS simulation assumption for LP-WUR based RRM measurement.

Table 1: General parameters

| Simulation parameters | Comments/values |
| --- | --- |
| Carrier frequency for Cell 1 and Cell 2 | FR1: 2.6 GHz |
| System bandwidth | 20MHz; |
| BS transmit antennas for LP-SS blocks | 1 tx or single layer transmissions |
| UE receive antennas | 1 rx |
| Data and control channel subcarrier spacing | 30kHz for data/control/SSB/LP-SS |
| Measurement period (in number of measurement samples) | OOK based: [4, other number of samples may also be studied upon a need] OFDM based: [4] |
| Subcarrier spacing | 30KHz |
| Number of LP-SS blocks per SS burst set, K | [1], FFS on other values |
| LP-SS/SSB burst periodicity | LP-SS based [320 ms] OFDM based: [320 ms] |
| Number of transmit antenna ports | 1 (the same port for NR-SSS, NR-PSS, NR-PBCH) |
| LP-SS block BW | [132 subcarriers for SCS=30kHz;] Note: May need be updated based on RAN1 conclusion |
| Actual LP-SS block transmissions | always transmitted |
| Guard band | 1 RB on each side of LP-SS/LP-WUS signal |

Table 2: Cell-specific parameters

| Parameter | Unit | Cell 1 | Cell 2 |
| --- | --- | --- | --- |
| RF Channel number | - | Channel 1 | Channel 1 |
| PSS, SSS of SSB(OFDM based LP-WUR) | - | Same as in R4-1708698 |  |
| Overlaid OFDM sequence(s) for LP-SS |  | FFS | FFS |
| LP-SS assumption |  | OOK-1, OOK-4 with M=2,4, FFS:1,8,16 Note: select one M for OOK-4 based simulation. | Same assumption as for cell 1. |
| Binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS |  | Wait for RAN1 conclusion | Same assumption as for cell 1. |
| number of binary LP-SS sequences for the ‘ON-OFF’ pattern |  | Wait for RAN1 conclusion | Same assumption as for cell 1. |
| PBCH and DMRS power offset with respect to NR-PSS, NR-SSS and LP-SS | dB | 0 | 0 |
| Data and control PSD relative to NR-PSS,NR-SSS and LP-SS | dB | 0 | 0 |
| RB Utilization | % | 100 | 100 |
| Data Modulation |  | Uncorrelated pseudo random QPSK modulated data | Uncorrelated pseudo random QPSK modulated data |
| Slot length | - | 14 symbols | 14 symbols |
| CP Length | - | Normal | Normal |
| Residual Frequency error |  | [0/10/20]ppm Note: can also be FFS | [0/10/20]ppm Note: can also be FFS |
| Timing error |  | 0 |  |
| 1)Relative Delay of 1st Path (synchronous) | µs | 0 | [CP/2] |
| 2) Relative Delay of 1st Path (asynchronous): Fixed delay | Ms | 0 | [3 ms] |
| SNR | dB | FFS | FFS |
| Es/IoT (calculated from SNR) | dB | [-3dB] Note: -3dB can be used as starting point but need FFS based on RAN1 discussion. Other candidate values are not precluded. | NA |
| Propagation conditions | - | FR1: AWGN TDL-C 300ns |  |
| UE speed |  | FR1: 3 km/h |  |

Table 3: UE-specific parameters

| Parameters | Assumptions |
| --- | --- |
| [Receiver Filter] | - 3th/5th Order lowpass Butterworth matching fixed 3.96MHz RF bandwidth for 10MHz/20MHz case - Other order lowpass filter is not precluded - The filter bandwidth is adapted with actual WUS RBs, for 5MHz case |
| [Receiver ADC bit width] | 4/8 bits ADC for ASCS/ACS |
| [Receiver Sampling Rate for LP-SS only] | 7.68MHz |

# References

[1] R4-2410296, WF for RRM core requirements for LP-WUS/WUR, Moderator (vivo), RAN4 #111

[2] R4-2408026, Topic summary for [111][229] NR_LPWUS, vivo, RAN4 #111

[3] R4-2410569, WF for [111][136] NR_LPWUS_UERF, vivo, RAN4 #111