# 3GPP TSG-RAN WG4 Meeting #112	R4-2411451

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.21.2

Source:	Apple

Title:	On RRM core requirements for R19 NES enhancement

Document for:	Discussion

# Introduction

From this meeting, RAN4 will start the work on R19 WI of Enhancements of network energy savings for NR, and the latest WID[1] approved in last RANP#104 meeting includes multiple RRM related topics, as followings:

The objectives of the work item are the following: - Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode, including: [RAN1/2/3] - Triggering method by uplink wake-up-signal using an existing signal/channel. - Wake-up-signal configuration provisioning to UE - Note: No modification of SSB will be discussed under this objective - Information exchange between gNBs at least for the configuration of wake-up signal, if necessary. - Checkpoint for normative work in RAN#105 - Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4] - Adaptation of SSB in time domain, e.g. adapting periodicity - Adaptation of PRACH in time domain - Adaptation of paging occasions including confining the paging occasions in the time domain - Note: there shall be no paging latency increase - Note: there shall be no negative impact to legacy UEs, unless significant benefits are shown - Specify the corresponding core requirements, for the above features [RAN4].

# On-demand SSB based RRM

Since RAN plenary agreed on “Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4]”, the OD-SSB shall be considered for the CA related RRM procedures, e.g., deactivated SCell measurement, SCell activation procedure and etc.

In previous RAN1 meeting, there were some agreements already for the scenarios and cases as followings:

RAN#116bis Agreement For the identified scenarios and cases (as per RAN1#116 agreement), on-demand SSB can be triggered by gNB at least for the following scenarios/cases: - Scenario #2 and Case #1 - Scenario #2 and Case #2 - Scenario #2A and Case #1 - Scenario #2A and Case #2 - FFS: Scenario #3A and Case #1 - FFS: Scenario #3A and Case #2 - FFS: Scenario #3B and Case #1 - FFS: Scenario #3B and Case #2 - For Case #1, once on-demand SSB is triggered, its transmission is in a periodic manner. - Note: This does not imply periodic on-demand SSB is transmitted indefinitely after triggered. - Notes: - Scenario #2A refers to - “When UE receives SCell activation command (e.g., as defined in TS 38.321)” - Scenario #3A refers to - “After UE receives SCell activation command (e.g., as defined in TS 38.321) until SCell activation is completed” - Scenario #3B refers to - “When SCell activation is completed and SCell is activated” or - “After SCell activation is completed and SCell is activated” - For discussion purpose under AI 9.5.1, always-on SSB is SSB supported in Rel-18 specifications. - Timing for on-demand SSB transmission (e.g. when the triggered SSB starts and ends) will be separately discussed. RAN1#116 Agreement Regarding the UE assumption on SSB transmission on a cell supporting on-demand SSB SCell operation, the following cases are identified for further study: - Case #1: No always-on SSB on the cell - Case #2: Always-on SSB is periodically transmitted on the cell - FFS: Whether always-on SSB and on-demand SSB are not cell-defining SSB if transmitted. FFS: Which scenario the above applies for Agreement For the following identified scenarios for on-demand SSB SCell operation, focus future RAN1 discussion to down-select (both may be selected) between the two scenarios. - Scenario #2: SCell is configured to a UE but before the UE receives SCell activation command (e.g., as defined in TS 38.321) - Scenario #3: After UE receives SCell activation command (e.g., as defined in TS 38.321) - This does not preclude SCell for which activation is completed - FFS: The case where SCell activation is completed FFS: Application timing between NW triggering message and on demand SSB transmission

The scenarios considered in RAN1 can also be illustrated in the following figure 1:

Figure 1. Scenarios of OD-SSB indication with respect to SCell activation procedure by MAC CE

Since scenario 2/2A has been agreed to consider in RAN1, the deactivated SCell measurement shall be one scope to discuss in RAN4. Regarding scenario 2A, on-demand SSB can be indication by gNB at when UE receives SCell activation command. In current RAN4 requirement, the processing time for UE to decode the MAC CE of SCell activation is: n+THARQ+3ms (n is the slot of SCell activation command). Now if UE also receives OD-SSB indication from network at the same time as SCell activation, the processing time of OD-SSB indication is still under discussing in RAN1, and following options are currently on the table,

Agreement - For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell. - FFS: Whether to support DCI based signaling to indicate on-demand SSB transmission on the cell. - This DCI signaling does not provide SCell activation/deactivation. - If supported, details on DCI including UE-specific or group-common DCI, DCI contents, etc. - FFS: Scenarios where the above signalings are applicable Agreement For SSB burst(s) indicated by on-demand SSB SCell operation via MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is [the slot boundary of] the first SSB time domain position [of actually transmitted on-demand SSB burst] which is T [slots or symbols] after the [slot or symbol] where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - FFS: Details of the value of T (≥ 0) including possibility of T comprising of multiple components - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - FFS: Whether the value of T is predefined or indicated/configured by gNB - FFS: Details of “the [slot or symbol] where UE receives a signalling from gNB” or “the [slot or symbol] where UE transmits HARQ-ACK corresponding to a signalling from gNB to trigger on-demand SSB” Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

The components of time instance A is still open in the RAN1 discussion, and it’s hard to say how much time it will extend to process OD-SSB indication together with the SCell activation command, and it might rely on many factors: e.g., whether or not such OD-SSB is in the same MAC CE as SCell activation CMD or in a separated MAC CE. But no matter how RAN1 concludes on this time instance A, the SCell activation delay requirement in RAN4 shall need potential clarification or revisiting, since so far only SCell activation command processing time is considered in the delay requirement.

On the other hand, since scenario 3A/3B is not agreed yet, RAN4 may need wait RAN1 conclusions for the next step on corresponding requirement, e.g., OD-SSB based L1/L3 measurements for activated SCell. In short, we think scenario 3A/3B related RRM requirements can be discussed after RAN1 has concrete conclusions.

Proposal 1: RAN4 to start work on OD-SSB based deactivated SCell measurement and OD-SSB based SCell activation delay requirement first for scenario 2/2A with case #1 and case #2.

Proposal 2: scenario 3A/3B related RRM requirements can be discussed after RAN1 has concrete conclusions, e.g., OD-SSB based L1/L3 measurements for activated SCell for scenario 3B and SCell activation delay requirement for scenario 3A.

So far both MAC CE based SCell activation and RRC based direct SCell activation is under discussing in RAN1, but we are not sure if the direct SCell activation or other CA enhancement feature needs to be considered in this R19 NES WI as high/urgent priority. The RRC based SCell activation includes (1) activation at SCell addition, (2) activation at handover, and (3) activation at RRC resume, however, it’s unclear which one or all of them shall be considered in this WI. Moreover, to simplify the work at the beginning of this WI, we prefer to focus on the simplest case first in requirement design, e.g., the SCell activation delay requirement design can firstly focus on baseline single SCell activation, and after we conclude on the baseline case, RAN4 may discuss other enhanced CA scenarios, e.g., multiple SCell activation, PUCCH SCell activation, as well as the direct SCell activation.

Proposal 3: for OD-SSB based SCell activation requirement, RAN4 to first focus on the single SCell activation case as baseline; and then after we conclude on the baseline case, RAN4 may discuss other enhanced CA scenarios if needed, e.g., multiple SCell activation, PUCCH SCell activation, and direct SCell activation.

Regarding the OD-SSB based SCell activation requirement, we have some preliminary thinking on the impacts of OD-SSB for Case #1 especially:

Potential issue 1: how to differentiate SSB-less SCell and OD-SSB SCell in Case#1.

We just identified one thing which may cause backward compatibility issue with our current work in R18. In R18 SSB-less SCell activation, the way UE to understand target SCell is SSB-less SCell is based on the following configuration:

If the SCell being activated belongs to FR1 and if the UE is not provided with SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell, and if there is one collocated active reference serving cell on different FR1 band, when the following conditions are fulfilled for UE supporting [scellWithoutSSB-interband], …

However, if the SCell being activated is an OD-SSB SCell but no SSB transmission is triggered yet (Case #1), network may not provide SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell to the UE, and then UE cannot differentiate if this is inter-band SSB-less SCell operation or UE needs to request SSB by itself for on-demand SSB SCell operation. However, in SSB-less SCell operation we have reference cell configuration or by-default reference cell assumed based on QCL-typeC source cell. So perhaps, if the target SCell is configured to UE without SSB (e.g., neither SSB configuration (absoluteFrequencySSB) in the target SCell (FrequencyInfoDL) nor SMTC configuration for the target SCell), and reference serving cell is not indicated by higher layer parameter [SSB-less-Referencecell], and QCL-typeC source cell is not configured to UE for the target SCell, then such target SCell can be treated as on-demand SSB SCell rather than a SSB-less SCell. Of course, we are open to further discuss it in future meetings. Moreover, the SSB-less non-collocated SCell activation is under discussing in RAN1 as a potential scope, and RAN4 may also need to work on the corresponding requirement once RAN1 has concrete conclusions.

Potential issue 2: when to start reporting OOR CQI and lowest L1 SS-RSRP during OD-SSB based SCell activation in Case#1.

The UE behavior in legacy SCell activation procedure is defined as following:

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

Starting from the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

However, the OD-SSB transmission might be available on time instance A after the OD-SSB indication, and such time uncertainty of OD-SSB could be longer than legacy SSB in case #1. Like shown in the following figure 2,

Figure 2. CQI reporting during OD-SSB based SCell activation

If UE supports OD-SSB operation and has been configured a SCell with OD-SSB, considering all possible scenarios for Case #1, we think the current CQI reporting and L1-RSRP reporting can be changed like following, but we are open to further discuss in future meetings.

After the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation), starting from the slot where OD-SSB indication is received and until the UE has completed the SCell activation, the UE shall report out of range if the UE has available uplink resources to report CQI for the SCell.

After the slot specified in clause 4.3 of TS 38.213 [3] (timing for secondary Cell activation/deactivation), starting from {the slot where OD-SSB indication is received} and until the UE has completed a first L1-RSRP measurement, the UE shall report lowest valid L1 SS-RSRP range if the UE has available uplink resources to report L1-RSRP for the SCell.

Proposal 4: RAN4 to discuss the following issues for SCell activation in case #1 besides the SCell activation delay:

- Potential issue 1: how to differentiate SSB-less SCell and OD-SSB SCell in Case#1.

- Potential issue 2: when to start reporting OOR CQI and lowest L1 SS-RSRP during OD-SSB based SCell activation in Case#1.

# On-demand SIB1 based RRM

The scope in WID for OD-SIB1 is to study procedures and signaling method(s) to support on-demand SIB1 for UEs in idle/inactive mode. In RAN4 spec, only the requirement of maximum interruption in paging reception has SI reading time as a delay component, and that SI reading is on a neighbor cell. Furthermore, WID stated that no modification of SSB will be discussed under this objective. Thus, the OD-SIB1 mechanism in this WI will not impact any existing RRM requirement from our point of view.

Proposal 5: no RRM work is needed for OD-SIB1 in R19 NES WI.

# Adaptation of common signal/channel transmissions

Regarding the common RS adaptation, the main RRM impacts probably come from the following two bullets of the WID:

Specify adaptation of common signal/channel transmissions. [RAN1/2/3/4]

  - Adaptation of SSB in time domain, e.g. adapting periodicity

  - Adaptation of PRACH in time domain

In previous RAN1 and RAN2 discussion, some agreements of SSB adaptation were achieved as followings:

RAN1#117 Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB RAN1#116bis Agreement For indication of adaptation of SSB in time-domain, - Support at least SSB adaptation provided by gNB without UE trigger Agreement Adaptation mechanism(s) of SSB in time-domain is supported at least for one of the following scenario(s): - For cell with both legacy UEs and Rel-19 NES-capable UEs - Rel-19 NES-capable UE’s PCell (Connected mode) - Study from the following options: - Option A1: adaptation for CD-SSB - Option A2: adaptation for SSB that is not CD-SSB - Option A3: adaptation for SSB not on sync raster - Rel-19 NES-capable UE’s SCell - Study from the following options: - Option B1: adaptation for CD-SSB - Option B2: adaptation for SSB that is not CD-SSB - Option B3: adaptation for SSB not on sync raster - FFS: Rel-19 NES-capable UE in idle/inactive mode - Note: Impact to idle/inactive UEs shall be minimized RAN1#116 Agreement For adaptation of SSB in time-domain, consider the following adaptation mechanisms for further study - Adaptation of SSB burst periodicity - Adaptation based on two SSB configurations where up to two configurations can be active - Adaptation based on skipping/transmitting some SSB bursts non-uniformly with single SSB configuration - Adapting the transmitted number of SSBs within a SSB burst - Cell DTX for SSB adaptation - Whether to support new SSB burst periodicity value(s) - Whether to support new SSB burst(s) (i.e. how SSB transmission is made within a burst) - New compact SSB burst(s) - Adapting the position of SSBs within a SSB burst - Other mechanisms/combinations are not precluded Agreement For the adaptation mechanisms of SSB in time-domain, study further applicable scenarios and associated legacy UE impact/handling (if any) based on the following: - Applicability to UE in idle/inactive and/or connected mode - Applicability to PCell and/or SCell(s) Agreement For the adaptation mechanisms of SSB in time-domain, study further following mechanisms: - Adaptation mechanism indicated or configured by gNB without UE trigger - Adaptation triggered by UE (if any) FFS: Details of associated signaling/indication/configuration

In last RAN1#117 meeting, only adaptation mechanism option 1 has been agreed, but the other SSB adaptation mechanisms are still open for further study, e.g.,

- Adaptation based on two SSB configurations where up to two configurations can be active

- Adapting the transmitted number of SSBs within a SSB burst

Many of RAN4 requirements are related with SSB periodicity, SSBs inside SSB burst and SMTC periodicity, it will be necessary to revisit the RRM requirements associated with those adaptations. Moreover, the DTX for SSB adaptation will also impact the RRM measurement if the SSB outside DTX active window will be disabled (it’s different from R18 NES DTX assumption). However, due to the RAN1 progress on SSB adaptation, we think RAN4 needs more conclusions from RAN1 for RRM requirement design.

In last RAN plenary meeting, only the adaptation of PRACH in time domain was kept in this WI, and corresponding agreements from RAN1/2 are as followings:

RAN1#117 Agreement For adaptation of PRACH in time-domain, support at least the following case(s) - Case 1: no time-domain overlap between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 2: time-domain overlap but no overlap in frequency domain between the additional PRACH resources for NES-capable UEs and the PRACH resources for legacy UEs - Case 3: additional PRACH resources for NES-capable UEs and legacy PRACH resources overlap neither in time nor frequency domains - FFS: whether additional conditions are needed to support the above cases - FFS: Additional case whether full/partial overlap in both time and frequency is allowed - Above does not preclude discussion for the case where the configuration for additional PRACH resources contains legacy PRACH resources Agreement At least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. - Mapping SS/PBCH block indexes to valid additional PRACH occasions provided by semi-static signalling follows the legacy mapping order for preamble/time resource/frequency/PRACH slot indexes. - Note: This mapping is not impacted by time domain PRACH adaptation - Validation rules for the additional PRACH resources follow the legacy validation rules for PRACH resources configured for legacy UEs. Agreement For adaptation of PRACH in time-domain, the additional PRACH resources are configured based on at least: - a PRACH configuration index - FFS: whether the PRACH configuration index is same and/or different from the PRACH configuration index for the legacy PRACH resources Study further the following - When the PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resource, - Additional parameter(s) for determining the additional PRACH resources e.g. - Scaled/adjusted PRACH configuration period - Additional timing offset - Adjusting the parameters (e.g., (x, y) value and slot number) of the PRACH configuration - Muting/masking ROs - When the PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resource - Additional mechanisms (if any) for determining the additional PRACH resources e.g. - Muting/masking ROs (e.g. for the case when the PRACH configuration index for the additional PRACH resources contains legacy resources) - Additional parameters to facilitate condensed/cluster RACH resources in time-domain (including whether needed) Agreement For the adaptation mechanism for additional PRACH resources, study further the following: - Option 1: Higher layer signalling (with potential enhancements) based PRACH resource adaptation - Option 2: L1-based adaptation to indicate whether the additional PRACH resources provided by semi-static signalling are available or not - FFS: details - Strive to re-use existing DCI format(s) - Option 3: Adaptation of PRACH transmission according to predefined condition(s) - FFS: details - Option 4-rev1: L1-based adaptation to indicate whether a subset of the additional PRACH resources provided by semi-static signalling are available or not - FFS: whether the subset of the additional PRACH resources is in RO level / SSB-to-RO mapping cycle level/PRACH association period level/PRACH association pattern period level for time-domain PRACH adaptation - Strive to re-use existing DCI format(s) - Option 5: Enhanced cell DRX

Based on the assumption of case 1/2/3 agreed in last RAN1 meeting, PRACH resources for legacy UEs won’t be impacted and at least for the case where legacy ROs and additional ROs overlap in neither time nor frequency domain, for adaptation of PRACH in time-domain, the SSB-RO mapping rule for additional PRACH resources follows the legacy SSB-RO mapping rule. In case1/2 it’s unclear so far if the new mapping relation between SSB and RO would be introduced together with the new signaling, and therefore it cannot be decided yet if the RRM requirement using legacy SSB-RO mapping needs to be revisited or not. For example, in the current RACH requirement, it specified as followings, and if the new signaling or mapping rule is introduced, it shall also be clarified in this requirement.

Proposal 6: RAN4 to discuss the RRM requirement for SSB adaption and RACH adaptation after more concrete conclusions are made by RAN1.

Proposal 7: No need to discuss RRM requirement for PO adaptation.

Regarding the SSB adaptation, even though so far the conclusions from RAN1 is not sufficient for RRM requirement design, we have some preliminary analysis for the potential RRM impacts.

If NES network adapts the transmitted number of SSBs within a SSB burst, and the following transition will happen for SSB bursts,

Figure 3. adapting number of SSBs within a SSB burst

In current RLM and BFD requirement, the UE behavior of indication (e.g., OOS/IS for RLM) is defined as in TS38.133 section 8.1.6:

When the downlink radio link quality on all the configured RLM-RS resources is worse than Qout, layer 1 of the UE shall send an out-of-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the out-of-sync indications as specified in TS 38.331 [2].

When the downlink radio link quality on at least one of the configured RLM-RS resources is better than Qin, layer 1 of the UE shall send an in-sync indication for the cell to the higher layers. A layer 3 filter shall be applied to the in-sync indications as specified in TS 38.331 [2].

It would have one potential issue that: if multiple SSBs or SSB+CSI-RS is configured for RLM or BFD and one or some of SSBs are turned off or turned on in the burst due to SSB adaptation, what’s the impact to the RLM and BFD results? It can be illustrated as following:

Figure 4. potential issues for SSB adaptation within SSB burst

In our view, the simplest assumption for UE behavior in this case is: UE can restart the RLM/BFD evaluation based on the remaining SSB and/or CSI-RS, i.e., UE is allowed to drop all the previous RLM indication and evaluation results, and restart the RLM/BFD evaluation based on the remaining SSB and/or CSI-RS in the RLM/BFD-RS list, and the timer and counter for the RLM/BFD can also be restarted as well, e.g., N310, N311, N313, N314, T310. However, we are open for other options in RAN4 discussion.

One more thing to consider is the processing time for UE to apply the SSB adaptation. The legacy SSB periodicity reconfiguration is defined in current TS38.331 by using ssb-periodicityServingCell, and the RRC reconfiguration time is also defined in TS38.331 as following. The reconfiguration processing time is normally 10ms or 16ms(for SCell) in RRC spec, and it can be understood as UE’s processing time of preparing and applying the reconfiguration.

One potential issue identified by us is as in figure 5: if network changes the SSB periodicity or the SSB muting happening before UE completing the RRC parameter processing, what shall UE do? E.g., if SSB burst #3 is a SSB for UE measurement in the old configuration, since UE has not completed processing the SSB adaptation indication when SSB burst #3 comes, it’s unclear what UE shall do for this SSB burst? Or from network perspective, if it’s assumed as an error case, what shall network do to avoid such error case?

Figure 5. potential issue for SSB adaptation processing time

In our view, by default, the processing time of SSB adaptation must be smaller than or equal to regular RRC reconfiguration time to gain the benefit for this feature (otherwise we can directly use RRC reconfiguration to achieve the same goal). Furthermore, we think if the time interval between upcoming SSB and SSB adaptation indication is greater than processing time, the periodicity change of SSB shall apply from the upcoming SSB; but if the time interval between upcoming SSB and SSB adaptation indication is less than processing time, the periodicity change of SSB shall apply from the SSB after the upcoming SSB (after UE complete the SSB adaptation indication).

# Conclusion

In this contribution, we continue the discussion on remaining RRM core requirements for NES.

Proposal 1: RAN4 to start work on OD-SSB based deactivated SCell measurement and OD-SSB based SCell activation delay requirement first for scenario 2/2A with case #1 and case #2.

Proposal 2: scenario 3A/3B related RRM requirements can be discussed after RAN1 has concrete conclusions, e.g., OD-SSB based L1/L3 measurements for activated SCell for scenario 3B and SCell activation delay requirement for scenario 3A.

Proposal 3: for OD-SSB based SCell activation requirement, RAN4 to first focus on the single SCell activation case as baseline; and then after we conclude on the baseline case, RAN4 may discuss other enhanced CA scenarios if needed, e.g., multiple SCell activation, PUCCH SCell activation, and direct SCell activation.

Proposal 4: RAN4 to discuss the following issues for SCell activation in case #1 besides the SCell activation delay:

- Potential issue 1: how to differentiate SSB-less SCell and OD-SSB SCell in Case#1.

- Potential issue 2: when to start reporting OOR CQI and lowest L1 SS-RSRP during OD-SSB based SCell activation in Case#1.

Proposal 5: no RRM work is needed for OD-SIB1 in R19 NES WI.

Proposal 6: RAN4 to discuss the RRM requirement for SSB adaption and RACH adaptation after more concrete conclusions are made by RAN1.

Proposal 7: No need to discuss RRM requirement for PO adaptation.

# References

[1] RP-241650, Enhancements of network energy savings for NR, Ericsson, Apple, RANP#104