# 3GPP TSG-RAN WG4 Meeting #112	R4-2411452

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.25.4

Source:	Apple

Title:	On RRM core for R19 NTN Phase 3

Document for:	Discussion

# Introduction

From this meeting, RAN4 will start the work on R19 WI of NTN enhancement Phase 3, and the latest WID[1] approved in last RANP#104 meeting includes following topics:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Uplink Capacity/Cell Throughput Enhancement for FR1-NTN [RAN1, RAN2, RAN4] - Specify signaling of the intended service area of a broadcast service (e.g. MBS broadcast) via NR NTN [RAN2, RAN3] - Support of regenerative payload [RAN3, RAN2, RAN4] - Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1]

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

# RRM for downlink coverage enhancements

In last RAN plenary meeting, the scope was revised as following:

- Study and specify if beneficial downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Study and if needed specify solutions, including link level enhancements for FR1-NTN (e.g. for PDCCH, PDSCH) and/or system level enhancements for FR1-NTN and/or FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - RAN1 to report at the latest by RAN#106 with the list of targeted physical channels/signals for link level enhancements (if any), and with the targeted system-level enhancements (if any) - RAN1 should report on impact to backward compatibility, if any, for potential extension of the SSB periodicity at the latest by RAN#106, in conjunction with the targeted system-level enhancements. - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km - Rel-18 network energy saving techniques should be considered as baseline in the system level study

The SSB periodicity extension for DL coverage enhancement can probably impact UE initial access procedure compared with the legacy assumption of 20ms. RAN4 may need to double check if any RRM requirement shall be clarified when the SSB periodicity is changed to a one different than 20ms.

In the current initial access assumption for cell selection, it was defined in RAN1 that,

In the current NTN network (R17, and R18), by default NTN UE will try the initial access to network assuming SSB periodicity as 20ms, however, if the R19 NTN network changes the SSB periodicity to be more than 20ms, it will cause the initial access failure for the R17/R18 NTN UEs. Even for the R19 NTN UE, it has no clear idea if the network behavior is following R17/R18 or R19, and the UE behavior is unclear for implementation during the initial access.

Besides the initial access, there are other UE behaviors requiring assumption of the by-default SSB periodicity in the previous NTN networks (R17/R18). Some examples as followings:

    - For handover or CHO (conditional HO), if the UE is not provided SMTC configuration or measurement object on this frequency, the requirement in this clause is applied with Trs=5ms assuming the SSB transmission periodicity is 5ms. There is no requirement if the SSB transmission periodicity is not 5ms.

    - For RRC connection release with redirection to NR, if the UE is not provided with SMTC configuration or measurement object for the frequency which is also configured for the RRC connection release with redirection then: the requirement in this clause is applied with Trs = 20 ms if the SSB transmission periodicity is not larger than 20 ms; otherwise, there is no requirement if the SSB transmission periodicity is larger than 20ms.

With the SSB periodicity change(e.g., change to a SSB periodicity > 20ms), the above corresponding requirement needs to be carefully reviewed if any RAN4 work is needed.

Proposal 1: RAN4 to decide if corresponding RRM requirement(s) needs to be updated/clarified after RAN1 has concrete conclusion on the SSB periodicity extension for DL coverage enhancement.

In our view, if such SSB periodicity extension is adopted in RAN1, it’s only applied for NTN operation on NTN bands. We think it might be beneficial to have some new assumption at UE to help initial access to R19 NTN, and one possible approach could be to specify band or region specific SSB periodicity. For example, UE will perform initial access by searching the cells with a specific SSB periodicity associated with the specific band. It can be defined in RF that each NTN band is associated with one or more SSB periodicity in RF spec, like the association between band and SCS in existing TS38.101-1/2, e.g., Band X is associated with SSB periodicity M. With this approach, network can follow the RF spec to transmit SSB on each specific band by using the associated periodicity. But details of the solution can be further discussed after RAN1 has more conclusions.

In the note of this SSB periodicity extension, WID has a note that “Rel-18 network energy saving techniques should be considered as baseline in the system level study”. One key technology in Rel-18 NES is the DTX for network energy saving, and RAN4 assumed the SSB and other RS for RRM shall be transmitted even outside DTX active window. However, in R19 NTN enhancement, to support DL coverage enhancement (network may sweep Tx beams among different footprints) SSB may not be transmitted within certain footprint outside certain time window, e.g., outside DTX active window. If that’s the assumption of DTX in Rel-19 NTN, we can foresee that it must have RRM requirement impact compared with Rel-18 NES DTX. Since how to apply the DTX/DRX mechanism of NES to Rel-19 NTN is still under discussing in RAN1, RAN4 shall wait more conclusions from RAN1 before starts RRM work.

Proposal 2: RAN4 to wait more conclusions from RAN1 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES.

# RRM for uplink Capacity/Cell Throughput Enhancement

For UL capability/cell throughput enhancement, enhancement may only need RF requirement update instead of any RRM part. Thus. RAN4 doesn’t need to discuss RRM requirement for uplink Capacity/Cell Throughput Enhancement.

Proposal 3: No need for RAN4 to discuss RRM requirement for uplink Capacity/Cell Throughput Enhancement.

# RRM for support of regenerative payload

In last RAN plenary meeting, the scope was agreed as following:

Support of regenerative payload [RAN3, RAN2, RAN4] - Specify the support of gNB on board in TS 38.300 - Specify, if needed, any necessary enhancements related to the intra and inter-gNB mobility, especially for Xn interface over feeder link or over ISL. [RAN3] - Note: if any additional necessary stage-3 specifications impact for e.g. NGAP is identified, RAN3 will handle it.

Since the regenerative model is used at satellite, the feeder link does not exist in this case, and therefore corresponding RAN4 requirement may need some clarification. For example, in timing requirement, we have following Te requirement in NTN,

Since the common TA and Kmac shall always be 0 in the regenerative mode, it may need to clarify such common TA parameter = 0 in timing requirement. Moreover, when regenerative mode is used, satellite switching can directly mean cell switch, and therefore there shall not be any satellite switching without PCI change for regenerative mode.

Proposal 4: RAN4 to clarify common TA = 0 in timing requirement for regenerative mode of NTN.

Proposal 5: RAN4 to clarify requirement of satellite switching without PCI change is not applied for regenerative mode of NTN.

# RRM for support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN

In last RAN plenary meeting, the scope for NTN with RedCap/eRedCap was agreed as following:

Support of Rel-17 RedCap and Rel-18 eRedCap UEs with NR NTN operating in FR1-NTN bands [RAN4, RAN1] - For full-duplex and half duplex FDD RedCap and eRedCap UEs, define the RF and RRM requirements [RAN4] - For HD-FDD RedCap UEs and eRedCap UEs, specify enhancements for mitigating issues caused by TA mismatch between actual TA used by the UE and assumed TA for the UE at the gNB [RAN1] - Enhancements are targeted for the following HD collisions cases (cases 3 and 4): - Semi-statically configured DL reception collides with semi-statically configured UL transmission - Dynamically scheduled DL reception collides with dynamic scheduled UL transmission - Note: enhancements for other HD collision cases are not targeted. Enhancements for HD collision cases 3 and 4 that may improve other HD collision cases are not precluded. - Note: potential work in RAN2, if any, starts after RAN1 has decided on the solution. - Notes for this objective: - GNSS (Global Navigation Satellite Systems) capabilities and simultaneous GNSS and NR-NTN operation is supported in RedCap/eRedCap UE.

Regarding the R17 RedCap UE, the key features impacted RRM requirements are:

- 1Rx

- RRM relaxation

- Idle/Inactive mode eDRX (Inactive mode eDRX<=10.24s)

- NCD-SSB

- HD-FDD

Regarding the R18 eRedCap UE, the key features impacted RRM requirements are:

- Inactive mode eDRX>=10.24s

Since the scenario is changed a lot for NTN compared with the legacy TN network, it’s hard to say if the legacy RedCap/eRedCap requirement can be still appliable to NTN UE in R19. We suggest to have more discussion on all these RRM related features for RedCap/eRedCap NTN. For HD-FDD RedCap UE in NTN, even though RAN1 will discuss the HD collisions cases for case 3 and case 4, the DL RRM measurement RS (including serving and neighbour cell) colliding with UL transmission still needs to be discussed in RAN4.

For 1Rx, NCD-SSB and HD-FDD, we think the legacy RedCap requirement can be used as baseline, while RAN4 needs more discussion on whether new requirement shall be introduced for RRM relaxation and eDRX. For example, in legacy NTN requirement, the 2.56s DRX is not assumed to be well utilized in the scenario where earth-moving LEO is deployed as the target satellite or serving satellite for measurement. The main reason is: if earth-moving LEO is used and the footprint on earth is not that large, e.g., couple kilometers, then the longer measurement delay due to using 2.56s will fail the mobility. If eDRX>10.24sec is considered/configured (PTW window is used) and earth-moving LEO is serving cell or is configured as target satellite for UE’s measurement, we think if DRX in PTW window is not greater than1.28s, UE shall complete the target NTN satellite measurement/evaluation within single PTW window, i.e., no cross PTW window measurement/evaluation; however, if DRX in PTW window is greater than 1.28s, we still think UE shall be allowed to skip or drop the measurement/detection/evaluation, i.e., UE is not required to meet the requirement for 2.56s DRX cycle length for earth moving LEO deployment when eDRX with PTW window is configured. We are open for more discussion in the future meeting.

Proposal 6: RAN4 to analyze the RRM requirement impacts from following features for RedCap/eRedCap NTN:

- 1Rx

- NCD-SSB

- HD-FDD

- RRM relaxation

- Idle/Inactive mode eDRX with and without PTW window

Proposal 7: For 1Rx, NCD-SSB and HD-FDD, the legacy RedCap/eRedCap requirement can be used as baseline for R19 NTN.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: RAN4 to decide if corresponding RRM requirement(s) needs to be updated/clarified after RAN1 has concrete conclusion on the SSB periodicity extension for DL coverage enhancement.

Proposal 2: RAN4 to wait more conclusions from RAN1 to decide whether and how NTN RRM can be impacted by DTX/DRX from NES.

Proposal 3: No need for RAN4 to discuss RRM requirement for uplink Capacity/Cell Throughput Enhancement.

Proposal 4: RAN4 to clarify common TA = 0 in timing requirement for regenerative mode of NTN.

Proposal 5: RAN4 to clarify requirement of satellite switching without PCI change is not applied for regenerative mode of NTN.

Proposal 6: RAN4 to analyze the RRM requirement impacts from following features for RedCap/eRedCap NTN:

- 1Rx

- NCD-SSB

- HD-FDD

- RRM relaxation

- Idle/Inactive mode eDRX with and without PTW window

Proposal 7: For 1Rx, NCD-SSB and HD-FDD, the legacy RedCap/eRedCap requirement can be used as baseline for R19 NTN.

# References

[1] RP-241667, Non-Terrestrial Networks (NTN) for NR Phase 3, Thales, CATT, RANP#104