# 3GPP TSG-RAN WG4 Meeting #112	R4-2411453

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.8.3.4

Source:	Apple

Title:	On less than 5MHz for NTN RRM

Document for:	Discussion

# Introduction

The RAN4 led NTN enhancement WI has been agreed in RAN plenary #104 and the latest WID was approved in [1]. The core part of this WI includes high power UE (HPUE) for NTN and less than 5MHz for NTN. We didn’t see impact of HPUE to RRM requirement but the less than 5MHz for NTN will decently need new RRM requirement in NTN.

Followings are the scope for less than 5MHz for NR NTN.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

In this contribution, we discuss the RRM requirement scope for less than 5MHz for NR NTN.

# Discussion

The TN RRM requirements have been specified in Rel-18 RRM spec TS38.133. Compared with the RRM requirement of legacy BW, some of the RRM requirement has been modified to accommodate the new BW, e.g., 3MHz. As stated in the WID,

  - Add less than 5MHz channel bandwidth

    - 3MHz with the RAN1 design in Rel-18

We assume that the 3MHz design in Rel-18 for TN shall be reused for NTN band n255/n256/254. However, in Rel-18 less than 5MHz WI, RAN4 didn’t define the RRM requirement for some features, e.g., CSI-RS based L3/L1/CBD/BFD/RLM requirement, and here for Rel-19 less than 5MHz for NR NTN, we think the features not specified in R18 TN RRM requirement for less than 5MHz WI shall not be considered for R19 NTN with less than 5MHz BW, e.g., CSI-RS based L1/L3/CBD/BFD/RLM requirement.

Proposal 1: the features not specified in R18 TN RRM requirement for less than 5MHz WI shall not be considered for R19 NTN with less than 5MHz BW, e.g., CSI-RS based L1/L3/CBD/BFD/RLM requirement.

In current Rel-18 TS38.133, the main change to legacy requirement due to less than 5MHz are followings:

- Applicability requirement in clause 3.6.18

- Handover requirement (T∆ = 3*Trs for both known and unknown target cells operating with 12 PRB SSB BW.)

- RLM requirement (PDCCH transmission parameters)

- BFD requirement (PDCCH transmission parameters)

- time index detection for intra-frequency/inter-frequency cell identification with/without MG

Like the TN less than 5MHz, we also think the above change shall be applied to NTN requirement as well. Thus, the RRM requirement for TN less than 5MHz shall be used as baseline to design NTN less than 5MHz requirement, e.g., T∆ in HO requirement, PDCCH transmission parameters in RLM/BFD requirement, and	time index detection for intra-frequency/inter-frequency cell identification with/without MG.

Proposal 2: the RRM requirement for TN less than 5MHz shall be used as baseline to design NTN less than 5MHz requirement, e.g.,

- T∆ in HO requirement,

- PDCCH transmission parameters in RLM/BFD requirement,

- time index detection for intra-frequency/inter-frequency cell identification with/without MG.

Proposal 3: Like in TN less than 5MHz WI, an applicability requirement is needed to clarify which R17/R18 NTN requirement can be applied for less than 5MHz band without any change in R19.

# Conclusion

In this contribution, we discuss the RRM requirement scope for WI of Non-Terrestrial Networks (NTN) for NR Phase 3.

Proposal 1: the features not specified in R18 TN RRM requirement for less than 5MHz WI shall not be considered for R19 NTN with less than 5MHz BW, e.g., CSI-RS based L1/L3/CBD/BFD/RLM requirement.

Proposal 2: the RRM requirement for TN less than 5MHz shall be used as baseline to design NTN less than 5MHz requirement, e.g.,

- T∆ in HO requirement,

- PDCCH transmission parameters in RLM/BFD requirement,

- time index detection for intra-frequency/inter-frequency cell identification with/without MG.

Proposal 3: Like in TN less than 5MHz WI, an applicability requirement is needed to clarify which R17/R18 NTN requirement can be applied for less than 5MHz band without any change in R19.

# References

[1] RP-241281, Enhanced requirements and conductive test methodology for NR NTN and IoT NTN, Samsung, Xiaomi, RANP#104