# 3GPP TSG-RAN WG4 Meeting #112	R4-2411454

# Maastricht, Netherlands, 19th – 23rd August, 2024

Agenda item:	8.15.2.2

Source:	Apple

Title:	On FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC

Document for:	Discussion

# Introduction

The WF of NR Radio Resource Management (RRM) Phase 5 [1] has been agreed in RAN4#111, and the open issues have been summarized in the topic summary for [111][228] for NR_RRM_Ph5 [2]. In this contribution, we continue the discussion for FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC.

# Discussion

Issue 2-2-1: Clarification on the bullets in WID for this CSSF optimization

FFS: - Option 1 (Apple): Rel-19 CSSF optimization applies for the both cases: (1)UE is not capable of Rel-18 multi-Rx simultaneous reception, (2)UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently. - Option 2 (ZTE): Besides the case UE is not capable of R18 multi-Rx simultaneous reception, R19 CSSF optimization is also applied to the case that UE is capable of multi-Rx but not configured with GBBR report. - Option 3 (Ericsson): Clarify whether ‘the case UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently’. - Does it indicate that the UE only can use a single panel (and subsequent receiver including baseband) out of multiple panels for reception and measurement? If so, is there any degradation of reception and measurement from measuring on a single panel? - Option 4 (Nokia): Rel-19 discussion on the scenarios for CSSF optimization will be considered in CA/DC scenarios, independently of the UE support of multi-Rx capabilities.

In our view, the case of “the UE has capability of multi-Rx but the multi-Rx function is not enabled or activated” shall be treated equivalently as the UE who’s not capable of multi-Rx feature. Since the key point is we don’t want to mix two enhancement together in the same scenario, and therefore, we are supportive to adopt the option 1 in last meeting. However, we also understand the intention in option 4 is to simplify the discussion, and we are also fine to make this CSSF enhancement somehow independent from the R18 multi-Rx simultaneous reception WI. Unfortunately, CA/DC scenario cannot be a condition to prevent the impact from R18 multi-Rx feature, as the following CA/DC scenario is also supported for R18 multi-Rx WI,

#### Issue 1-8: Applicable scenarios of Rel-18 multi-Rx feature - RAN4 Rel-18 multi-Rx related requirements are applicable to PCell, PSCell, or SCell, provided the cell is the only serving cell in the single FR2-1 band and UE is configured with one FR2-1 band.

Thus, the CA/DC scenario in this R19 CSSF enhancement can be further clarified as CA/DC with at least 2 FR2 serving cells, and such clarification can preclude the scenario of using R18 multi-Rx feature. We are either agree on the option 1 or the option 4 with our further clarification.

Proposal 1: agree on either one of following options:

- Option 1: Rel-19 CSSF optimization applies for CA/DC scenario when (1)UE is not capable of Rel-18 multi-Rx simultaneous reception or (2)UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently.

- Option 4a: Rel-19 discussion on the scenarios for CSSF optimization will be considered in CA/DC scenarios with at least two FR2 serving cells, independently of the UE support of multi-Rx capabilities.

Issue 2-2-2: UE measurement procedure to use L3 measurement delay reduction by optimizing CSSF

Agreement: - Only consider CSSF outside MG case. FFS: - Option 1: the following scenarios in CA/DC to use L3 measurement delay reduction by optimizing CSSF shall be prioritized: - SSB based Intra-frequency measurement without MG (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Alt2: including TPSS/SSS_sync_intra, TSSB_time_index_intra and TSSB_measurement_period_intra - SSB based Inter-frequency measurement without MG, (Apple, CATT, CMCC, vivo, CTC, HW, Nokia, MTK) - Alt1: including TPSS/SSS_sync_inter and TSSB_measurement_period_inter - Alt2: including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Inter-RAT SSB measurement without MG (CMCC, Ericsson) - NeedForGaps measurement without MG, including both with and without interruption (Ericsson) - NCSG measurement without MG without interruption (Ericsson)

In WID description, it states that,

  - For UE not in multiple-Rx simultaneous reception mode:

    - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing:

      - CSSF outside gap in CA/DC scenarios

        - Baseline assumption on number of searchers is 2

We also think RAN4 can prioritize the baseline scenarios for CSSF optimization study as starting point, i.e., SSB based Intra-frequency measurement without MG and SSB based Inter-frequency measurement without MG. In the current spec, the SSB index reading is not needed for intra-frequency measurement without MG for FR2-1, but it’s needed for inter-frequency measurement without MG for FR2-1.So the scope of intra-frequency measurement without MG could be different from the inter-frequency measurement without MG in this WI.

Proposal 2: the following scenarios to use L3 measurement delay reduction by optimizing CSSF shall be prioritized:

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_measurement_period_inter and TSSB_time_index_inter

Issue 2-2-3: Applicability requirement of L3 measurement delay reduction by optimizing CSSF

FFS: - Option 1 (Apple, CATT, OPPO, CMCC, ZTE): RAN4 to consider following CA/DC mode for L3 measurement delay reduction by optimizing CSSFoutside_gap,i - EN-DC (Apple, CATT, OPPO, CMCC, ZTE, Ericsson): - Intel proposed to deprioritize EN-DC - NE-DC (Apple, CATT, OPPO, CMCC, ZTE, Intel, Ericsson): - SA (Apple, CATT, OPPO, CMCC, ZTE, CTC, Intel, HW): - FR2+FR2 CA (HW), FR1+FR2 CA (HW, Ericsson), FR2 only intra band CA (Ericsson), FR2 only inter band CA (Ericsson) - NR-DC (Apple, CATT, OPPO, CMCC, ZTE, CTC, Intel) - FR1+FR2 NR-DC (HW, Ericsson)

The WID include the CSSF related enhancement for FR2-1 SSB based L3 measurement delay reduction, however, it doesn’t preclude the CA or DC case when the FR1 serving carrier is also configured to UE. In last meeting, companies proposed different kinds of CA/DC modes for study, and we have no strong view on which one shall be chosen since the current CSSF outside gap in the TS38.133 covers all EN-DC, NE-DC, SA and NR-DC.

To be more specific, we are also fine to further narrow down the CA/DC mode to the following FR1/FR2 combination:

  - EN-DC

    - EN-DC with FR2 only intra band CA

    - EN-DC with FR2 only inter band CA

    - EN-DC with FR1 +FR2 CA (FR1 PSCell)

    - EN-DC with FR1 +FR2 CA (FR2 PSCell)

  - NE-DC

    - NE-DC with FR2 only intra band CA

    - NE-DC with FR2 only inter band CA

    - NE-DC with FR1 +FR2 CA (FR1 PCell)

  - SA

    - FR1+FR2 CA (FR1 PCell)

    - FR1+FR2 CA (FR2 PCell)

    - FR2 only intra-band CA

    - FR2 only inter-band CA

  - NR-DC

    - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

Proposal 3: RAN4 to consider EN-DC, NE-DC, SA and NR-DC for L3 measurement delay reduction by optimizing CSSFoutside_gap,i, and the specific scope can be as following:

- EN-DC

  - EN-DC with FR2 only intra band CA

  - EN-DC with FR2 only inter band CA

  - EN-DC with FR1 +FR2 CA (FR1 PSCell)

  - EN-DC with FR1 +FR2 CA (FR2 PSCell)

- NE-DC

  - NE-DC with FR2 only intra band CA

  - NE-DC with FR2 only inter band CA

  - NE-DC with FR1 +FR2 CA (FR1 PCell)

- SA

  - FR1+FR2 CA (FR1 PCell)

  - FR1+FR2 CA (FR2 PCell)

  - FR2 only intra-band CA

  - FR2 only inter-band CA

- NR-DC

  - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

Issue 2-2-4: Searcher assumption to apply L3 measurement delay reduction by optimizing CSSF

In WID: For UE not in multiple-Rx simultaneous reception mode: - Study suitable scenarios and conditions and, if feasible, introduce methods to reduce FR2-1 L3 measurement delay by optimizing: - CSSF outside gap in CA/DC scenarios - Baseline assumption on number of searchers is 2. - Option 1 (Apple, CTC, Intel, MTK, Ericsson): RAN4 only consider the R19 enhancement based on 2 searchers. - Option 2 (CATT, CMCC): enhance R19 CSSF outside gap by considering both 2 searchers and 3 searchers. - Option 2a (HW): enhance R19 CSSF outside gap by considering 3 searchers for per FR gap capable UE. - Option 4 (Nokia): - RAN4 to confirm if one of the searchers is assumed for PCell measurement and the other is assumed for the measurements on all the SCells. - Recommended WF - Moderator note: if companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers.

RAN plenary agreed in the WID that the baseline assumption on number of searchers is 2, and that’s also aligned with the RAN4 fundamental assumption from Rel-15. Some companies raised one issue that: in current spec, if UE supports per-FR MG, and if one FR measurement is using MG and the other FR measurement is not using MG, we don’t have a joint section of CSSF requirement to define a per-UE CSSF factor including both FRs, and that implies UE supports 3 searchers in this case. Unfortunately, we disagree with this assumption and interpretation, our view is: this is just a missing piece of current spec, and we don’t have specific test case to verify UE behavior in this case. In the field, the deployment of FR2 is very rear and the UE measurement speed cannot be easily verified in the current field log (at least no error observed in current industry). Additionally, searcher number change is a quite big challenge to UE implementation and design, and it will even impact the most essential part of the UE architecture. Thus, we don’t think the current spec implies certain UE capability beyond 2 searcher assumption, and 2 searcher assumption shall still be kept for this R19 WI.

To move forward, we also suggested that if companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers. But such searcher discussion can still be contribution driven as business as usual.

Proposal 4: RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF.

- If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers.

Issue 2-2-5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC

- Option 1 (Apple, CTC, Intel, Ericsson): UE only needs to measure one serving CC per band if multiple serving CCs are in the same band - Option 1a (Apple): details of option 1 is: - If PCC in the band, measure PCC - Otherwise if PSCC in the band, measure PSCC - Otherwise if SCC is in the band, measure the SCC with neighbor cell MO - Otherwise up to UE implementation - [Opponent view]Option 1b (HW, Ericsson): The candidate solution of optimizing CSSF (UE only needs to measure one serving CC per band if multiple serving CCs are in the same band) can be implemented by network configuration, e.g., Only one servingCellMO is configured on one SCC and no MOs are configured on the other SCCs, if multiple serving CCs are in the same band. - Option 1c(Ericsson): - If UE only measures one CC (e.g., determined by either UE or NW) out of multiple CCs, regarding measurement (including reporting) configurations to the CC and other CC(s), some promising approaches are listed as follows: - Option 1c-1: NW measurement configuration doesn’t cover the CC(s) not to be measured. - Option 1c-2: NW measurement configuration covers all CC, by further (e.g. dynamical) indication, - Option 1c-2.1. No measurement report even measurement configuration is configured for the CC(s) which not to be measured. - Option 1c-2.2: If measurement configuration is configured for the CC(s) which not to be measured, the report on the CC(s) reuses the measured result of the CC to be measured. - Option 2 (Apple, CTC, Ericsson): UE can reduce the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for some conditions - The conditions can be FFS. - [Opponent view] (HW): Dynamical CSSF would complicate UE implementation. - Option 3 (OPPO): - RAN4 to discuss the optimization of counting the number of configured SCell(s) or MOs without MG for CSSFoutside_gap in FR2 L3 measurements. - Option 4 (Ericsson): - RAN4 to study different SMTC configurations in different CCs to optimize CSSF. - RAN4 to study the minimal CC number to apply CSSF enhancement. - Option 5 (Nokia): - To identify the scenarios where SSB-based measurements can be relaxed so that CSSF can be optimized. - To consider the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements. - Option 6 (QC): - NW can group among intra-frequency layers from configured MO. UE can prioritize to measure one frequency layer in each group. - Option 7 (Huawei): - For UE supporting per FR gap, when all MOs are to be measured outside gap, the CSSFoutsidegap can be optimized, as these MOs can share the three searchers. - Three searchers capability is naturally supported by UE who supports per FR gap and it is not a new or enhanced capability. - Recommended WF - Moderator note: - the option 1 has most support, to check if option 1 can be the starting point. - If issue 2-2-4 concluded on 3 searchers, then 3 searchers based solution can also be discussed.

For the case that UE is not in multiple-Rx simultaneous reception mode but it’s in CA/DC mode, the CSSF enhancement can be considered if multiple serving carriers are in the same band. In the current spec, the UE measurement capability requirement is defined as following:

#### 9.2.3.2 Requirements for FR2 For one single intra-frequency layer in a band, during each layer 1 measurement period, the UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least: - 6 identified cells, and - 24 SSBs with different SSB index and/or PCI, where this single intra-frequency layer shall be: - PCC when UE is configured with SA NR operation mode with PCC in the band; or - PSCC when UE is configured with EN-DC with PSCC in the band; or - PSCC when UE is configured with NR-DC with PSCC in the band; or - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation. The UE shall also be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band.

For intra-frequency layer measurement, RAN4 assumed UE is allowed to only measure one CC in a same band, as highlighted in red above, but for serving CC measurement we require UE shall be capable of performing SS-RSRP, SS-RSRQ, and SS-SINR measurements for at least 2 SSBs on serving cell for each of the other intra-frequency layer(s) in the same band. In R15 discussion, the reason behind such requirement is because RSRQ might be different among different serving carriers (different interference level), however, FR2 is actually noise-limited system rather than interference-limited system, and therefore we don’t think RSRQ would be too much different among serving CCs in the same band. Thus, we can enhance to require UE only measure one serving CC in the same FR2 band like what RAN4 specified for intra-frequency neighbor cell measurement.

In last meeting, companies mentioned in option 1b that the candidate solution of optimizing CSSF (UE only needs to measure one serving CC per band if multiple serving CCs are in the same band) can be implemented by network configuration, e.g., Only one servingCellMO is configured on one SCC and no MOs are configured on the other SCCs, if multiple serving CCs are in the same band. We think it’s same situation as the discussion in R15 for neighbor cell measurements, i.e., we cannot limit the network implementation and configuration behavior, and therefore such enhancement to measure one serving cell in one band is based on the scenario when network configures multiple serving MOs in the same FR2 band to the UE.

Another option we are thinking for enhancement is to reduce the searcher occupancy ratio of PCC and PSCC for some special conditions, and then the SCC measurement can be speeded up by increasing the searcher occupancy ratio. For example, in current spec, for NR SA (e.g., FR2 only CA), the CSSF table is specified as following,

It means that: PCC measurement utilizes a dedicated searcher, and all the SCCs share the 2nd searcher. If SCC has FR2 neighbor cell configured CCs, the SCCs with FR2 neighbor cell utilize the 50% of the 2nd searcher. And all the other SCCs and inter-freq CCs equally share the other 50% of the 2nd searcher. If SCC has no FR2 neighbor cell configured CCs, all the SCCs and inter-freq CCs without MG equally share the 2nd searcher.

It's obvious that, if the PCC searcher occupancy ratio can be reduced, e.g., to 50% of the 1st searcher, the other 50% of the 1st searcher can be shared by SCCs to speed up the measurement. However, PCC measurement is most important thing to the RRM, and we can discuss the conditions in which PCC measurement can reduce the searcher occupancy ratio. Such enhancements and condition can also be applied to PSCC in DC case.

Regarding option 5, we also observed that current CSI-RS L3 measurement would occupy much measurement resource if CSI-RS L3 is configured to UE. The legacy CSI-RS measurement in CSSF design is specified in TS38.133 section 9.1.5.1, and an example of the table is as in the following table:

Take FR2 intra-band CA as an example:

NSCC_CSIRS=Number of configured SCell(s) with either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured. It means UE needs to use two measurement resources (SSB+CSI-RS) for one SCC measurement, if this SCC has either both SSB and CSI-RS based L3 measurement configured or only CSI-RS based L3 measurement configured. The reason to have such assumption is because UE needs to measurement associated SSB before CSI-RS measurement, so as long as CSI-RS measurement is configured on this SCC, UE has to use twice of measurement resources or time to measurement the associated SSB and CSI-RS. However, if associated SSB of the CSI-RS measurement of target serving CC is on another serving CC, the SSB measurement resource on the target serving CC can be released at all.

If “both SSB and CSI-RS based L3 measurement is configured on SCC1” or “only CSI-RS based L3 measurement configured on SCC1” and the associated SSB of this CSI-RS measurement is on another serving CC, e.g., PCC or PSCC, then UE can skip to counting the CSSF for SSB measurement on the SCC1, and the CSI-RS L3 measurement needs only to wait for the completion of associated SSB measurement on another serving CC.

Proposal 5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC:

- Option 1/1a: UE only needs to measure one serving carrier per band if multiple serving carriers are in the same band,

  - If PCC in the band, measure PCC

  - Otherwise if PSCC in the band, measure PSCC

  - Otherwise if SCC is in the band, measure the SCC with neighbor cell MO

  - Otherwise up to UE implementation

- Option 2: UE can reduce the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for certain conditions

  - The conditions can be FFS.

- Option 5: To consider the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements.

We also identified other possible enhancement for L3 measurement, e.g., in legacy requirement, the neighbor cell measurement is required on one of the CC within a FR2 band (TS38.133), the key justification to require one CC neighbor cell measurement is because of the co-located assumption for the CCs within one FR2 band. If the FR2 intra-band CA is non-contiguous CC combination, it’s possible in the future study that the non-collocated deployment is used. If it’s FR2 intra-band non-collocated CA in this RRM enhancement study, then one CC measurement in one band is not sufficient, as the CCs in the same band could be deployed at different geographic location which will have quite difference channel conditions. Thus, we may need to further clarify the neighbor cell measurement in a FR2 band to be: UE may measure neighbor cells on one CC (e.g., reference CC) among the contiguous CCs in a FR2 band, and this CC can be:

- PCC when UE is configured with SA NR operation mode with PCC among the contiguous CCs in the band or

- PSCC when UE is configured with EN-DC with PSCC among the contiguous CCs in the band; or

- PSCC when UE is configured with NR-DC with PSCC among the contiguous CCs in the band; or

- One of the SCCs among the contiguous CCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.

Issue 2-2-7: feature capability of L3 measurement delay reduction by optimizing CSSF

- Option 1 (Apple, Nokia): - RAN4 to introduce a new individual capability for CSSF reduction in R19. - Option 1a (Apple): fine to delay the capability discussion to the end of the core part. - Option 1b (Nokia) - The reduced CSSF shall be applied to the UE supporting the capability and starting from Release 19. - Recommended WF - Moderator note: - check if option 1/1b can be agreed.

Even though it’s too early to discuss the capability at this stage, we share the same view with companies that a new capability is needed for this CSSF optimization feature. We are also fine to delay this capability discussion to the end of the core part as legacy procedure.

Proposal 6: RAN4 to introduce a new individual capability for CSSF reduction in R19. But also fine to delay the capability discussion to the end of the core part.

Issue 2-3-1: whether and/or which previous release feature shall also be considered in “FR2-1 SSB based L3 measurement delay reduction for connected mode”

- Option 1 (Apple): - for “FR2-1 SSB based L3 measurement delay reduction for connected mode” by optimizing Rx beam sweeping factor, R18 feature of FR2 multi-Rx reception shall be considered - for “FR2-1 SSB based L3 measurement delay reduction for connected mode” by optimizing CSSF outside gap, both R16 inter-frequency measurement without MG and R18 inter-RAT measurement without MG shall be considered. - Option 2 (LGE, QC): - RAN4 not to consider SSB based L3 measurement delay enhancement with previous release features. - R19 L3 measurement enhancement for both fast beam sweeping and CSSF optimization is independent to R18 multi-Rx feature. (QC) - Option 3 (Ericsson): - Only Rel-18 multi-Rx scenario should be assumed, and we should not assume that the UE is also supporting other features.

For FR2-1 L3 measurement delay by optimizing CSSF outside gap in CA/DC, the CSSF in SA mode includes both R16 inter-frequency measurement without MG and R18 inter-RAT measurement without MG, like circled in the following:

Thus, for CSSF enhancement, we propose to consider both R16 inter-frequency measurement without MG and R18 inter-RAT measurement without MG. However, as commented by companies in last meeting, to avoid ambiguity of mixed features, we are also fine with option 2.

Proposal 7: RAN4 not to consider SSB based L3 measurement delay enhancement with previous release features, only except R16 inter-frequency measurement without MG and R18 inter-RAT measurement without MG for CSSF enhancement.

# Conclusion

In this contribution, we continue the discussion for FR2-1 SSB based L3 measurement delay reduction for connected mode.

Proposal 1: agree on either one of following options:

- Option 1: Rel-19 CSSF optimization applies for CA/DC scenario when (1)UE is not capable of Rel-18 multi-Rx simultaneous reception or (2)UE is capable of Rel-18 multi-Rx simultaneous reception but work in single-Rx currently.

- Option 4a: Rel-19 discussion on the scenarios for CSSF optimization will be considered in CA/DC scenarios with at least two FR2 serving cells, independently of the UE support of multi-Rx capabilities.

Proposal 2: the following scenarios to use L3 measurement delay reduction by optimizing CSSF shall be prioritized:

- SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra

- SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_measurement_period_inter and TSSB_time_index_inter

Proposal 3: RAN4 to consider EN-DC, NE-DC, SA and NR-DC for L3 measurement delay reduction by optimizing CSSFoutside_gap,i, and the specific scope can be as following:

- EN-DC

  - EN-DC with FR2 only intra band CA

  - EN-DC with FR2 only inter band CA

  - EN-DC with FR1 +FR2 CA (FR1 PSCell)

  - EN-DC with FR1 +FR2 CA (FR2 PSCell)

- NE-DC

  - NE-DC with FR2 only intra band CA

  - NE-DC with FR2 only inter band CA

  - NE-DC with FR1 +FR2 CA (FR1 PCell)

- SA

  - FR1+FR2 CA (FR1 PCell)

  - FR1+FR2 CA (FR2 PCell)

  - FR2 only intra-band CA

  - FR2 only inter-band CA

- NR-DC

  - FR1 + FR2 NR-DC (FR1 PCell and FR2 PScell)

Proposal 4: RAN4 only consider the enhancement based on 2 searchers, i.e., same as previous release, for L3 measurement delay reduction by optimizing CSSF.

- If companies cannot achieve consensus, RAN4 work can start with the baseline assumption in WID, i.e., 2 searchers.

Proposal 5: Solutions to apply/specify L3 measurement delay reduction by optimizing CSSF outside gap in CA/DC:

- Option 1/1a: UE only needs to measure one serving carrier per band if multiple serving carriers are in the same band,

  - If PCC in the band, measure PCC

  - Otherwise if PSCC in the band, measure PSCC

  - Otherwise if SCC is in the band, measure the SCC with neighbor cell MO

  - Otherwise up to UE implementation

- Option 2: UE can reduce the searcher occupancy ratio of PCC or PSCC measurement to speed up SCC measurement for certain conditions

  - The conditions can be FFS.

- Option 5: To consider the CSSF optimization by minimizing the impact from CSI-RS based measurements on SSB-based measurements.

Proposal 6: RAN4 to introduce a new individual capability for CSSF reduction in R19. But also fine to delay the capability discussion to the end of the core part.

Proposal 7: RAN4 not to consider SSB based L3 measurement delay enhancement with previous release features, only except R16 inter-frequency measurement without MG and R18 inter-RAT measurement without MG for CSSF enhancement.

# References

[1] R4-2410260, Way Forward for [111][228] NR_RRM_Ph5, Moderator (Apple), RAN4#111

[2] R4-2408025, Topic summary for [111][228], Moderator (Apple), RAN4#111